16S rRNA Library Preparation

- Dilute sample DNA to ~2-5ng/l. If the sample is  <2ng/l, use sample as is.

- Sample PCR should be done in duplicate, one serving as the base sample and another as a technical replicate. Prepare reaction mix for #of reactions +1 to prevent shortage of mix. Reagents for one 10l reaction are as follows:

H2O – 					4.7 μl

5X Q5 Hot Start Buffer -		2.0 μl

10μM Fprimer (515F)-		0.5 μl

10μM Rprimer (806R)-		0.5 μl

10mM dNTPs-			0.2 μl

Q5 Hot Start DNA Polymerase-	0.1 μl

Distribute 8 l of reaction mix to reaction tubes and then add 2 l of diluted DNA (total 10l).

(Note: Add reagents as listed. 5x buffer and primers may be thawed at room temp but dNTPs should be thawed on top of ice and polymerase should be kept on ice at all times.)

- Run on thermocycler as follows:

98°C 	30sec

98°C 	10sec

52°C 	30sec	        x15

72°C 	30sec

72°C 	2min

10°C	hold

- Prepare 2nd reaction mix while the first reaction is running. Prepare reaction mix for # of reactions +1. Reagents for one 30l reaction are as follows:

H2O-					11.1 μl

5X Q5 Hot Start Buffer-	 	6.0 μl

10μM 515_BCF primer- 	  	1.5 μl

10μM 806_BCR primer- 	  	1.5 μl

10mM dNTPs-		  	0.6 μl

Q5 Hot Start DNA Polymerase – 	0.3 μl

Remove reactions from thermocycler once the first reaction is completed. Keep reactions on cold block. Distribute 21μl of 2nd reaction mix to each new reaction tube. Add 9μl of reaction 1 product to the appropriate 2nd reaction tube. Pipette up and down to mix.

- Run on thermocycler as follows:

98°C 	30sec

98°C 	10sec

52°C 	30sec	       x4

72°C 	30sec

98°C 	10sec          x6

72°C 	1min

72°C 	2min

10°C	hold

- Verify PCR product by gel electrophoresis.

- Purify reactions using Omega Bio-Tek’s E.Z.N.A. Cycle-Pure Kit according to manufacturer’s protocol.

- Measure sample concentration and A260/A280 via nanodrop, record values. Store at -20°C.

- Pool together an equal amount of amplicons (generally 50ng, but adjust if necessary) from each sample into subpools composed of 15-30 unique samples.

- Verify subpool products by gel electrophoresis.

- Combine an equal amount of amplicons from each subpool and submit the pooled library for sequencing.