Agarose Gel Separation of Reovirus Particles by sigma1 Content

- Combine 0.4 g of agarose with 40 ml of TAE

- Microwave until agarose is thoroughly melted

- Add water to replace water lost during boiling up to 40 ml and mix well

- Pour 35 ml agarose to clean casting rig (7x10 cm)

- Mix purified reovirus and loading dye 1:1 at 1x1011 particles per well (25 l max)

- Run at constant 25V for 18 h

- Stain gel with colloidal blue as directed

- Destain gel in water overnight

- Image gel by on Licor Odyssey Imaging System

Materials and Reagents

-1M Tris Acetate pH 7.2

- MW=121.1g/mol

- Add 60.5 g Tris base to 300 mL ddH2O

- pH to 7.2 with glacial acetic acid

- Fill to 500 mL Tv

-10X TAE pH 7.2

- 125 mL 1M Tris acetate pH7.2

- 50 mL EDTA pH 8.0

- 75 mL ddH2O

- Dilute to 1X as needed and filter 0.45 uM to remove any potential debris

-UltraPureTM Agarose, Thermo Scientific

-NOVEX® Colloidal Blue Staining Kit (follow directions for [high] Tris Glycine)

-Reovirus 2X Loading Dye

- 5% Ficoll 400 (0.5g)

- 0.05% Bromophenol blue (50 mg)

- 2 mL 10X TAE pH 7.2

- Fill to 10 mL with ddH2O