GC/MS Settings:

| Column used | DB-5 30m long, 0.25 mm ID and 0.25 mm film thickness |
| --- | --- |
| ------- ----------- |  |
| Sample Inlet : GC |  |
| Injection Source : GC ALS |  |
| Mass Spectrometer : Enabled |  |
| No Sample Prep method has been assigned to this method. |  |
| Oven |  |
| Equilibration Time 0.2 min |  |
| Max Temperature 325 degrees C |  |
| Oven Program |  |
| 70 °C for 4 min |  |
| then 6 °C/min to 230 °C for 0 min |  |
| then 60 °C/min to 280 °C for 5 min |  |
| Run Time 36.5 min |  |
| 2 min (Post Run) 300 °C |  |
| Front Injector |  |
| Syringe Size 5 µL |  |
| Injection Volume 1 µL |  |
| Solvent A Washes (PreInj) 3 |  |
| Solvent A Washes (PostInj) 3 |  |
| Solvent A Volume 1.5 µL |  |
| Solvent B Washes (PreInj) 0 |  |
| Solvent B Washes (PostInj) 0 |  |
| Solvent B Volume 4 µL |  |
| Sample Washes 3 |  |
| Sample Wash Volume 2 µL |  |
| Sample Pumps 4 |  |
| Dwell Time (PreInj) 0 min |  |
| Dwell Time (PostInj) 0 min |  |
| Solvent Wash Draw Speed 150 µL /min |  |
| Solvent Wash Dispense Speed 3000 µL /min |  |
| Sample Wash Draw Speed 150 µL /min |  |
| Sample Wash Dispense Speed 3000 µL /min |  |
| Injection Dispense Speed 3000 µL /min |  |
| Viscosity Delay 0 sec |  |
| Sample Depth Disabled |  |
| Injection Type Standard |  |
| L1 Airgap 0.1 µL |  |
| Front SS Inlet He |  |
| Mode Split |  |
| Heater On 280 °C |  |
| Pressure On 15.213 psi |  |
| Total Flow On 9 mL/min |  |
| Septum Purge Flow On 3 mL/min |  |
| Gas Saver Off |  |
| Split Ratio 5 :1 |  |
| Split Flow 5 mL/min |  |
| Back Volatiles Interface He |  |
| Mode Split |  |
| Heater Off |  |
| Pressure Off |  |
| Total Flow Off |  |
| Septum Purge Flow Off |  |
| Gas Saver Off |  |
| Split Ratio 100 :1 |  |
| Split Flow 0 mL/min |  |
| Thermal Aux 1 {MSD Transfer Line} |  |
| Heater On |  |
| Temperature Program On |  |
| 300 °C for 0 min |  |
| Run Time 36.5 min |  |
| Column #1 |  |
| J&W 122-5532: 2602.40205 |  |
| DB-5ms |  |
| 325 °C: 30 m x 250 µLm x 0.25 µLm |  |
| In: Front SS Inlet He |  |
| Out: PCM C-2 |  |
| (Initial) 70 °C |  |
| Pressure 15.213 psi |  |
| Flow 1 mL/min |  |
| Average Velocity 23.39 cm/sec |  |
| Holdup Time 2.1377 min |  |
| Flow Program Off |  |
| 1 mL/min for 0 min |  |
| Run Time 36.5 min |  |
| 2 min (Post Run) 1.5503 mL/min |  |
| Column #2 |  |
| Agilent 160-2615-10FS-DEACTIVATED |  |
| 350 °C: 1.06 m x 180 µLm x 0 µLm |  |
| In: PCM C-2 He |  |
| Out: Front Detector FID |  |
| (Initial) 70 °C |  |
| Pressure 3.8 psi |  |
| Flow 1.7504 mL/min |  |
| Average Velocity 116.33 cm/sec |  |
| Holdup Time 0.015186 min |  |
| Pressure Program On |  |
| 3.8 psi for 0 min |  |
| Run Time 36.5 min |  |
| 2 min (Post Run) 20 psi |  |
| Column #3 |  |
| Agilent 160-2615-11FS-DEACTIVATED |  |
| 350 °C: 1.44 m x 180 µLm x 0 µLm |  |
| In: PCM C-2 He |  |
| Out: Vacuum |  |
| (Initial) 70 °C |  |
| Pressure 3.8 psi |  |
| Flow 3.4948 mL/min |  |
| Average Velocity 313.98 cm/sec |  |
| Holdup Time 0.0076439 min |  |
| Pressure Program On |  |
| 3.8 psi for 0 min |  |
| Run Time 36.5 min |  |
| 2 min (Post Run) 20 psi |  |
| Front Detector FID |  |
| Heater On 300 °C |  |
| H2 Flow On 35 mL/min |  |
| Air Flow On 400 mL/min |  |
| Makeup Flow On 15 mL/min |  |
| Const Col + Makeup Off |  |
| Flame On |  |
| Electrometer On |  |
| Back Detector NPD |  |
| Heater Off |  |
| H2 Flow Off |  |
| Air Flow Off |  |
| Makeup Flow Off |  |
| Const Col + Makeup Off |  |
| Bead Off |  |
| Back Detector |  |
| Signal 1 Type NPD |  |
| Maximum Bead Voltage 4.095 V |  |
| Blos Bead No |  |
| Dry Bead Yes |  |
| Auto Adjust Bead Yes |  |
| PCM C-1 He |  |
| ***Excluded from Affecting GC's Readiness State*** |  |
| Flow Program Off |  |
| 0 mL/min for 0 min |  |
| Run Time 36.5 min |  |
| PCM C-2 He: Supplies Column 2 |  |
| ***Excluded from Affecting GC's Readiness State*** |  |
| Signals |  |
| Front Signal Save On |  |
| 5 Hz |  |
| Back Signal Save Off |  |
| 5 Hz |  |
| Test Plot Save Off |  |
| 50 Hz |  |
| Test Plot Save Off |  |
| 50 Hz |  |
| MS ACQUISITION PARAMETERS |  |
| General Information |  |
| ------- ----------- |  |
| Tune File : atune.u |  |
| Acquistion Mode : Scan |  |
| MS Information |  |
| -- ----------- |  |
| Solvent Delay : 9.00 min |  |
| EMV Mode : Relative |  |
| Relative Voltage : 0 |  |
| Resulting EM Voltage : 1953 |  |
| [Scan Parameters] |  |
| Low Mass : 35.0 |  |
| High Mass : 600.0 |  |
| Threshold : 150 |  |
| Sample # : 3 A/D Samples 8 |  |
| Plot 2 low mass : 35.0 |  |
| Plot 2 high mass : 600.0 |  |
| [MSZones] |  |
| MS Source : 230 C maximum 250 C |  |
| MS Quad : 150 C maximum 200 C |  |
| END OF MS ACQUISITION PARAMETERS |  |
| TUNE PARAMETERS for SN: US93443443 |  |
| ----------------------------- |  |
| Trace Ion Detection is OFF. |  |
| EMISSION : 34.610 |  |
| ENERGY : 69.922 |  |
| REPELLER : 20.071 |  |
| IONFOCUS : 90.157 |  |
| ENTRANCE_LE : 32.000 |  |
| EMVOLTS : 1952.941 |  |
| Actual EMV : 1952.94 |  |
| GAIN FACTOR : 1.45 |  |
| AMUGAIN : 2248.000 |  |
| AMUOFFSET : 121.625 |  |
| FILAMENT : 1.000 |  |
| DCPOLARITY : 1.000 |  |
| ENTLENSOFFS : 18.573 |  |
| MASSGAIN : -511.000 |  |
| MASSOFFSET : -37.000 |  |