# Collecting eDNA from marine water samples in the field

Reindert Nijland, Marine Animal Ecology, Wageningen University

Version 08-07-2019

Starting remarks:

All non-DNA free/not-gamma sterilized sampling devices are cleaned with a bleach solution or UV exposure before use to break down contamination DNA. Always wear gloves and minimize potential contamination with DNA from human handling and/or nearby food sources or animals.

Collect at least one control sample on site of sampling collection. Filter 500 ml of DNA free Milli-Q (if not available, use tap water) at each location, to monitor contamination during the filtering and subsequent DNA extraction.

Filter units used:

For eDNA isolation I use disposable filter units from Nalgene; 50mm diameter with 0.8uM cellulose nitrate filter.  Ordernumbers: 10792351 ( 250ML 0,8µm) and 10720622 (500ML, 0,8µm) see: https://www.fishersci.co.uk/shop/products/nalgene-rapid-flow-sterile-disposable-filter-units-cn-membrane/10792351

Collection of eDNA from the water on a filter membrane:

- Collect water in an appropriate sterile, DNA free container

  - For marine samples collected during scuba diving, we use a small hand-pump and a punch-balloon

- Filter 1-2 l. with the 250ml filters (0,8micron, cellulose nitrate) from Nalgene.

- Connect filter unit to vacuum pump, filter by vacuum. I use a small vacuum-pump from ebay, powered with a 12V power pack. (DC12V Mini Vacuum Pump Negative Pressure Suction Pump)

- Using the vacuum, make sure the filter is dry, as to much transfer of liquid influences preservation and downstream DNA isolation

- Cut filter from filter unit, it is glued in place. No problem to cut off outer 0,5cm of filter. Use sterile scalpel + scalpel holder. Fold filter twice using scalpel+tweezers.

- Insert filter in 2ml screwcap tube prefilled with 400ul preservation solution.  It is possible to use other tubes, but not when using 3d-fuge, as these do not fit and will snap open during centrifugation.

- Shake tube with filter, to fully soak in the preservation solution. Repeat after 2-3 minutes to re-soak. Filter ready for storage.

Preservation of filters with eDNA:

The DNA is now collected on the filter. Either directly continue with the DNA isolation protocol of choice, or store the preserved the DNA on the filter, depending on preservation method used.

Qiagen DNeasy kit, short term  (<48h) preservation:

- Store filter in 400ul ATL buffer from the DNeasy kit, shake well, store at RT or below.

Zymo DNA/RNA shield, short+longterm preservation

- Store filter in 400ul DNA/RNA Shield, shake well, store at RT or below

Longmire’s solution: longterm preservation.

- Store filter in 500ul-1ml longmire’s solution, shake well, store at RT or below

- cheap, works ok, but only compatible with Phenol/Chlorophorm extraction, which is laborious and also does not yield the best and purest DNA.

Other alternative preservation methods: ethanol, freezing, DMSO, DESS, dehydration, combinations, etc.

eDNA extraction methods:

Zymo Quick DNA Miniprep Plus Kit:

- In the lab, add 20ul ProtK, incubate 10-30 minutes at 55°C

- Add 1 volume of genomic extraction buffer, follow protocol (mix by vortexing, load on spin column, etc.).

- Pipetting is a bit of hassle with the filter in the tube, best to remove most buffer, spin shortly, remove rest.

- Elute in either recommended volume (50ul) or use higher elution volume if more dilute DNA is required.

Qiagen DNeasy kit:

- When in the lab, add 20ul protK, inucbate at 56°C for 10-30 minutes.

- Follow standard protocol for blood samples: Since using 400ul volume, double amount of AL and Ethanol: add 400 μl Buffer AL. Mix thoroughly by vortexing, add 400ul Ethanol. mix by vortexing, load on spin column, etc..

- Pipetting is a bit of hassle with the filter in the tube, best to remove most buffer, spin shortly, remove rest. Also you will need to load the DNA filter tube twice to process all liquid.

- Elute in either recommended 200ul AE or use lower elution volume (down to 50ul)  if more concentrated DNA is required.

Alternative: use classic phenol/chlorophorm extraction, that works also very well. I did this with the samples I preserved in Longmire solution, can also be done with samples in DNA/RNA shield. Before doing DNA extraction I first incubated 30 minutes with protK at 56°C. With the larger filter units (500ml) you also get larger filters, which do not fit in a 2ml tube. Either use a 15ml tube, or the following idea will also work: cut the filter from the 500 ml units in 4 equal pieces and process these separately. Isolate DNA from 2 quarters, store the other 2 in Longmire for later if something goes wrong with the isolation..

Remarks/Tips:

Water filtering:

The 250ml Nalgene unit is a nice size, but will clog quite rapidly if filtering lots of dirty water. You also need to empty and replace the bottom container more often, of just use a 1l (or 2l) scott/duran bottle as a replacement for the supplied 250ml bottle.

DNA Preservation method used:

For short preservation times, filters can be directly placed in ATL buffer (Qiagen DNeasy kit). This works well if you are able to process the samples within 48h or you have a freezer available within that time.

For preservation longer than a few days, I tested Longmire’s solution. Works well, but only compatible with Phenol/Chlorophorm extraction, which is laborious and also does not yield the best and purest DNA.

Now I switched to using Zymo DNA/RNA shield. Have no test results back, but expect good results, and major advantage is that it directly fits into both the Zymo Quick-DNA  miniprep Plus kits as well as the Qiagen DNeasy kit. And DNA will be stable at RT for 1 month.

Isolation of eDNA from longmire preserved filter-membranes

- If filters in 2ml tubes and volume <1ml, extraction can be done in the 2ml tubes, much easier. If filter preserved in more than 1 ml liquid, transfer to 15ml falcon tubes, or use the 15ml tubes with the filters.

  - Optional when using 15ml tubes: Add several sterile, DNA free, 4mm glass-beads

- Add 30ul proteaseK

- Vigorously vortex the tube with filter+longmire solution

- Incubate 30minutes – 2h at 56-60C, with regular vortexing.

- Add 1 volume of Phenol/Chlorphorm/Isoamylalcohol (PCI) (2-3ml usually)

- homogenize on vortex mixer or vortex for ~2 minutes, invert some more for about 5minutes. Often a red-pinkish colour develops (probably some reaction between the algal chlorophyll and the PCI?)

- Spin down 10 minutes at 5000rpm

- Waterphase now pink or more dark red (some samples). PCI yellowish, but not pink

- Collect waterphase.

- Second extraction with 2-3ml Chlorophorm/IAA. Spin 10min @ 5000RPM

- Collect ~2-3ml of the waterphase.

- Add NaAcetate 3M, 500ul, and 7ml cold EtOH.

- White precipitate forms, this is normal.

- Precipitate 1h at -20??

- Spin down, 15’ @5000RPM

- Sometimes yellowish sticky/slimy pellet visible(polysaccharides?), can be removed by washing with 70% EtOH. Necessary to resuspend the gooey pellet by shaking/vortexing

- Wash pellet in 3ml 70% EtOH,.

- Repeat wash

- Dry pellet to air

- Resuspend in 100-200ul NFW/T10E1/EB