DNA Extraction Protocol for Cockroach Gut Bacteria

This protocol uses an adapted version of Omega Bio-Tek’s E.Z.N.A. Bacterial DNA Kit.

- Remove samples, usually P. americana gut lumen stored in 100μL RNALater, from -80°C freezer and set them on ice to thaw. Remove lysozyme solution from the -20°C freezer and set it on ice to thaw. Preheat Thermomixer to 37°C.

- Remove ~50μL sample preserved in RNAlater. Place aliquot in a new 1.5mL microcentrifuge tube.

- Add 100μL Balanced Salt Solution  (2.5g K2HPO4, 1 g KH2PO4, 1.6 g KCl, 1.4 g NaCl, and 10 ml of 1 M NaHCO3 per liter, pH 7.2) to the new microcentrifuge tube. Spin microcentrifuge tube 10 minutes at 5,000xg. Discard supernatant.

- Add 100μL 1xTE (10 nM Tris, 1mMEDTA [pH 8]) and vortex to completely resuspend the pellet. Add 10μL of lysozyme solution. Incubate 30 minutes at 37°C. Add 25mg glass beads to new 1.5mL microcentrifuge tube.

- Preheat Thermomixer to 55°C. Add sample to the glass beads. Vortex at maximum speed for 5 minutes.

- Let sample stand to allow the beads to settle. Transfer supernatant to a new 1.5mL microcentrifuge tube.

- Add 100μL BTL Buffer and 20μL Proteinase K Solution. Vortex to mix thoroughly.

- Incubate at 55°C, shaking at 600rpm for 60 minutes.

- Centrifuge at 10,000xg for 2 minutes to pellet any undigested material. Preheat Thermomixer to 65°C. Place Elution Buffer in the hybridization oven and preheat to 65°C.

- Transfer the supernatant to a new 1.5mL microcentrifuge tube. Do not disturb the pellet.

- Add 220μL BDL Buffer to the new microcentrifuge tube with the supernatant. Vortex to mix.

- Incubate at 65°C for 10 minutes.

- Add 220μL ethanol. Vortex for 20 seconds at maximum speed to mix thoroughly. Note: If any precipitate can be seen at this point, break the precipitate by pipetting up and down 10 times.

- Insert a HiBind DNA Mini Column into a 2mL Collection Tube.

- Transfer the entire sample to the HiBind DNA Mini Column, including any precipitate that may have formed.

- Centrifuge at 10,000xg for 1 minute.

- Discard the filtrate and the collection tube.

- Insert the HiBind DNA Mini Column into a new 2mL Collection Tube

- Add 500μL HB Buffer.

- Centrifuge at 10,000xg for 1 minute.

- Discard filtrate and reuse the collection tube.

- Add 700μL DNA Wash Buffer. Centrifuge at 10,000xg for 1 minute.

- Discard the filtrate and reuse the collection tube.

- Add 700μL DNA Wash Buffer. Centrifuge at 10,000xg for 1 minute.

- Discard the filtrate.

- Centrifuge the empty HiBind DNA Mini Column at maximum speed for 2 minutes to dry the column.

- Insert HiBind DNA Mini Column into a new nuclease-free 1.5mL microcentrifuge tube.

- Add 50μL preheated Elution Buffer to the HiBind DNA Mini Column. Make sure to add the elution buffer to the center of the HiBind matrix.

- Incubate the HiBind DNA Column for 5 minutes at 65°C.

- Centrifuge at 10,000xg for 1 minute to elute the DNA.

- Measure DNA concentration on the NanoDrop and store eluted DNA at -20°C.