Allison Lab Protocol: Fluorimetric Enzyme Assays, 9/2009, Donovan P. German

Fluorimetric Enzyme Assay Protocol: BX

(Modified from Saiya-Cork, K. R., R. L. Sinsabaugh, and D. R. Zak. 2002. The effects of long term nitrogen deposition on extracellular enzyme activity in an Acer saccharum forest soil. Soil Biology & Biochemistry 34:1309-1315.)

Reagents:

- M NaOH

4 g NaOH pellets; 100 mL DI water

50 mM sodium acetate buffer, pH 5.0 (can make a 10X stock solution; pH can vary with soil)

4.374 g sodium acetate trihydrate

1.1 ml glacial acetic acid (add more to make pH = 5)

1 L sterile DI water (miliQ water)

Substrate solutions (1000 μM unless otherwise noted):

| Enzyme | Substrate |
| --- | --- |
| Cellobiohydrolase (CBH) | 4-MUB-β-D-cellobioside (in refrigerator) 12.5 mg/50 mL 500 μM |
| β-Glucosidase (BG) | 4-Methylumbelliferyl β-D-glucopyranoside 16.9 mg/50 mL 1000 μM |
| α-Glucosidase (AG) | 4-Methylumbelliferyl α-D-glucopyranoside 16.9 mg/50 mL 1000 μM |
| β-xylosidase (BX) | 4-Methylumbelliferyl β-D-xylopyranoside 15.4 mg/50 mL 1000 μM |
| MUB standard | 1000 μM 4-Methylumbelliferone 0.89 mg/50 ml 100 μM |

Make substrate and fluorescent standard solutions in 125 mL red glass bottles using milliQ water. Store solutions in the 4°C refrigerator. Substrates are in freezer unless noted. Remake solutions every week, and make them one day BEFORE you want to start working, as this affects the measured activity levels. MUB standard should be diluted to 25 μM before use, depending on expected sample fluorescence. MUB is made by heating the solution to 90°C for a few minutes, with constant stirring (don’t let it boil).

Sample Preparation: Soils

Weigh other part (1-2 g wet weight), record mass, and place in labeled 500 ml container. Add 125 ml acetate buffer and blend on highest speed for 1 minute to make a homogenate slurry. Rinse blender with DI water between samples.

Assay Set-up

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |
| CBHbl | BGbl | AGbl | BXbl | CBH | BG | AG | BX |  |  | Hombl | Buf |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NaOAc 200 uL | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |
| 200 | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |
| 200 | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |
| 200 | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |
| 200 | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |
| 200 | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |
| 200 | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |
| 200 | 200 | 200 | 200 |  |  |  |  |  |  | 50 | 250 |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| milliQ |  |  |  |  |  |  |  | 0 | 0 |  |  |
|  |  |  |  |  |  |  |  | 25 | 25 |  |  |
|  |  |  |  |  |  |  |  | 37.5 | 37.5 |  |  |
|  |  |  |  |  |  |  |  | 43.8 | 43.8 |  |  |
|  |  |  |  |  |  |  |  | 46.9 | 46.9 |  |  |
|  |  |  |  |  |  |  |  | 48.4 | 48.4 |  |  |
|  |  |  |  |  |  |  |  | 49.2 | 49.2 |  |  |
|  |  |  |  |  |  |  |  | 49.5 | 49.5 |  |  |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25mM MUB |  |  |  |  |  |  |  | 50 | 50 |  |  |
|  |  |  |  |  |  |  |  | 25 | 25 |  |  |
|  |  |  |  |  |  |  |  | 12.5 | 12.5 |  |  |
|  |  |  |  |  |  |  |  | 6.2 | 6.2 |  |  |
|  |  |  |  |  |  |  |  | 3.1 | 3.1 |  |  |
|  |  |  |  |  |  |  |  | 1.6 | 1.6 |  |  |
|  |  |  |  |  |  |  |  | 0.8 | 0.8 |  |  |
|  |  |  |  |  |  |  |  | 0.5 | 0.5 |  |  |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subs 50 | BG 50 | AG 50 | BX 50 | CBH 50 | BG 50 | AG 50 | BX 50 |  |  |  |  |
| CBH 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |
| 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |
| 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |
| 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |
| 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |
| 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |
| 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |

Using the multi-channel pipette with wide-mouth tips (i.e., trimmed with scissors), add the homogenate to all wells receiving homogenate

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |
|  |  |  |  | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |

All wells should now contain 250 μl. Place plates in incubator at proper temperature for soils (e.g., 10°C for Alaskan soils). Incubate for one hour.

Assay Termination: After one hour of incubation, add 10 μL of 1 M NaOH to each well, noting time. Note: this is not done in advance. Add the NaOH to the plates and wait 10 min, then read the plates. This is important. Give the NaOH time to interact with the MUB. MUB will only fluoresce in alkaline conditions.

Reading Plates: Read fluorescence plates at 365 nm excitation and 450 nm emission.

Standard Curve Construction

It is important to calculate the standard curve for the MUB based on the MUB standard wells (wells 9-10). First, obtain the “homogenate blank” value by averaging the values in column 11. Then, take the MUB standard values, average them for each MUB concentration, and subtract the homogenate blank from the fluorescence values for each concentration. Construct a standard curve using linear regression (MUB concentration on the x-axis, and Fluorescence on the y-axis) to obtain the slope. Make sure this is calculated with the MUB concentrations in μmols/L

(which is synonymous with nmols/mL). The slope is your Extinction Coefficient (ε). To calculate the “quench”, simply construct a second standard curve (in a separate plate) that receives buffer and no homogenate. This “unquenched” curve gives the true fluorescence of MUB. The ratio of the slope of the standard curve containing soils to the standard curve containing buffer tells you how much the soil quenches the fluorescence of the MUB.

Fluorescence Activity

Activity (nmol h-1 g-1) = {NFU x Buffer vol(mL)}/{ε x Homogenate vol(mL) x Time(h) x Soil mass(g)} NFU = net fluorescence units = {(Assay – Homogenate Control)/Quench Coeff} – Substrate control Quench coefficient = (Quench control – Homogenate control)/Standard Fluorescence

ε = extinction coefficient Assay volume = 0.250 mL Homogenate volume = 0.200 mL Buffer vol = volume of buffer used to dilute the soil when making homogenate (likely 125 mL) Soil mass = mass of soil included when making homogenate (g) For soils, the mass of the soil can be wet weight, dry weight, organic matter, or Cmic.