Extraction and detection of free-gossypol in cottonseed samples

Extraction

1 – Dry the sample at 40 °C for 24 hours.

2 – Grind the dry sample to form a fine powder with an ultra centrifugal mill

3 – Weight 1 gram of the powdered sample into a 15mL plastic centrifuge tube with screw.

4 - Add 3 mL of ultrapure water into the tube.

5 – Agitate vigorously for about 10 seconds.

6 - Ultra-sonicate for 5 minutes using ice in the sonicator.

7 -  Add 7 mL of acetone onto the mixture.

8 - Ultra-sonicate for 5 minutes using ice in the sonicator.

9 - Vortex vigorously for about 10 seconds.

10 – Centrifuge the tubes at 9000 rpm for 5 minutes at 8 °C.

11 – Transfer 2 mL of the supernatant to 15 mL centrifuge tube.

12 – Evaporate the solvent in a speed vacuum at room temperature.

13 – Resuspend the residue with 200 µL of 70% acetone.

14 – Transfer the solution to a 1.5 mL Eppendorf tube.

15 – Centrifuge at 14000 rpm for 10 minutes at 8 °C.

16 – Transfer the supernatant to vial.

Detection

1 – Set a reversed phase C18 column (100 x 2.1 mm, 2.6 µm) in a UHPLC equipped with a photodiode array (PDA).

2 – Set column temperature to 35 °C.

3 – Use methanol as organic solvent (A) and 0.1% TFA in water as aqueous solvent (B) for mobile phase constituents.

4 – Set elution as gradient. The elution starts with 60% of A, changing linearly to 100% of A , it is reached at 8 minutes and maintained until 10 minutes. After this, the mobile phase suddenly changes to 60% of A, maintaining this isocratic flow till the end of run at 14 minutes.

Table 1. Gradient elution applied for free gossypol (FG) detection by UHPLC.

| Time (min) | Aqueous (%) | Organic (%) |
| --- | --- | --- |
| Initial | 40 | 60 |
| 8 | 0 | 100 |
| 10 | 0 | 100 |
| 10.01 | 40 | 60 |
| 14 | 40 | 60 |

5 – Set the flow rate to 0.4 mL/min.

6 – Set the sample manage to 8 °C.

7 – Set the PDA monitoring to 254 nm.

8 – Inject 2 µl of the sample.

Observation:

Tested equipment -  KINETEX® column and a Waters ACQUITY UPLC H-Class system.