# Blood collection protocol

Adapted from SOP Virginia Tech: Mouse Blood Collection from Submandibullar Vein (Cheek Punch)

Materials List

- Appropriate collection tubes (e.g., EDTA coated tubes, low-protein binding tubes)

- Ice container

- Lancet

  - 4mm for a small/young mouse

  - 5mm to 5.5mm for an average adult mouse

- Gauze pads

- Micropipette and filtered tips (100ul)

- Scale - All mice should be weighted prior to blood collection

- centrifuge

- Facial Vein Collection

Notes

- Frequency: The submandibular vein can be used for serial blood collection of small samples by alternating cheek sides, and/or by gently removing the clot/scab.

- Anesthesia: This method can be performed without anesthesia, but does require training to perfect technique.

Procedure

- Properly restrain the animal with one hand, which will apply adequate pressure to the maxillary vein.

- Visualize landmarks

  - Albino mice have a convenient landmark in the area of the maxillary vein. Often viewed as a small bald spot or hair whorl, you can find this along the curvature of the mandible; the vein is just below this mark in the groove past the jawbone. Darker mice have the same spot but it may be more difficult to see.

  - You can also align a lancet caudal to the angle of the mandible, extending to the ear. Location of the puncture site is at the midway point of the lancet.

- Using the point of the lancet, apply firm pressure at the point, caudal to the eye and ventral to the ear, where the submandibular vein is located, then release until blood flows.

  - CAUTION: hold needle or lancet perpendicular to the bleed site to avoid puncturing the ear canal.

- Quickly place blood collection tube below the puncture site until desired volume of blood is collected.

- Apply gentle pressure with gauze until bleeding has stopped.

- Dispose of the needle/lancet into the approved sharps container

- Plasma separation

- Blood is collected into commercially available anticoagulant-treated tubes e.g., EDTA-treated (lavender tops US, red-tops EUROPE).

- Cells are removed from plasma by centrifugation for 15 minutes at 2,000 x g using a refrigerated centrifuge (4°C). The resulting supernatant is designated plasma.

- Immediately transfer the liquid component (plasma) into a clean polypropylene low-protein binding tube using a micropipette.

- The samples should be maintained at 2–8°C while handling.

- If the plasma is not analysed immediately, the plasma should be apportioned into 0.5 ml aliquots, stored, and transported at –80°C or lower.

  - It is important to avoid freeze-thaw cycles.