96-well Plate LICOR Fluorescent Focus Unit Assay

Cell Fixing

- Remove media from wells.

- Wash 1x with Phosphate Buffered Saline (PBS).

- Add 100 uL ice-cold methanol per well.

- Store plate at -20°C for at least 30 min (can remain at -20°C for several weeks).

Cell Staining

- Remove methanol, allow plate to come to room temperature, and excess ethanol to evaporate.

- Wash wells with 150 ul PBS

- Add 150 uL Dulbecco’s PBS (DPBS) with 0.5% Tween-20 (DPBS-T) to each well.

- Remove immediately.

- Add 50 uL rabbit anti-reovirus polyclonal antiserum (1:1000 dilution) in DPBS with 1% BSA (DPBS-BSA) to experimental wells.

Add 50 uL DPBS-BSA with no antibody to background control.

- Incubate for 1 h at 37°C.

- Remove DPBS-BSA.

- Wash 3x with DPBS-T for 5 min each wash while shaking.

- Add 50 uL of DPBS-BSA to all wells.

- Incubate 1 h at 37°C.

- Wash plates 3x with DPBS-T for 5 min each wash.

- Prepare cell staining solution in DPBS-BSA:

  - Secondary antibody (e.g. Goat Anti Rabbit Alexa 488) - 1:1000

  - Draq5 - 1:10,000

  - Sapphire700 - 1:1000

- Add 50 uL cell staining solution to all wells that were treated with primary.

Add 50 uL of DPBS-BSA with secondary antibody (1:1000) only to background control wells.

- Remove solution.

- Incubate 1 h at 37°C.

- Wash 3x with 150 uL DPBS-T.

- Add 50 uL of water.

- Read plates on LICOR Odyssey Imaging System.

  - Focus offset = 3.0 (depends on the plates)

  - 700nm Intensity = 6.5

  - 800nm Intensity = 7.5

Plates and reagents:

Corning Black with clear bottom TC treated 96 well plates (Corning #3904)

DRAQ5 #4084 from Cell Signaling

Sapphire 700 #928-40022 from LICOR

Antibody 800 from LICOR (Life Technologies ones don’t work for this)