Reovirus Labeling with Succinimidyl Ester Dyes

- Dilute 3x1012 particles in up to 495 l of fresh 0.05 M sodium bicarbonate (pH 8.5)

  - Sodium Bicarbonate (FW = 84.01)

  - Make a 10X Stock (0.5 M) in ddH20 (0.42 g into 10 ml)

  - Make a 1X working solution (0.05 M) in ddH20

  - Filter Sterilize with a 0.2 m filter

- Make a 1 mM stock of the succinimidyl ester Alexa fluor in dimethylformamide

  - For Alexa 546 (MW = 1159.6)

    - 862 l dimethylformamide into 1 mg of dye

- Add 5 l of 1 mM Alexa dye to virus-sodium bicarbonate solution

  - In effect, you are labeling the virus with 10 M of dye

- Rock for 90 minutes in the dark at room temperature or 4C

- Dialyze in PBS overnight at 4°C, replacing with the PBS after the first hour of dialysis

- Store virus at 4°C

Note: for a Mock control, simply follow the above steps without including viral particles. In other words, add 5 l of 1 mM dye to 500 L of 0.05 M sodium bicarbonate and proceed with the labeling protocol.