Reagents and Materials:

- Lymphoprep (stem cell) or equivalent

- Sterile PBS

- 15 and 50 mL conical tubes

- Cryovials

- 10 mL and 25 mL Serological Pipettes

- Pipet tips, filtered

- Freezing media (bambanker)

Preparation

- Place reagents and materials PBS, Ficoll, and blood tubes  on orange rack in the hood and disinfect by spraying your hands, the hood, everything in the hood with 70% reagent alcohol.

Procedure

- If first blood draw or if not previously collected; Gently invert 1 blood tube (vacutainer) several times. Uncap tube and

  - aliquot 250uL of whole blood in a 1.5 eppendorf tube and save in 4C for haplotyping

  - aliquot 1mL of whole blood into a 15mL conical tube and save in -20C freezer for DNA isolation

  - Recap the blood tube

- Take all blood tubes and centrifuge at 500 x G for 10 minutes to separate plasma from whole blood

  - Remove 5 mL of plasma into a 15mL conical, mix by inversion or pipet.

  - Aliquot 1mL plasma into labeled cryovials (OnCore labels). Make 5 aliquots

  - Replace volume of removed plasma with equal volume of sterile PBS

- Pour or pipet blood into each 50mL conical tube. Each tube should be relatively equal volume

- Note: Do not exceed 20 mL of whole blood per 50mL conical tube.

- To maximize yield, using a 10 mL serological pipette, rinse blood tubes with 5 mL PBS per 2 tubes. Add PBS/blood mixture to collection 50mL conical tube after each set of two rinses.

- Make a 1:1 PBS/Blood mixture. Fill up 50mL tube with PBS to whatever twice the initial volume of whole blood was per tube.

- Gently invert 50mL tubes to mix.

- Using a 10mL serological pipet, aspirate 12.5 mL of Lymphoprep. Gently place pipet into bottom of 50mL conical and remove the entire pipette from pipet aid.

  - Lymphoprep and release gently into bottom of PBS/Blood mixture.

- Wait for pipette to drain. A layer of Ficoll will form at the bottom.

- Lift pipette up and stop at the interface, wait 3 seconds (for the last 1 mL of Ficoll to release), cap the pipette with your fingers, and continue to lift the pipette entirely out of the 50mL conical tub.

- Uncap your finger from the pipette and transfer the liquid into the liquid waste.

- Centrifuge for 800g for 20 minutes, acceleration at 4, deceleration at 4, and temperature should be at room temp.

- After centrifugation is complete, about 30mins, transfer PBMC layer to new 50mL collection tube by using a 10 or 25mL serological pipet.

  - Do not touch the red blood cell pellet

  - Remove as much of the PBMC layer as you can on the  first pass and then move on to the next tube. After you have done all the tubes, repeat again as a second pass.

  - If you position the pipette so that the end of the pipette is touching the 50mL conical tube wall and the tip is touching the front of the tube, you can stabilize your hand this way.

- Make a 1:1 PBMC layer/PBS. Aka Fill up 50mL conical tube to whatever twice the volume of the PBMC layer. Cap 50mL conical tube. Invert to mix.

- Centrifuge for 400 g for 8 minutes max acceleration: 9 and max deceleration: 9 at room temperature.

- During spin, clean up and prepare:

- Pour off supernatant with one quick motion. Resuspend pellet with 10 mL PBS. If you have multiple tubes use the same 10 mL to resuspend and wash all 4. Repeat rinse with another 10 mL PBS. Avoid Bubbles.

- Fill up resuspended pellet up to 50 mL PBS. The total volume doesn’t matter since this is just a wash.

- Invert 50mL conical tube to mix.  If you are counting, aliquot 100 uL of resuspension into Eppendorf tube.

- Centrifuge for 400 g for 5 minutes max acceleration: 9 and max deceleration: 9 at room temperature.

- During spin:

- Count cells and/or roughly estimate the total # of PBMCs you have.

  - 1-2 million PBMCs for every mL of whole blood is expected range

  - Aliquot cell numbers will be recorded for each visit

- Pour off supernatant. Resuspend pellet first in 1-2 mL of freezing media. Pipette up and down to mix. Then add the rest of the freezing media and mix well by pipetting.

  - Aliquot 0.5-1.5 mL of PBMCs in freezing media to cryovials. If you have any excess PBMCs, it should go into the last them.

  - Record aliquot cell number

- Transfer aliquots quickly to room temperature Mr. Frosty with isopropyl alcohol and transfer to -80°C.

  - Make sure it has isopropyl alcohol.

- Clean up:

  - If the waste bottle is full, take the waste bottle to the sink.

  - Place pipette aid back to charge.

  - Slide the hood door all the way down. Turn off the blower and switch off light to UV if you are the last person to use the hood for the day.

  - Spray hood