Protocol: IF staining Iba1 + CD68

General note

- Make sure that at each incubation step the tissue is flat and completely in touch with the liquid!!!

- 488: green channel

- 555: yellow/orange channel

- 647: red channel

- 405: blue channel

4 sections per animal

8

6

3

1

Rinse the sections with PBS

Day 1

- ANTIGEN RETRIEVAL

- Add 500 µl 1 M Citrate buffer (pH 6) in each well. Make sure the tissue is flat and completely on contact with the buffer!

- 30 min incubation at 80 °C, 20 min in the same buffer on ICE.

- 1X PBS Rinse

- 1X PBS Rinse

- 1X PBS 5 min on the wobbler

- 250 µl blocking step: in  1 x PBS 0.1% TritonX100  + 10% Donkey serum  (accordingly to the secondary Abs)  40 min – 1 h incubation on the wobbler.

- 250 µl Primary Abs (in 1 x PBS 0.1% TritonX100 + 10% Donkey serum  (accordingly to the secondary Abs) ON at RT on wobbler

Day 2

- 1 x PBS 0.1% TritonX100 rinse

- 1 x PBS 0.1% TritonX100 5 min on wobbler

- 1 x PBS 0.1% TritonX100 5 min on wobbler

- 250 µl secondary Abs 1/500 (in  1 x PBS 0.1% TritonX100) 2h RT on wobbler (dark by covering with aluminium foil)

- 1 x PBS rinse

- 1 x PBS 5 min on wobbler

- 1 x PBS 5 min on wobbler

- Briefly rinse sections in ½ PBS + ½ AD, mount them on a glass slide and allow them to dry ON while covered.

- Cover with 80 µl Mowiol and if necessary DAPI, 1/500 (stains nucleus, 405 nm). Use cover glasses #1.5.