| Title: | Live/Dead qPCR of B. pertussis IS481 |
| --- | --- |
| Scope: | Determination of the number of live and dead B. pertussis in a sample. |

| Written by: | Iain MacArthur | Date: | 5/6/17 |
| --- | --- | --- | --- |
| Approved by: |  | Date: |  |
| Cleared for issue: |  | Date: |  |

RECORD OF AMENDMENTS:

| VERSION | SECTION | AMENDMENT | CURRENT VERSION |
| --- | --- | --- | --- |
| 1.0 | 4.4, 4.5 | 10min dark, 5min light | 1.1 |

- Purpose / Introduction

This protocol describes the use of qPCR for the quantitative determination of live B. pertussis bacteria in a sample. The method uses propidium monoazide, a compound that can cross the cell wall of dead bacteria and binds DNA following photo activation to inhibit amplification of DNA from dead bacteria. This method uses a TaqMan probe specific to IS481 to ensure maximum sensitivity due to the high copy number of the target. Although IS481 is not specific to B. pertussis for experimental purposes, it provides acceptable specificity.

- Equipment/Reagents/Materials

  - qPCR machine (Applied biosystems, StepOne Real-Time PCR system)

  - Centrifuge (96-well plate)

  - Microfuge

  - PMA Lite box (VWR)

  - Water

  - Clear microfuge tubes

  - 2ml microfuge tubes

  - QiaAMP kit (Qiagen, 51304)

  - RNase A (Sigma, R4642)

  - 20mM PMA (Biotium, 40013)

  - MicroAmp FAST 96-well plates (Applied Biosystems, 4346907)

  - TaqMan Gene Expression Master mix (Applied Biosystems,4369016)

  - Filter tips 1000, 200, 20, 2 µl

  - Sealing film (Applied Biosystems. AB-0558)

  - Primers (IS481F/R)

  - Probe (IS481)

  - Control DNA for standard curve (Purified DNA from BP1917 or strain being tested)

|  | Sequence (5’-3’) |
| --- | --- |
| IS481 Forward Primer | ATCAAGCACCGCTTTACCC |
| IS481 Reverse Primer | TTGGGAGTTCTGGTAGGTGTG |
| IS481 Probe | FAM-AATGGCAAGGCCGAACGCTTCA-BHQ1 |

- Principle

To use PMA to inhibit the PCR amplification signal of DNA from dead bacteria. Purified DNA will be used as a template for the quantification of IS481 using a TaqMan probe. A standard curve consisting of DNA from pure BP1917 will be used to determine absolute quantity in unknowns.

- Procedure

PMA treatment

  - Pellet fresh samples by centrifuging at 2,000 x g for 10 minutes and resuspend in 1.2 ml of PBS.

  - Transfer 200µl of resuspended sample into two clear microfuge tubes (continue with 1 sample for 4.3 and 1 sample skip to 4.4).

  - Add 0.5µl of PMA to one of the 200µl of samples from 4.2.

  - Incubate microfuge tubes in the dark for 10 min at room temperature. Cover samples with aluminium foil and incubate on a rocker.

  - Expose samples to light using the PMA-Lite™ LED Photolysis Device for 5 min.

DNA purification

  - Add 20µl of QIAGEN Protease and 4 µl of RNase A.

  - Add 200µl of Buffer AL and vortex for 15 s/

  - Incubate at 56ºC for 10 min.

  - Briefly spin tube to recover condensation.

  - Add 200µl of ethanol (96-100%), and vortex and spin briefly.

  - Add to spin column and centrifuge for 1 min at 6000xg.

  - Transfer to a new collection tube.

  - Add 500µl AW1 buffer and spin for 1 min at 6000xg.

  - Transfer to a new collection tube.

  - Add 500µl AW2 buffer and spin for 3 min at 17000xg.

  - Transfer to a 2ml microfuge tube.

  - Spin at max speed for 1 min.

  - Transfer to a 1.5ml microfuge tube.

  - Add 200µl of Buffer AE and incubate at room temperature for 1 min.

  - Elute by centrifuging for 1 min at 6000xg.

qPCR

  - Dilute stock primers (50µM) 9 in 50µl, to give a reaction concentration of 900nm.

  - Dilute stock probe (50µM) 3 in 100µl, to give a reaction concentration of 150nm.

  - Mastermix per reaction:	TaqMan MM		10µl		1x

Forward primer	2µl		900nM

Reverse primer	2µl		900nM

Probe			2µl		150nM

  - Add 16µl of master mix per well.

  - Serial dilute 9 times, 5:50 positive control DNA (25ng/ul).

  - Use dilutions 3-9 for 1000, 100, 10, 1, 0.1, 0.01, 0.001pg.

  - Load in triplicate 4µl of DNA for each control dilution, unknown and water samples.

PCR and analysis

  - Cover plate with film and spin for 1 min at 1000xg.

  - Cycle:	50ºC	2min

95ºC	10min

40 cycles:	95ºC	15sec

60ºC	1min

- Results

  - Run analysis on data using StepOnePlus™ Software.

  - Outliers of triplicate samples should be ignored (omit).

  - Confirm standard curve has an r2 >0.95.

  - Convert pg of DNA to copy number:

B. pertussis/sample=(50x(mass in pg)x6.022x1023)/4102186*1x1012x650)

  - Untreated sample (no  PMA), represents total B. pertussis in the sample.

  - PMA treated sample gives number of live B. pertussis in the sample.

  - The number of dead can be determined from the difference between untreated and treated.

- Risk Assessment

Good Laboratory Practice must be followed. Wear laboratory coats and gloves.