Children’s Research Institute

Center for Genetic Medicine

Summar/Chapman’s Laboratory

Laboratory Procedure #010

Procedure: Mitochondria extraction from liver tissue flash frozen in liquid nitrogen

Prepared by: Elena Pumbo

Date adopted: 03/03/16

Date modified: 4/20/20

INTRODUCTION	3

REAGENTS	3

MATERIALS AND EQUIPMENT	4

INSTRUMENTATION	5

Mettler Toledo pH Meter	5

FastPrep-24 5G Homogenizer MP Biomedicals	6

Multipurpose Centrifuge Eppendorf Model 5430	7

BUFFERS and solutions	9

70% Ethanol                                                                                                                                                                                                      9

Tissue Lysis Buffer	9

Cell Lysis Buffer	10

Cell Lysis buffer pH 7.4 with Proteinase inhibitor cocktail tablets	11

PROCEDURE	12

Tissue preparation	12

Mitochondria Extraction	13

Mitochondria Protein Extraction	15

INTRODUCTION

A method to isolate functioning mitochondria from frozen liver tissue.

This protocol is based on following papers: C. Fezza at all Organelle isolation: functional mitochondria from mouse liver, muscle and cultured filroblasts, 2007; Th.Ranault at all,  Mouse Liver Mitochondria Isolation, Size Fractionation, and Real-time MOMP Measurement, 2016; I. Amigo at all, Isolating Liver Mitochondria by Differential Centrifugation, 2016.

REAGENTS

|  | Product | Company | Catalog Number | Storage Condition |
| --- | --- | --- | --- | --- |
| 1 | PBS pH 7.4 | AMERESCO | 23209 | room temp |
| 2 | Tris Base | FISHER | BP152-1 | room temp |
| 3 | NaCl | VWR | BDH9286.2.5 | room temp |
| 4 | EDTA 0.5 M, pH 8.0 | Sigma - Aldrich | 03690-100ML | room temp |
| 5 | Sucrose | Sigma - Aldrich | S5016-1KG | room temp |
| 6 | Proteinase inhibitor cocktail tablets | Sigma - Aldrich | 5892791001 | 4oC |
| 7 | HCl 5N | FISHER | 35011 | room temp |
| 8 | Calibration Buffer for pH Meter pH 4 | FISHER | 13-300-154 | room temp |
| 9 | Calibration Buffer for pH Meter pH 10 | FISHER | 13-300-152 | room temp |
| 10 | Ethanol 99.9% | Sigma - Aldrich | E7023-4L | room temp |
| 11 | Triton-X100 | Sigma - Aldrich | X100-500ML | room temp |

Miscellaneous Reagents:

- Milli-Q water

- Dry ice

- Regular ice

- 70% ethanol in Milli-Q water

MATERIALS AND EQUIPMENT

|  | Equipment | Company | Catalog Number |
| --- | --- | --- | --- |
| 1 | Multipurpose Centrifuge Eppendorf Model 5430 | VWR | 80094-180 |
| 2 | Rotor FA-45-48-11 for 48 2 mL tubes | VWR | FA-45-48-11 |
| 3 | Rotor F-35-6-30 for 6 50 mL and 5 15 mL tubes | VWR | F-35-6-30 |
| 4 | Vortex mixer | VWR | 97043-562 |
| 5 | Mettler Toledo Analytical Balance | FISHER | 30029067 |
| 6 | Toploading Balance | FISHER | 14-557-432 |
| 7 | FastPrep-24 5G Homogenizer | MP Biomedicals | 116005500 |
| 8 | QuickPrep-3 Sample Holder | MP Biomedicals | 6005512 |
| 9 | Lysing Matrix A, 2 mL tubes | MP Biomedicals | 116910050-CF |
| 10 | Porcelain Mortar and Pestle 145ml | VWR | 89038-148 |
| 11 | Stirring Hotplate | FISHER | 07-770-101 |
| 12 | Mettler Toledo pH Meter | FISHER | 01-912-345 |
| 13 | Sterile Disposable Filter Units 0.2mm 250 ml | FISHER | FB12566502 |
| 14 | Liquid Nitrogen Container | FISHER | 13-262-780 |

Miscellaneous equipment:

- Microcentrifuge tubes (1.7 mL)

- Microcentrifuge tubes (2.0 mL)

- Conical centrifuge tube (15.0 mL)

- Conical centrifuge tubes (50.0 mL)

- Micro-pipette tips (200 L)

- Micro-pipette tips (1000 L)

- Micro-pipettor (200 L)

- Micro-pipettor (1000 L)

- Disposable Graduated Transfer Pipettes

- Basket for dry ice

- Basket for regular ice

- Pipet Controller

- Measuring cylinder 100 mL

- Measuring cylinder 250 mL

- Beaker 500 mL

- Serological Pipettes (5 mL, 10 mL, 25 mL)

- Magnet stirring bars

- Weighing Paper 3 x 3 in.

- Weighing Dish 1.62 x 1.62 x 0.312 in.

- Weighing Dish 3.5 x 3.5 x 1 in.

- Kimwipe

- Q-tips Cotton Swabs.

- Parafilm 4 in (10 cm).

- Aluminum foil.

INSTRUMENTATION

- Mettler Toledo pH Meter

Instrument Set-Up

Mettler Toledo pH Meter is used for pH measurement. The instrument can only operate properly if the calibration was done before pH measurement in buffer.

## Calibration

## By calibration the pH meter is adjusted to zero point and slope of the electrode used normalized

Buffers for calibration: pH 4, pH 10.

- Press ON/OFF button on pH Meter to turn it on.

- Using 5mL serological pipette put 5 mL of Calibration Buffer pH 4 in one 15 mL conical tube.

- Label it.

- Using 5ml serological pipette put 5 mL of Calibration Buffer pH 10 in other 15 mL conical tube.

- Label it.

- Take an electrode from 3M KCl storage solution.

- Rinse with Milli-Q water

- Remove a liquid with Kimwipe

- Immerse electrode in 1st 15 ml tube with Calibration Buffer pH 4.

- Press “CAL” to activate calibration.

- Wait till numbers on display stabilize. Display should show “4.00”

- Take electrode from 1st tube and rinse with Milli-Q water.

- Remove a liquid with Kimwipe.

- Immerse electrode in 2nd 15 mL tube with Calibration Buffer pH 10.

- Press “CAL” to activate calibration.

- Wait till numbers on display stabilize. Display should show “10.00”

- Take electrode from 2nd tube and rinse with Milli-Q water.

- Remove a liquid with Kimwipe.

- Now pH Meter is ready.

Measurement

- Put the container with solution to be measured with magnet stirring bar in it on Stirring Hotplate.

- Turn on stirring, make speed medium.

- Immerse electrode in solution with to determine pH.

- Press “MEAS”

- Wait till numbers on display stabilize.

- To adjust pH use acid or base.

- Use micro-pipettor with tips (200 L or 1000 L) add drop by drop with stirring.

- Wait till numbers on display stabilize.

- If pH is not correct, add more.

- When pH become desirable stop stirring, take container with solution from Stirring Hotplate.

- Rinse electrode with Milli-Q water.

- Put in bottle with storage solution – 3M KCl.

- FastPrep-24 5G Homogenizer MP Biomedicals

The FastPrep-24 5G (FP5G) Instrument is a “high-speed, benchtop reciprocating instrument for the optimal lysis of challenging and routine sample types. It is intended for use in applications that require grinding, lysing or homogenization of various solid sample materials”.

Instrument Set-Up

- Switch on the homogenizer using the main power switch (O/ l), on the rear panel.

- Disengage the ‘Emergency Stop’ by turning clockwise until it pops out.

- When icons on touch screen appear, push “recommended programs”

- On next screen push “OPEN” to unlock the dome.

- Open dome, unscrew the sample holder lock.

- Take sample holder out and examine its condition.

- If plastic is damaged don’t use it.

- If plastic parts look whole it can be used.

- Have 2 cups of dry ice at the ready.

- Put dry ice in the reservoir under sample holder.

- Choose program from “Recommended Settings” screen.

      - Choose “organism”

      - Choose tissue type.

Homogenizing.

            - Aliquot tissue in Tissue Lysis Buffer for 2ml into Matrix A Tubs.

            - Close tight.

            - On sample holder turn and lift spoke plate.

            - Insert 2ml Martix A tubes with grinded tissue in Tissue Lysis Buffer into sample holder.

            - Turn and push spoke plate so it holds tubes on place.

            - Put sample holder onto ratchet bolt and rotate clockwise till it seated and ratchet bolt is long enough to secure sample holder lock. Tight it.

            - Close the dome until it clicked.

            - Check if dome is secure by trying to push it opened.

            - If dome is closed push “RUN” on touch screen.

            - Hold dome to minimize sound and vibration.

            - After program finished and vibration stopped push “OPEN” on touch screen.

            - Unlock sample holder.

            - Remove Matrix A Tubes from sample holder.

            - Remove dry ice from reservoir under sample holder.

            - Check if there are any dirt on sample holder, reservoir and dome. Remove dirt with 70% ethanol.

            - After all parts are clean put sample holder back, secure it with lock, close dome and turn homogenizer off.

- Multipurpose Centrifuge Eppendorf Model 5430

Instrument Set-Up

- Switch on the centrifuge using the main power switch on the side of centrifuge or standby key on the front panel.

- Open the closed centrifuge lid by pressing the “open” key on the front panel.

- The centrifuge can be operated with following rotors:

- Rotor FA-45-48-11 with aerosol-tight Quick Lock rotor lid for 48 tubes 1.5/2.0 mL.

Max. g-force:   18,213 x g outer ring, 16, 048 x g inner ring or 12,700 rpm.

- Rotor F-35-6-30 for 6 conical tubes 50 mL with or without skirted bottom and 6 conical tubes 15 mL, with or without enclosed adaptor.

Max g-force: 7,745 x g or 7,830 rpm.

- Check if correct rotor is inserted.

- Inserting the rotor.

        - With special rotor key (screwdriver) supplied by company with centrifuge unfasten the rotor.

        - Take it out with both hands and set aside.

        - Insert rotor you need – fit it vertically on the motor shaft.

        - Insert rotor key into the rotor nut.

        - Turn rotor key clockwise until the rotor nut is firmly tightened.

        - Close and secure lid.

- Setting speed.

- Use the “rpm/rcf” key to switch the speed of centrifugation display from rpm to g-force. Ensure that the g-force displayed during switching is standardized to suit the rotor.

- Use the “speed” arrow keys to set desired speed.

- Setting temperature.

- Using up and down keys under “Temp” sign set desired temperature.

- If it’s +40C and current temperature on the display is different, check if correct rotor is inserted, close and fasten rotor lid, close centrifuge lid and push “fast temp” key.

- After desired temperature is reached centrifuge will stop and beep.

- Automatic rotor detection.

It detects a newly inserted rotor during centrifugation and displays its name for approximately 2 seconds. The speed is automatically limited to the maximum permissible value of the rotor.

Centrifugation

            - Loading the rotor.

      - Load rotors only with the tubes intended for them.

      - Insert tubes opposite each other in pairs into the rotor bores.

      - To ensure symmetric loading, tubes that are arranged opposite each other must be on the same type and contain the same filling quantity.

      - Close and fasten rotor lid.

      - Gently push centrifuge lid to close.

- Press “start/stop” to start centrifuging.

- The centrifugation time is shown in a display during the braking process.

- A signal tone sounds when the rotor is at a standstill.

- The centrifuge lid remains closed to maintain the sample temperature. To open it press key “open”.

- Remove tubes from the rotor.

BUFFERS and SOLUTIONS

- 70% Ethanol

Store at room temperature

Procedure: Follow the table below (Table 1) to make 1 L of 70% ethanol

Table 1

| Order | Reagent | Volume/Weight | Final Concentration |
| --- | --- | --- | --- |
| 1 | Ethanol 99.9% | 700 mL | 70% |
| 2 | Milli-Q water | Up to 1 L | 30% |

- Using 1000 mL measuring cylinder add 700 mL of Ethanol 99.9%.

- Fill up to 1000 mL with Milli-Q water.

- Cover with Parafilm and secure it tight around cylinder opening.

- Put a palm of one hand around parafilm on cylinder opening and overturn cylinder 3 times to mix solution.

- Fill spraying bottle with 70% ethanol.

- Label it.

- Put the rest in the storage bottle for later use.

- Tissue Lysis buffer pH 7.4

Store at +40C

Composition: 1x Tissue Lysis Buffer

Procedure: Follow the table below (Table 2) to make 250 mL of 1x Tissue Lysis Buffer

Table 2

| Order | Reagent | Volume/Weight | Final Concentration |
| --- | --- | --- | --- |
| 1 | Milli-Q water | 100.00 mL |  |
| 2 | Tris Base | 0.152 gm | 5 mM |
| 3 | Sucrose | 21.39 gm | 0.25 M |
| 4 | 0.5 M EDTA | 0.5 mL | 1 mM |
| 5 | HCl (hydrochloric acid) 5N | To adjust pH to 7.4 |  |
| 6 | Milli-Q water | Up to 250.00 mL |  |

- Using 100 mL measuring cylinder add 100 mL of Milli-Q water to 500ml beaker

- Using an Analytical Balance and a weighing paper measure 0.152 gm of Tris Base.

- Add Tris Base to water in the beaker.

- Using an Analytical Balance and a weighing dish measure 21.39 gm of Sucrose.

- Add Sucrose to water in the beaker.

- Using 1000ml pipette with tip take 0.5 mL of 0.5 M EDTA.

- Add EDTA to water with other reagents in the beaker.

- Put magnet stirring bar into the beaker and put beaker on top of the Stirring Hotplate.

- Turn on stirring, but keep heater off.

- Measure pH with Mettler Toledo pH Meter. It’s about 9.8.

- Using 1000 mL pipette with tip take 1 mL of HCl (hydrochloric acid) 5 N and drop by drop add to solution in the beaker measuring pH in the process. Stop when it reaches 7.4.

- Rinse pH Meter sensor with Milli-Q water and put it back to KCl solution.

- Transfer Tissue Lysis buffer to 250 mL cylinder.

- Add Milli-Q water up to 250 mL.

- Using Sterile Disposable Filter Units 0.2 mm 250 mL filer sterilize Tissue Lysis Buffer.

- Remove and discard filter part, close bottle and label it.

- Now Tissue Lysis Buffer can be cooled on the ice or refrigerated and used or stored in +40C for 2 months.

- Cell Lysis buffer pH 7.4

Lysis buffer for mitochondria protein extraction.

Store at +40C

Composition: 1x Cell Lysis Buffer

Procedure: Follow the table below (Table 3) to make 250 mL of 1x Cell Lysis Buffer

Table 3

| Order | Reagent | Volume/Weight | Final Concentration |
| --- | --- | --- | --- |
| 1 | Milli-Q water | 100.00 mL |  |
| 2 | Tris Base | 1.52 gm | 50 mM |
| 3 | Sodium Chloride (NaCl) | 2.19 gm | 150 mM |
| 4 | 0.5 M EDTA | 0.5 mL | 1 mM |
| 5 | HCl (hydrochloric acid) 5 N | To adjust pH to 7.4 |  |
| 6 | Milli-Q water | Up to 250.00 mL |  |

- Using 1000 mL measuring cylinder add 1000 mL of Milli-Q water to 500ml beaker

- Using an Analytical Balance and a weighing dish measure 1.52 gm of Tris Base.

- Add Tris Base to water in the beaker.

- Using an Analytical Balance and a weighing dish measure 2.19 gm of Sodium Chloride (NaCl).

- Add to water in the beaker.

- Using 1000 mL pipette with tip take 0.5 mL of 0.5 M EDTA.

- Add EDTA to water with other reagents in the beaker.

- Put magnet stirring bar into the beaker and put beaker on top of the Stirring Hotplate.

- Turn on stirring, but keep heater off.

- Measure pH with Mettler Toledo pH Meter. It’s about 9.8.

- Using 1000ml pipette with tip take 1mL of HCl (hydrochloric acid) 5N and drop by drop add to solution in the beaker measuring pH in the process. Stop when it reaches 7.4.

- Rinse pH Meter sensor with Milli-Q water and put it back to KCl solution.

- Transfer Tissue Lysis buffer to 250 mL cylinder.

- Add Milli-Q water up to 250 ml.

- Using Sterile Disposable Filter Units 0.2mm 250 mL filer sterilize Tissue Lysis Buffer.

- Remove and discard filter part, close bottle and label it.

- Now Tissue Lysis Buffer can be cooled on the ice or refrigerated and used or stored in +40C for 2 months.

- Cell Lysis buffer pH 7.4 with Proteinase inhibitor (PI) cocktail tablets.

1 tablet of Proteinase inhibitor cocktail for 10 ml of Cell Lysis buffer pH 7.4.

Store at +40C for 2 weeks. Always write a preparation date on the storage container.

PROCEDURE

            - Tissue preparation

For 5 mL of 15-20 mg/mL of liver mitochondria protein we need 10-15 gm of frozen mouse liver tissue. If more of less protein needed scale these numbers up or down.

Cool down centrifuge and Rotor F-35-6-30 for 6 x 50 mL and 5 x 15 mL tubes. Set temperature +40C.

      - Take livers from -800C freezer put on dry ice.

      - Measure weight of a 10x10 cm square of aluminum foil using Mettler Toledo Analytical Balance and write it down.

      - Put on the foil unwrapped frozen livers up to 15 gm (about 10 livers).

      - Subtract foil weight.

      - Write number.

      - Keep livers on ice.

      - Bring liquid nitrogen in special 1 L liquid nitrogen container.

      - Take ceramic mortar and pestle

      - Fill mortar with liquid nitrogen and put pestle on dry ice cool down.

      - Put metal spatula on dry ice to cool down.

      - After liquid nitrogen evaporate mortar is cod enough.

      - Prepare 50 mL conical tube and put it on dry ice.

      - Put 1st frozen liver in the mortar and add ½ liquid nitrogen up to ½ mortar volume.

      - Wrap pestle with paper towel to avoid frost bite and cover mortar with paper towel.

      - Grind liver till there is only powder in the mortar.

      - With cold spatula put grinded tissue in the 50 mL tube.

      - Repeat with next liver till all of them are grinded.

      - Put mortar and pestle aside to warm up.

      - Clean the bench with 70% ethanol.

      - Prepare fresh 50 mL conical tube.

      - Measure weight using Mettler Toledo Analytical Balance and wright it down.

      - Transfer in fresh tube 10 gm of grinded liver powder (subtract weight of empty tube from weight of the tube with tissue).

      - Extra grinded liver store in -800 C.

      - To the tube with 10 gm of frozen liver add Tissue lysis Buffer up to 50 mL.

      - Wait till all tissue thawed. Vortex from time to time.

      - Prepare 25 Matrix A 2 mL tubes for MP Homogenizer.

      - When tissue is fully thawed using transfer pipettes aliquot 50 mL in Martix A tubes.

      - Following instruction for MP Homogenizer choose on touch screen

  - Recommended Settings

  - Animal

  - Mouse

  - Liver

  - Run

- After program finished put tubes with homogenized liver on regular ice.

- Clean and turn off MP Homogenizer

- Using transfer pipette transfer homogenate into fresh 50ml tube.

2. Mitochondria Extraction

- Prepare 50 mL conical tube with water for balance

- Using transfer pipette transfer homogenate from Matrix A tubes into fresh 50ml tube.

- Label tube “Pellet 1”

- Put 50 mL tube with homogenate and balance into Rotor F-35-6-30.

1st slow centrifugation.

- Close rotor lid tight.

- Close centrifuge.

- Set centrifuge:

  - speed 2000 rpm

  - time 10 minutes

  - temperature +40C

- After centrifugation finished, take tube from centrifuge and put on ice.

- Transfer supernatant into fresh tube. Label tube “Supernatant 1”

- Discard pellet.

- Adjust balance volume to new sample volume.

- Put 50 mL tube with homogenate and balance into Rotor F-35-6-30.

1st fast centrifugation

- Close rotor lid tight.

- Close centrifuge.

- Set centrifuge:

  - speed 7830 rpm

  - time 20 minutes

  - temperature +40C

- After centrifugation finished, take tube from centrifuge and put on ice.

- With q-tips remove white fat floating on the top of supernatant.

- Discard supernatant.

- Resuspend pellet in 25 mL of Tissue Lysis Buffer.

- Transfer into fresh tube. Label tube “Pellet 2”

- Adjust balance volume to new sample volume.

- Put 50 mL tube with homogenate and balance into Rotor F-35-6-30.

2st slow centrifugation

- Close rotor lid tight.

- Close centrifuge.

- Set centrifuge:

  - speed 2000 rpm

  - time 10 minutes

  - temperature +40C

- After centrifugation finished, take tube from centrifuge and put on ice.

- Transfer supernatant into fresh tube. Label tube “Supernatant 2”

- Discard pellet.

- Adjust balance volume to new sample volume.

2st fast centrifugation

- Close rotor lid tight.

- Close centrifuge.

- Set centrifuge:

  - speed 7830 rpm

  - time 20 minutes

  - temperature +40C

- After centrifugation finished, take tube from centrifuge and put on ice.

- Change rotor for Rotor FA-45-48-11 for 48 2 mL tubes.

- With q-tips carefully remove fat floating on top of supernatant.

- Discard supernatant.

- Resuspend pellet in 12 mL of Tissue Lysis Buffer.

- Transfer for 2 mL into fresh 2 mL microcentrifuge tubes.

- Load rotor distributing tubes equally.

Wash centrifugation

- Close rotor lid tight.

- Close centrifuge.

- Set centrifuge:

  - speed 12,700 rpm

  - time 20 minutes

  - temperature +40C

- After centrifugation finished, take tube from centrifuge and put on ice.

- Discard supernatant.

- Pellets resuspend in 5-6 mL of Tissue Lysis Buffer and transfer into 15 mL conical tube.

- Put back Rotor F-35-6-30 for 15 and 50 mL tubes.

- Set centrifuge:

  - speed 7830 rpm

  - time 20 minutes

  - temperature +40C

- After centrifugation finished, take tube from centrifuge and put on ice.

- Discard supernatant.

3. Mitochondria Protein Extraction

- Prepare 10 mL of Cell Lysis Buffer with 1 tablet of Proteinase Inhibitor (PI) cocktail.

- Label tube with preparation date.

- Using 5 mL serological pipette add to mitochondria pellet up to 5 mL of Cell Lysis Buffer.

- Using 200 mL pipettor with 200 mL pipette tip add 50 mL of Triton X-100.

- Pipette up and down 20 times to make sure all Triton X-100 was washed from the tip.

- Resuspend the pellet.

- Incubate 15 minutes in +40C (in cold room) with rotation.

- Transfer for 2 mL into fresh 2 mL microcentrifuge tubes.

- Change rotor for Rotor FA-45-48-11 for 48 x 2 mL tubes.

- Load rotor distributing tubes equally.

- Close rotor lid tight.

- Close centrifuge.

- Set centrifuge:

  - speed 12,700 rpm

  - time 20 minutes

  - temperature +40C

- After centrifugation finished, take tube from centrifuge and put on ice.

- Transfer supernatant into 5 mL tube.

Next step – protein concentration measurement. Summar Lab SOP #4 “Protein Quantification by Bradford Assay”