gDNA Agar Plug Extractions 2018

Buffers:

- 1 M sorbitol (filter sterilized)

- SPEM solution (1 M sorbitol, 10 mM EDTA pH 7.5, Na2HPO4-7H20 (2.08gL-1), NaH2PO4-1H20 (0.32gL-1))

- Protoplasting solution (4.56 mL SPEM, 200 uL Zymolase-T100 (50 mg/mL dissolved in H20), 200 uL lysozyme (25 mg/mL), 40 uL B-mercaptoethanol)

- 1x TAE (40 mM Tris, 20 mM acetic acid, 1mM EDTA, sterilized by autoclaving)

- Wash buffer (20 mM Tris, 50 mM EDTA, pH 8.0)

- Proteinase K solution (100 mM EDTA (pH 8.0), 0.2% sodium deoxycholate, 1% sodium dodecyl sulfate, 1 mg ml-1 Proteinase K)

- Pronase solution (50 mM EDTA (pH 8.0), 0.5% sodium dodecyl sulfate, 500 ug mL-1 Pronase)

# Agarose Prep

- Add 0.5 g of LMP agarose to 25 mL TAE (2% final)

- Microwave for 30 secs, or until solution begins to bubble

- Repeat heating steps as necessary (2-3x)

- Move flask to 50oC air incubator, swirling a few times to bring agarose into solution

# Gel Plugs

- Thaw 50 mL conical vial with cell pellets on ice (cell pellet ~1.5g for this expt)

- Add 50 mL 1M sorbitol and resuspend pellet gently

- Let cells sit on ice 1 min

- Harvest pellet with 10 min spin at 3000 x g at 4oC

- Remove sorbitol wash supernatant

- Resuspend cells in 5 mL protoplasting solution

- Incubate cell suspension at 37oC for 10 min, then remove from water bath

- Keeping agarose warm, mix 5 mL of 2% agarose with cell suspension

- Pipette 100 uL of cell-agar solution into disposable plug molds (Biorad #170-3713)

Note:  wide-bore tips can make it easier to dispense agar-cell solution into the molds if regular tips keep clogging

- Place molds at 4oC for 15-30 min to solidify

- Once solid, transfer plugs to new 50 mL conical vial

- Wash plugs with 25 mL of wash buffer (30 min at RT with soft shaking)

- Add 10-20 mL of proteinase K solution (enough to cover all of the plugs)

- Incubate for 24 hrs at 50oC

- Wash plugs 2x with 25 mL of wash buffer (30 min at RT)

- Add 10-20 mL of Pronase solution to plugs

- Incubate for 4 hrs at 37oC

- Wash plugs 2x with 25 mL wash buffer (30 min at RT)

- Store at 4oC

# Note: Plugs can be left in wash buffer after this step for up to 1 month at 4oC

# Agarose digestion

- Remove 3-4 plugs (or more) from conical vial and transfer into new Eppendorf tube

- Wash 2x with 500 uL of 1x beta-agarase wash buffer (NEB) on ice for 30 min each cycle

- Remove excess wash buffer from tube with pipette

- Heat tube at 65oC for 10 min in heating block

- Cool block and solution to 42oC (about 10 min, agarase gets inactivated over 45oC)

- Add 1U Beta-agarase per 100 uL melted agarose volume

- Incubate for 1-2 hr at 42oC

- Allow tube to cool to RT

- Store at 4oC overnight or move on to extraction (below)

# Phenol extraction for HMW DNA

- Mix 250 uL of liquefied agar solution with 250 uL of Tris-buffered phenol

- Rock end over end slowly for 10 min at RT

Note: Extractions can go for up to an hour at RT if greater DNA yield is desired

- Separate phases by centrifuging for 10 min at 16,100 x g

- Transfer aqueous layer (around 250 ul) to new tube

- Add 25 ul of 3M NaOAC to tube

Note: we’ve noticed that addition of the 3M NaOAc causes the aqueous layer to become turbid and opaque, so if you see this it’s not unusual

- Invert tube slowly 10 times until mixed

- Add 825 uL of 100% EtOH

- Invert tube slowly 10 times until mixed

- Allow to sit at -20oC for 1 hr minimum, overnight precipitations may increase DNA yield

- Centrifuge for 20 min at 16,100 x g at RT

Note:  make sure to have the centrifuge set to lowest acceleration/deceleration speed possible if you want to avoid shearing

- Remove supernatant gently with pipet

- Wash pellet quickly in 500 uL 70% EtOH + 50mM CaCl2

- Centrifuge for 5 min at 16,100 x g at RT

- Repeat 70% EtOH + 50 mM CaCl2 wash

- Centrifuge for 5 min at 16,100 x g at RT

- Remove wash solution

- Allow pellet to air dry for 5-10 min at RT

- Resuspend pellet gently in 50-100 uL TE, pH 8.0