Amplicon Protocol – Version 1.6

08/13/2017

Viral RNA extraction using QIAamp vRNA mini kit.

Primers (NOTE: HIV-1 ENV REGION AS AN EXAMPLE, SUBSTITUTE THE BLUE Sequences in the forward primer and cDNA primer)

| V1F(forward) 5’-3’ | GCCTCCCTCGCGCCATCAGAGATGTGTATAAGAGACAGNNNNTTATGGGATCAAAGCCTAAAGCCATGTGTA |
| --- | --- |
| BV3R Uni (cDNA Primer) 5’-3’ | GTGACTGGAGTTCAGACGTGTGCTCTTCCGATCTNNNNNNNNNCAGTCCATTTTGCTCTACTAATGTTACAATGTGC |
| Universal Adapter | AATGATACGGCGACCACCGAGATCTACACGCCTCCCTCGCGCCATCAGAGATGTG |
| Indexed Adapter | CAAGCAGAAGACGGCATACGAGATNNNNNNGTGACTGGAGTTCAGACGTGTGCTC |
| ADPT_2a | GTGACTGGAGTTCAGACGTGTGCTC |

Note: Primer ID primer and forward primer use random bases. Indexed primers have 24 fixed barcodes.

Table of Indexed Primers

| Indexed Primer | Index | Index Sequence | Sequence |
| --- | --- | --- | --- |
| PCR Primer, Index 1 | 1 | ATCACGA | CAAGCAGAAGACGGCATACGAGATCGTGATGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 2 | 2 | CGATGTA | CAAGCAGAAGACGGCATACGAGATACATCGGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 3 | 3 | TTAGGCA | CAAGCAGAAGACGGCATACGAGATGCCTAAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 4 | 4 | TGACCAA | CAAGCAGAAGACGGCATACGAGATTGGTCAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 5 | 5 | ACAGTGA | CAAGCAGAAGACGGCATACGAGATCACTGTGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 6 | 6 | GCCAATA | CAAGCAGAAGACGGCATACGAGATATTGGCGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 7 | 7 | CAGATCA | CAAGCAGAAGACGGCATACGAGATGATCTGGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 8 | 8 | ACTTGAA | CAAGCAGAAGACGGCATACGAGATTCAAGTGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 9 | 9 | GATCAGA | CAAGCAGAAGACGGCATACGAGATCTGATCGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 10 | 10 | TAGCTTA | CAAGCAGAAGACGGCATACGAGATAAGCTAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 11 | 11 | GGCTACA | CAAGCAGAAGACGGCATACGAGATGTAGCCGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 12 | 12 | CTTGTAA | CAAGCAGAAGACGGCATACGAGATTACAAGGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 13 | 13 | TCCATAA | CAAGCAGAAGACGGCATACGAGATTATGGAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 14 | 14 | GTACTAA | CAAGCAGAAGACGGCATACGAGATTAGTACGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 15 | 15 | ACAGTAA | CAAGCAGAAGACGGCATACGAGATTACTGTGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 16 | 16 | CTCATGA | CAAGCAGAAGACGGCATACGAGATCATGAGGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 17 | 17 | ACGATAA | CAAGCAGAAGACGGCATACGAGATTATCGTGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 18 | 18 | TGCAGAA | CAAGCAGAAGACGGCATACGAGATTCTGCAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 19 | 19 | TTCATAA | CAAGCAGAAGACGGCATACGAGATTATGAAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 20 | 20 | TGCTGTA | CAAGCAGAAGACGGCATACGAGATACAGCAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 21 | 21 | TATCACA | CAAGCAGAAGACGGCATACGAGATGTGATAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 22 | 22 | TGGATAA | CAAGCAGAAGACGGCATACGAGATTATCCAGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 23 | 23 | CGCATTA | CAAGCAGAAGACGGCATACGAGATAATGCGGTGACTGGAGTTCAGACGTGTGCTC |
| PCR Primer, Index 24 | 24 | GCCTTAA | CAAGCAGAAGACGGCATACGAGATTAAGGCGTGACTGGAGTTCAGACGTGTGCTC |

For multiplexing sequencing, first prepare Primer Mix.

Example:

| Regions | DR cDNA primer | DR F primer |
| --- | --- | --- |
| PR | R2614_PID | F2163AD |
| RT | R3284_PID11 | F2620_AD |
| INT | R4752_PID11 | F4383_AD |
| V3 | V3R_Buni_11 | V1F_AD |

- Make 10 µM primer mix: mix 10 µL of each primer in one set and 60 µL of dH2O.

- Mix well.

cDNA synthesis protocol

- Pipette the following components into a 0.5 ml RNase-free tube:

| μl/tube |  | [stock] | [final] | [mastermix] |
| --- | --- | --- | --- | --- |
| 2.0 | dNTP Mix | 10 mM each | 0.5 |  |
| 1.0 | cDNA primer | 10 μM | 0.25 μM |  |
| 23.0* | RNA template |  |  |  |
| 26.00 | Total volume |  |  |  |

* The input volume of vRNA depends on the viral loads. We recommend 1,000 to 20,000 copies, but it can work with samples as little as 200 copies. If copy number unknown, use the maximum volume of 23 ul.

- Place tube in 65°C heat block for 3-5’, remove to ice for 1’.

- Add the following components:

| μl/tube |  | [stock] | [final] | [mastermix] |
| --- | --- | --- | --- | --- |
| 8.0 | 5x buffer | 5x | 1x |  |
| 2.0 | DTT | 100 mM | 5 |  |
| 2.0 | RNaseOUT | 40 u/μl | 2 |  |
| 2.0 | SSIII RT | 200 u/μl | 10 |  |
| 14.0 | Per tube |  |  |  |

- Mix and incubate at 50°C for 1 hr and then increase to 55°C for 1 hr.

- Inactivate SSIII RT by heating at 70°C for 15’.

- To each tube, add 0.5 μl RNase H, incubate at 37°C for 20’.

- Purify cDNA using Agencourt RNAClean XP

  - Resuspend the beads and take an aliquot out. Keep in room temperature for at least 30 minutes before use. (Should be in 1ml aliquots)

  - Resuspend the beads (Vortex). Add 28 µl of beads Agencourt RNAClean XP beads to 1.5 ml tubes.  Transfer all cDNA reaction (40 µl) to each tubes with beads.

  - Mix the Agencourt RNAClean XP and sample thoroughly by pipette mixing 15 times. No votexing. Let the tube incubate at room temperature for 20 minutes.

  - Place the tube onto the magnetic tube rack for 5 minutes to separate the beads from solution.

  - Slowly aspirate the cleared solution from the tube and discard. This step should be performed while the tube is situated on the rack. Do not disturb the magnetic beads, which have formed a spot on the side of the tube.

  - Dispense 400 μL of 70% ethanol into the tube and incubate for 30 seconds at room temperature. Aspirate out the ethanol and discard. Repeat for a total of TWO washes. It is important to perform these steps with the tube situated on the rack. Do not disturb the separated magnetic beads. Be sure to remove all of the ethanol from the bottom of the well as it may contain residual contaminants.

  - Let the reaction tube air-dry up to 10 minutes on the rack with the cap open. The tube(s) should air-dry until the last visible traces of ethanol evaporate. Over drying the sample may result in a lower recovery.

  - Remove tube from rack and resuspend beads in 40 µl DNase-free water by pipetting up and down. Place tube back on rack and leave for 3 min.

  - Pipette the eluant from the tube while it is situated on the magnetic tube rack.

  - DOUBLE WASH: repeat step c to g

  - Remove tube from rack and resuspend beads in 24 µl DNase-free water by pipetting up and down. Place tube back on rack and leave for 3 min.

  - Pipette the eluent (23.5 µl) from the tube while it is situated on the magnetic tube rack. It is fine to have some beads in the eluent when transferring.

PCR 1

1. Complete thaw and votex KAPA reagents (except for enzyme) before use.

2.

| μl/tube |  | [stock] | [final] | [mastermix] |
| --- | --- | --- | --- | --- |
| 10.0 | 5x Buffer A | 5x | 1x |  |
| 10.0 | Enhancer | 5x | 1x |  |
| 1.0 | dNTPs | 10 mM | 0.2 mM |  |
| 2.5 | Forward primer | 10 µM | 0.5 µM |  |
| 2.5 | ADPT_2a | 10 µM | 0.5 µM |  |
| 0.5 | KAPA Robust polymerase | 5 U/µl | 2.5 U |  |
| 23.5 | Template cDNA |  |  |  |

Prepare master mix in cold box and use repeater pipette to add to each tube in the cold box.  Add template cDNA to each tube and pipette up and down to mix.

3. Cycle

| 95˚C | 1 min |
| --- | --- |
| 95˚C | 15 s |
| 58˚C | 1 min |
| 72˚C | 30 s |
| 15-25 cycles |  |
| 72˚C | 3 min |
| 4˚C | On hold |

4.  Purify PCR products using AmpureXP PCR cleanup kits.

  - Vortex the 1 ml aliquot and remove the needed volume. Keep in room temperature for at least 30 minutes before use.

  - Transfer the PCR1 reactions into 1.7 mL RNase-free tubes.

  - Resuspend the beads. Add 35 µl Ampure XP beads to each cDNA reaction.

  - Mix the Ampure XP and sample thoroughly by votexing. Let the tube incubate at room temperature for 5 minutes before proceeding to the next step (incubate off the rack).

  - Place the tube onto the magnetic tube rack for 5 minutes to separate the beads from solution.

  - Slowly aspirate the cleared solution from the tube and discard. This step should be performed while the tube is situated on the rack. Do not disturb the magnetic beads, which have formed a spot on the side of the tube.

  - Dispense 500 μL of 70% ethanol into the tube and incubate for 30 seconds at room temperature. Aspirate out the ethanol and discard. Repeat for a total of two washes. It is important to perform these steps with the tube situated on the rack. Do not disturb the separated magnetic beads. Be sure to remove all of the ethanol from the bottom of the well as it may contain residual contaminants.

  - Let the reaction tube air-dry 10 minutes on the rack with the cap open. The tube(s) should air-dry until the last visible traces of ethanol evaporate. Over drying the sample may result in a lower recovery.

  - Remove tube from rack and resuspend beads in 50 µl DNase-free water by pipetting up and down. Place tube back on rack and leave for 3 min.

  - Pipette the 45 µl eluant from the tube while it is situated on the magnetic tube rack.

PCR 2

1.  Complete thaw and votex KAPA reagents (except for enzyme) before use.

2.

| μl/tube |  | [stock] | [final] | [mastermix] |
| --- | --- | --- | --- | --- |
| 5.0 | 5x KAPA HiFi Fidelity Buffer | 5x | 1x |  |
| 1.0 | dNTP Mix | 10 mM | 0.4 mM |  |
| 1.0 | Uni Adapter (ADPT_P1) | 10 µM | 0.4 µM |  |
| 0.5 | KAPA HiFI polymerase | 1 U/µl | 0.5 U |  |
| 1.0 | Indexed Adapter | 10 µM | 0.4 µM |  |
| 2.0 | Purified template DNA |  |  |  |
| 14.5 | Water |  |  |  |

Prepare mastermix in cold box and use repeater pipette to add to each tube in the cold box.  Add Indexed Adapter to each tube.

3. Cycle (PCR machine #5 SZ -> ILM2):

| 95˚C | 2 min |
| --- | --- |
| 98˚C | 20 s |
| 63˚C | 15 s |
| 72˚C | 30 s |
| 25 - 35 cycles |  |
| 72˚C | 3 min |
| 4˚C | On hold |

Gel Purification and quantification

- Before gel purification, run 2 µl products on 1% agarose gel to check the bands.

- Gel purification. (Qiagen QIAquick gel extraction kit)

  - Run 2nd round PCR products on 1.2% agarose gel. E = 4 V/cm, T = 60 min.

  - Excise DNA fragment.

  - Weight the gel; add 3 volume of Buffer QG to 1 volume of gel.

  - Incubate at 50 ˚C for 10 minutes to completing dissolve. Vortex every 2-3 minutes to help dissolve.

  - Check the color of gel solution (should be yellow, otherwise add 10 µl 3M sodium acetate).

  - Place MinElute column, apply the sample to the column and centrifuge at 1 min.

  - Add 500 µl buffer QG and centrifuge for 1 min.

  - Add 0.75 ml buffer PE, incubate for 5 minutes at room temperature, centrifuge for 1 minute.

  - Discard the fluid, centrifuge for additional 3 minutes.

  - Put the column in a new 1.7 ml tube, add 10 µl buffer EB. Stand for 4 minutes, centrifuge for 2 minutes.

- Quantification using Invitrogen Qubit dsDNA BR Assay kit. See Qubit dsDNA BR assay protocol. Don’t use Nanodrop to quantify!

- After quantification, pool libraries in equal amount (usually choose 100-200 ng per library). Use AMPure XP beads to purify pooled libraries (beads to DNA ration 0.7), elute in 20 to 30 µL Elution Buffer.

- Quantify the pooled library using Qubit dsDNA BR assay kit. Check the quality of the library by Agilent Bioanalyzer or Bio-rad Experion. Repeat purification if primer dimmer is present (~200bp).