Phenol Chloroform Isoamyl (PCI) Protocol for RNA Extraction – Frank Xavier Ferrer-González, modified from Marine Landa, Summer 2017

Extraction

- Cut filters before if using, really fine. Prepare a 2.0 ml tube (Low Binding DNA) for each sample, with each tube containing 400 μl beads (mix of 200 μl 0.1 mm zirconium beads: 100 μl 0.4-0.6 mm glass beads(for phyto): 100 μl 0.5 mm zirconium beads “ashed; 4 hours, 500oC”).

- Add T1E1 buffer with SDS 0.6%, Proteinase K 120 ng/μl.  Vortex for 10 minutes, incubate at RT for 50 minutes.

  - Add 964μl of T1E1 buffer, 30 μl of 20% SDS (20% SDS stock), and 6 μl of proteinase K, total 1000 μl.

- Add the RNA standard - 0.5% of expected yield

- Transfer supernatant to a new 2.0 ml microtube (Low Binding DNA), careful not to transfer the beads.

- Centrifuge for 1 min at 8,000 rpm. If you have beads in your sample, repeat 4 and 5.

- Add Acidic Phenol:Chloroform:Isoamylalcohol (25:24:1) at a 1:1 volume. Shake until emulsified, don’t vortex…

- Incubate the mixture on ice for 15 min.

- Centrifuge at 13,400 rpm for 15 min, 4oC. Collect supernatant in a new 2.0 ml tube.

- Add 1 volume of Chloroform:Isoamylalcohol (24:1), shake until emulsified. Chlroform by itself will also work but it will create foam.

- Incubate the mixture 15 min, on ice.

- Centrifuge at 13,400 rpm for 15 min, 4oC. Collect supernatant in a new tube.

- Repeat 8,9,10 again

- Add 1 volume of isopropanol.  Mix with a 21g syringe and shear DNA (8-10 times), Incubate at -20oC overnight.

- Centrifuge 30 min, 13,400 rpm, 4oC. Remove supernatant in a new microtube, be careful not to lose the pellet. You can transfer the supernatant to a new microtube, in case you lose the pellet you can recover it. This pellet almost always sticks to the microtube, the wash step is the one that mostly gets lose.

- Add 1.0 ml 75% ethanol. I like to make ethanol at 80%, so I can store it for long term use.

- Centrifuge 10 min, 13,400 rpm, 4oC. Decant supernatant in a new microtube.

- Repeat steps 11, 12, 13.

- Short spin (~15s), remove remaining supernatant.

- Repeat the short spin and make sure there is no ethanol in the sample. If the pellet is wet it will look white, if it’s dry it will look clear.

- Resuspend in ~30 ul RNAse free water. This is depending on your experiment and needs, wait from 30 min to 1 hour on ice to take a reading, RNA takes time to dissolve in the sample.

- Proceed to other applications or store at -80oC.

Tips:

RNA is easy to work with; you just need to be very clean and fast with the samples. Don’t let the samples thaw, do one of them at a time. If you are fast you can run 6-12 samples at a time. Tips on working, clean the station very well and keep it clean at all times. Make sure all the pipette tips are very clean. Always clean all instruments before starting an extraction with ethanol and RNAseaway or equivalent. Add RNaseaway to your gloves, and always work on the clean zone. Some people like to put tape on their bench; I find it a bit overwhelming (your choice). Don’t talk or sing while doing samples… if you have a cold, don’t do samples. Also, this protocol uses a lot of microtubes, make sure you have the right ones at all time and before starting. The low binding DNA microtubes don’t make a pellet, keep that in mind. *Make sure you turn the cold microcentrifuge before starting.