|  | Department: Clinical Sciences | Unit: Tropical Laboratory Medicine |
| --- | --- | --- |
|  | Title: Susceptibility testing |  |
|  | Project/study: NA |  |

Contents

1	General information	2

1.1	Scope and application	2

1.2	Principles	2

1.3	Reagents	3

1.4	Equipment, utensils and materials	4

1.5	Sample for analysis	4

2	Abbreviations	4

3	Procedures	4

3.1	Preparing the inoculum	4

3.2	Inoculation of the agar plate	5

3.3	Preparing the disk diffusion and MIC-test	5

3.4	Incubation	5

3.5	Reading the results	6

4.  Guidelines per species or group of bacteria	7

4.1	Salmonella species	8

4.2	Yersinia (note: quality control not updated)	9

4.4	Enterobacteriaceae	10

4.5	Pseudomonas aeruginosa	11

4.6	Acinetobacter	11

4.7	Stenotrophomonas maltophilia	12

4.8	Burkholderia cepacia (note: quality control not updated)	12

4.9	Staphylococcus	13

4.10	Streptococcus pneumoniae	14

4.11	Streptococcus pyogenes, agalactiae and other large β-hemolytic Streptococcus of gr. C & G	16

4.12	Streptococcus viridans (note: quality control not updated)	16

4.13	Enterococcus (note: quality control not updated)	17

4.14	Haemophilus influenzae and Haemophilus parainfluenzae	18

4.15	Neisseria meningitidis	18

4.16	Neisseria gonorrhoeae (note: quality control not updated)	19

4.17	Vibrio cholerae	20

5.  Detection of resistance mechanisms	21

5.1 ESBL testing using the combination disk test for K. pneumoniae, K. oxytoca, E. coli, P. mirabilis and Salmonella spp.	21

5.2 Inducible clindamycin resistance testing using the D-test	21

6.	Quality control	22

6.1	First line quality control	22

6.2	Second line quality control	22

7.	Registration and processing results	22

8.	Method verification literature	22

9.	Records and archives	23

# General information

## Scope and application

The purpose of this procedure is to determine the susceptibility of a bacterial strain to an antimicrobial agent.

There are 3 possibilities:

S 	The strain is susceptible: The infection will respond to the therapy.

I	The strain is intermediate: The infection will respond in certain conditions when the concentration of the antimicrobial agent is very high.

R	The strain is resistant: The infection will not respond to the therapy.

The Clinical & Laboratory Standards Institute (CLSI) groups antibiotic agents according to their clinical relevance for different pathogens in 6 groups:

- Group A: agents considered appropriate for inclusion in a routine, primary testing panel, as well as for routine reporting of results for the specific organism groups

- Group B: agents that may warrant primary testing, but they may be reported only selectively

- Group C: alternative or supplemental agents that may necessitate testing in special circumstances (resistance to first-line drugs, allergies to first line drugs, treatment of unusual organisms, epidemiological aid, …)

- Group U (urine): agents used only or primarily for treating UTI

- Group O (other): agents not routinely tested and reported, but with clinical indication

- Group Inv (investigational): agents that are investigational for the organism group, not yet approved by FDA

## Principles

There are two possible methods for antibiotic susceptibility testing that are based on the following principles:

Disk diffusion: A standardized inoculum of the bacterium is spread upon the surface of a petri-dish with a specific solid culture medium (for specifics see further). Antibiotic disks, impregnated with a specific antibiotic, are placed on the culture medium. During overnight incubation, the antibiotic spreads through the agar around the disk, while the strain is also growing. If the strain is susceptible for the antibiotic in the disk, it will not be able to grow close to the disk.  In that case, you can see an area without growth around the antibiotic disk, this is called “inhibition zone”. After incubation the diameter of the inhibition zone around the antibiotic disk is measured.

The diameter is a measure for the susceptibility of the organism to the antibiotic. Based on defined criteria (which are mentioned below per antibiotic and per species of bacteria) a semi quantitative assessment of the organism’s susceptibility can be given; susceptible (S), intermediate susceptible (I) and resistant (R).

The picture shows different zones of inhibition around different antibiotic disks placed on a solid culture medium.

E-Test: This is a method that is based on a combination of dilution and disk-diffusion methods. The MIC (minimal inhibitory concentration) of a particular antibiotic is determined by using a thin, inert, non-porous strip with on the top side the MIC-indicator scale in µg/ml and a two-digit code for identification of the applied antibiotic, and on the bottom-side a dried and stabilized pre-determined exponential gradient of that antibiotic. The gradient is comprised of consecutive dilutions, which means that the concentration of antibiotic that is released by the strip decreases along the length of the stip. The zone of inhibition along the length of the strip will therefore also decrease. When a MIC test strip is applied to a petri-dish, the antibiotic is immediately released onto the culture medium in a way that creates a constant exponential gradient of the antibiotic in the culture medium underneath the strip. When the bacterial growth becomes clear after incubation a symmetrical ellipse-shaped inhibition zone is visible. This shape is the result of the difference in antibiotic concentration that is released along the length of the strip; where the antibiotic has a higher concentration the zone of inhibition will be larger than where the antibiotic has a lower concentration. The MIC value is measured on the MIC-indicator scale on top of the strip, at the intersection of the border of the ellipse and the strip.

The picture shows MIC-test strips with different shaped inhibition zones. The intersection between the inhibition zone and the test strip is the ‘MIC-value’ and is a measure of antibiotic susceptibility of the bacterium to that specific antibiotic.

## Reagents

- Neo-sensitabs (Rosco)

- E-Test Strips (Biomérieux)

- Sterile saline

- Müller Hinton 2 agar

- Müller Hinton 2 agar + 5% sheep blood (CLSI) or MH-F agar (EUCAST)

- HTM agar (Haemophilus test medium for AST of Haemophilus (para)influenzae, CLSI)

## Equipment, utensils and materials

- Incubator (33 – 37 °C)

- CO2-incubator (33 – 37 °C)

- Incubator (40 – 44 °C)

- Freezer (-80°C)

- Refrigerator (2-8°C)

- Inoculating loop

- Sterile swabs

- Bunsen burner

- Forceps

- 0,5 McF turbidity standard or densitometer

- Ruler

## Sample for analysis

A pure culture of 3-4 isolated colonies with identical morphology is used to detect antibiotic susceptibility.

# Abbreviations

ATCC 	American Type Culture Collection

CFU 	Colony forming unit

CG	Chocolate agar

CLSI 	Clinical and laboratory standards institute

CNS 	Coagulase negative Staphylococcus

CPE 	Carbapenemase producing Enterobacteriaceae

DST	Direct Susceptibility Testing

ESBL 	Extended spectrum beta-lactamase (broad spectrum beta-lactamase)

EUCAST	 European Committee on Antibiotic Susceptibility Testing

HTM	Haemophilus test medium
ITM       Institute of Tropical Medicine

MBL	Metallobetalactamase

MIC 	minimal inhibitory concentration: Minimal concentration of a specific antibiotic that inhibits the growth of a particular bacterium under specified in vitro circumstances.

MHA	Müller-Hinton agar

MH-F	Müller-Hinton fastidious

McF 	Mc Farland

MRSA	Methicillin resistant Staphylococcus aureus

QC	Quality Control

VRE 	Vancomycin resistant Enterococcus

WHO    World Health Organisation

# Procedures

All Petri-dishes and antibiotic disks / strips must be at room temperature before use. Prevent prolonged exposure of these products to higher temperatures.  In case of E-tests in multiple use packages (e.g. 100 strips per package), store the package after opening in a hermetically sealed recipient at -20°C with a desiccant. In case of single use packages, store the package according to the instructions on the package.

The 15-15-15 rule must be applied; this means there is not more than 15 minutes between the preparation of the inoculum and the inoculation of the plate; not more than 15 minutes between inoculation of the plate and placement of the antibiotic disks or strips; and not more than 15 minutes between the placement of the antibiotic disks/strips and incubation.

## Preparing the inoculum

- Use pure isolates from an overnight inoculated Petri-dish.

- Make a smooth suspension of 0.5 McF using 3 – 5 isolated colonies with identical morphology. Suspend them in the appropriate suspension medium (sterile saline)

  - NOTE: Using an inoculating loop or a cotton swab, pick only well-isolated colonies from the plate to avoid testing mixed cultures. If you do not have well-isolated colonies, subculture the organism to a fresh plate.

- Use the McF standard to compare turbidity

## Inoculation of the agar plate

- Dip a sterile swab into the inoculum

- Press the swab against the tube wall to remove excess fluid

- Swab the entire agar surface in one direction

  - Rotate the agar plate 60 degrees and swab again.

  - Again rotate the agar plate 60 degrees and swab again.

  - Pass the swab round the edge of the agar surface.

## Preparing the disk diffusion and MIC-test

This section explains how to place the antibiotic susceptibility disks on the inoculated agar plate.

Disk-diffusion:

- If available use a dispenser for the antibiotic disks to make sure that sufficient room is left between the different disks. There may be no overlap between the inhibition zones.

- If not available, place the antibiotic disks on the medium using sterile forceps.

- Press lightly on the disks to ensure that the disk makes full contact with the agar plate.

- Keep a minimal distance of 24mm between the disks (unless explicitly mentioned otherwise), this corresponds with maximal 7 disks on a 90mm petri dish.

- Do not move the disk after contact with the agar has been made, because the disk starts releasing the antibiotics as soon as it touches the agar.

E- Test:

- In case the agar plate is still condensed after inoculation of the bacterial strain, it is allowed to pre-incubate the plate before the application of the E-tests. The 15-15-15 rule does not apply in this case, it has been shown that application of the E-test can be done until 30 minutes after inoculation of the agar.

- Take one E-test of the correct antibiotic by the upper side of the strip (the zone indicated with 'E') using sterile forceps.

- Place the strip on the agar with the MIC scale facing upwards.

- Do not move the strip after it has touched the culture medium.

- Ensure that the whole strip is in full contact with the agar.

- Remove any potential air bubble that may have gotten under the E-test by gently pressing the strip using sterile forceps. Always perform this procedure stroking toward the side with higher concentrations antibiotics.

## Incubation

- Place the petri dishes in the incubator facing downwards, to prevent condensation on the culture medium.

- Most bacterial species are incubated for 16-18 hours in an incubator at 35 ± 2 °C.

- Some species require longer incubation or incubation in a CO2 incubator.

- For requirements of individual species see below.

## Reading the results

This section explains how to interpret and record the results from the different tests. Before reading the results:

- Inspect the plate after the appropriate incubation time:

  - Growth must be confluent over the entire agar plate.

  - If you see individual colonies, the suspension was not dense enough (< 0.5 McF) and the test must be repeated.

  - If the growth is excessive the test suspension was too dense and the test must be repeated.

Disk-diffusion:

- In case of translucent culture medium place the ruler against the bottom of the petri dish.

- In case of non-translucent media (blood containing media) remove the lid from the dish and hand measure the diameter of the inhibition zone at the surface of the culture medium (CAVE do not measure the haemolysis zone).

- When there are colonies inside the inhibition zone or when there is a “haze” of growth, the diameter is measured from the colonies nearest to the disk or from the start of the haze. Exceptions to this rule are co-trimoxazole, ampicillin and fosfomycin; for co-trimoxazole and ampicillin, the diameter is measured from the clear end of growth, and not from the start of the haze (haze must be ignored). In case of fosfomycin, colonies inside the inhibition zone can be dismissed (see picture from EUCAST below)

- Proteus species may swarm into areas of growth inhibition. The thin veil of swarming growth in an otherwise obvious zone of inhibition must be ignored.

- If a double inhibition zone is visible the inner inhibition zone is recorded

- When testing Staphylococcus aureus, colonies inside the zone of the cefoxitin disk are considered a sign of “hetero-resistance”, and the isolate is categorised as resistant (MRSA)

- When testing Enterococcus species for vancomycin, the zone edge must be sharp and there can be no colonies inside the zone to categorize as susceptible. If the zone edge is hazy and/or there are colonies inside the zone, the isolate is categorised as resistant (VRE) (see picture from EUCAST below).

Enterococcus species: only zone with sharp edge (A) indicates susceptibility!

B: colonies inside zone: resistant

C & D : hazy edges : resistant

E-test:

- Read the MIC value at the intersection of the border between the strip and the ellipse shaped inhibition zone

- For some bacteriostatic antibiotics (e.g. azithromycin), MIC is read at 80% inhibition. In this case, there will be a zone of clear growth and a “haze” of growth. The MIC value is read at the point of real growth; the haze must be dismissed. See picture below for an example.

- If no inhibition zone is present: record MIC as larger than the highest MIC value on the scale.

- If no intersection is discernible because the inhibition zone is too large: record MIC as smaller than the lowest MIC value on the scale.

- The presence of macro-colonies (individual, visible colonies), are always a sign of resistance. Macro-colonies should as such not be interpreted as ‘haze’.

- Refer to the E-test reading guide, which is available in the package inserts of the E-tests in use, or on the website: mybiomerieux.com

- For translating the MIC-value to a semi-quantitative assessment of susceptibility we refer to:

    - Performance Standards for antimicrobial susceptibility testing; Twenty-Eighth Informational Supplement. CLSI document M100-S28.

    - Methods for Antimicrobial dilution and disk susceptibility testing of infrequently isolated or fastidious bacteria; approved guideline—second Edition. CLSI document M45-Ed3.

Haze

# 4.  Guidelines per species or group of bacteria

An overview of the used culture media, procedures of inoculation, inoculum, incubation conditions and choice of antibiotics per species or group of bacteria, are reported below.

Two international guidelines are currently recommended for guidance in antibiotic susceptibility testing: CLSI (Clinical and laboratory standards institute) and EUCAST (European Committee on Antibiotic Susceptibility Testing). Both are freely available online, on http://www.eucast.org/fileadmin/src/media/PDFs/EUCAST_files/Breakpoint_tables/v_8.0_Breakpoint_Tables.pdf and http://em100.edaptivedocs.info/Login.aspx?_ga=2.51183081.1768928803.1518510871-2144262098.1485185887 (select “Guest Login”).

In the tables below, cut-off values for zone diameters and MIC are given according to the guidelines of both institutes.

Note that the selection of antibiotics presented here is extensive and designed for in-depth surveillance purposes. For patient care, consult national guidelines and the WHO Essential Medicines List to select the appropriate antibiotics to test. Consider selective reporting (see Appendix 4: Reporting of antibiotic susceptibility results).

### Salmonella species

- Medium	Müller-Hinton 2 agar

- Inoculum	Grown broth or suspension of 0,5 McF

(which corresponds to an OD of 660nm of 0.06-0.10)

- Incubation	Air incubator at 35 ± 2 °C, for 16-18 hours

|  | Antibiotic | Code | Content | Remarks | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | R | I | S | R | S |
| A | ampicillin1 | AMP10 | 10 µg |  | ≤ 13 | 14 – 16 | ≥ 17 | < 14 | ≥ 14 |
| B | ciprofloxacin | E-test2 |  |  | ≥ 1 | 0.125-0.5 | ≤ 0.064 | > 0.06 | ≤ 0.06 |
| Inv | azithromycin | E-test |  |  | ≥ 32 | - | ≤ 16 | > 16 | ≤ 16 |
|  | azithromycin | AZI15 | 15 µg |  | ≤ 12 | - | ≥ 13 | - | - |
| B | cotrimoxazole | SxT25 | 1.25/23.75 |  | ≤ 10 | 11 – 15 | ≥ 16 | < 11 | ≥ 14 |
| B | ceftriaxone³ | CTR30 | 30 µg |  | ≤ 19 | 20 – 22 | ≥ 23 | < 22 | ≥ 25 |
| C | ceftazidime³ | CAZ30 | 30 µg4 |  | ≤ 17 | 18 – 20 | ≥ 21 | < 19 | ≥ 22 |
| C | chloramphenicol | CLR30 | 30 µg |  | ≤ 12 | 13 – 17 | ≥ 18 | < 17 | ≥ 17 |
| Inv | pefloxacin5 | PEF5 | 5 µg |  | ≤ 23 | - | ≥ 24 | < 24 | ≥ 24 |
| - | nalidixic acid6 | NAL30 | 30 µg |  | ≤ 13 | 14 – 18 | ≥ 19 | - | - |
| A | gentamicin | GEN10 | 10 µg |  | ≤ 12 | 13-14 | ≥ 15 | < 14 | ≥ 17 |
| C | tetracyclines | TET30 | 30 µg |  | ≤ 11 | 12 - 14 | ≥ 15 | - | - |
| B | meropenem7 | MRP10 | 10 µg |  | ≤ 19 | 20 - 22 | ≥ 23 | < 16 | ≥ 22 |
|  | sulfonamides | SULFA250 | 250 µg |  | ≤ 12 | 13 - 16 | ≥ 17 | - | - |
|  | trimethoprim | TRIM5 | 5 µg |  | ≤ 11 | 11 - 15 | ≥ 16 | < 15 | ≥ 18 |

(1) Results of ampicillin testing are predictive for amoxicillin.

(2) Assess potential discrepancies between the disk and the MIC result, decreased Ciprofloxacin resistance (DCS) if MIC-value ≤ 0.064

(3) Reference lab: assess ESBL production in Salmonella species if ceftazidime < 22 or ceftriaxone < 23 (EUCAST) or if ceftazidime ≤ 22 mm or ceftriaxone ≤ 25 mm (CLSI). See 5.1 for detection of ESBL.

(4) EUCAST uses ceftazidime 10 µg disks

(5) results of pefloxacin testing are reported as ciprofloxacin (6) nalidixic acid testing: only at ITM

(7) If diameter < 28 mm: test for carbapenemase (see Appendix 3: Testing for carbapenemase production). Isolates with 25-27 mm only need to be investigated for carbapenemase-production if they are resistant to piperacillin-tazobactam and/or temocillin. Investigation for carbapenemases is always warranted if zone diameter of meropenem is <25 mm.

| Quality control | E. coli ATCC 25922 | S. aureus ATCC 29213 | K. pneumoniae ATCC 700603 |
| --- | --- | --- | --- |
|  | All NEO Sensitabs | E-test; azithromycin + ciprofloxacin | ESBL testing |

### Yersinia (note: quality control not updated)

- Medium 	Müller-Hinton 2 agar

- Inoculum 	Grown broth or suspension of 0,5 McF

- Incubation 	Air incubator at 35 ± 2 °C, for 16-18 hours

|  | Antibiotic | Code | Content |  | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Remarks | R | I | S | R | S |
| B | ciprofloxacin | CIPR5 | 5 µg |  | ≤ 21 | 22 - 25 | ≥ 26 | < 22 | ≥ 25 |
| B | cotrimoxazole | SxT25 | 1.25/23.75 |  | ≤ 10 | 11 – 15 | ≥ 16 | < 13 | ≥ 16 |
| C | tetracycline | TET30 | 30 µg |  | ≤ 11 | 12 – 14 | ≥ 15 | - | - |
| B | cefuroxime | CXM30 | 30 µg | parenteral | ≤ 14 | 15 – 17 | ≥ 18 | < 19 | ≥ 19 |

| Quality control | E. coli ATCC 25922 | S. aureus ATCC 25923 |
| --- | --- | --- |
|  | NEO sensitabs | NEO sensitabs |

  - Campylobacter (note: quality control not updated)

- Medium 	Müller-Hinton 2 agar with 5% sheep blood.

- Inoculum	Suspension of 0.5 McF

- Incubation	microaerophilic-atmosphere at 42, for 24 hours

|  | Antibiotic | Code | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | R | I | S | R | S |
| A | ciprofloxacin | E-test | ≥ 4 | 2 | ≤ 1 | > 0,5 | ≤ 0,5 |
| A | erythromycin1 | E-test | ≥ 32 | 16 | ≤ 8 | > 4/82 | ≤ 4/82 |
| B | doxycycline3 | E-test | ≥ 8 | 4 | ≤ 2 |  |  |
| A | tetracycline | E-test | ≥ 16 | 8 | ≤ 4 | > 2 | ≤ 2 |
| B | meropenem4 | E-test | - | - | - | ≥ 8 | ≤ 2 |
| B | amoxicillin/ clavulanic4 | E-test | - | - | - | ≥ 8 | ≤ 2 |

(1) Azithromycin can be determined using erythromycin

(2) C. jejuni : 4 µg/ml/ C. coli : 8 µg/ml

(3) Tetracycline can be used to determine susceptibility to doxycycline (EUCAST)

(4) MIC cut-offs given are general, not species-specific (no cut-offs given for these antibiotics in EUCAST)

| Quality control | S. aureus ATCC 29213 |
| --- | --- |
|  | E-test; azithromycin, ciprofloxacin, tetracycline, doxycycline, erythromycin |

### Enterobacteriaceae

- Medium 	Müller-Hinton 2 agar

- Inoculum 	Grown broth or suspension of 0,5 McF

- Incubation 	Incubator at 35 ± 2 °C, for 16-18 hours

  - For urine: add nitrofurantoin and fosfomycin to testing panel

|  | Antibiotic | Code | Content | Remarks | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | R | I | S | R | S |
| A | ampicillin1 | AMP10 | 10 µg |  | ≤ 13 | 14 – 16 | ≥ 17 | < 14 | ≥ 14 |
| B | amoxicillin/clav | AMC30 | 20/10 µg |  | ≤ 13 | 14 – 17 | ≥ 18 | < 19 | ≥ 19 |
| B | piperacillin/ tazobactam | PI+TZ | 100/10µg |  | ≤ 17 | 18 – 20 | ≥ 21 | < 17 | ≥ 20 |
| U | fosfomycin | FO200 | 200 µg | Only for urine | ≤ 12 | 13 – 15 | ≥16 | < 24 | ≥ 24 |
| U | nitrofurantoin | NI300 | 300 µg | Only for urine | ≤ 14 | 15 – 16 | ≥ 17 | < 11 | ≥ 11 |
| B | ciprofloxacin | CIPR5 | 5 µg |  | ≤ 21 | 22 - 25 | ≥ 26 | < 22 | ≥ 25 |
| B | co-trimoxazole | SxT | 1.25/23.75 |  | ≤ 10 | 11 – 15 | ≥ 16 | < 11 | ≥ 14 |
| C | chloramphenicol | CLR30 | 30 µg |  | ≤ 12 | 13 – 17 | ≥ 18 | < 17 | ≥ 17 |
|  | temocillin | TEMOC | 30 µg |  | ≤ 14 | 15 – 17 | ≥ 18 | - | - |
| A | gentamicin | GEN10 | 10 µg |  | ≤ 12 | 13 – 14 | ≥ 15 | < 14 | ≥ 17 |
| B | amikacin | AMI30 | 30 µg |  | ≤ 14 | 15 – 16 | ≥ 17 | < 15 | ≥ 18 |
| B | cefuroxime | CXM30 | 30 µg | parenteral | ≤ 14 | 15 – 17 | ≥ 18 | < 19 | ≥ 19 |
| B | ceftriaxon2,3 | CTR30 | 30 µg3 |  | ≤ 19 | 20 – 22 | ≥ 23 | < 22 | ≥ 25 |
| C4 | ceftazidime2,3 | CAZ30 | 30 µg3 |  | ≤ 17 | 18 – 20 | ≥ 21 | < 19 | ≥ 22 |
| B | meropenem5 | MRP10 | 10 µg |  | ≤ 19 | 20 – 22 | ≥ 23 | < 16 | ≥ 22 |
| C | tetracyclines | TET30 | 30 µg |  | ≤ 11 | 12 - 14 | ≥ 15 | - | - |
|  | sulfonamides | SULFA250 | 250 µg |  | ≤ 12 | 13 - 16 | ≥ 17 | - | - |
|  | trimethoprim | TRIM5 | 5 µg |  | ≤ 11 | 11 - 15 | ≥ 16 | < 15 | ≥ 18 |
|  | azithromycin6 | E-test |  | Shigella sonnei | ≥ 32 | - | ≤ 16 | - | - |
|  | azithromycin6 | E-test |  | Shigella spp. | ≥ 16 | - | ≤ 8 | - | - |
|  | azithromycin6 | AZI15 |  | Shigella spp. | ≥ 16 | - | ≤ 15 | - | - |

(1) Results of ampicillin are predictive for amoxicillin.

(2) Reference lab: assess ESBL production if ceftazidime < 22 or ceftriaxone < 23 (EUCAST) OR if ceftazidime ≤ 22 mm or ceftriaxone ≤ 25 mm (CLSI). See 5.1 for detection of ESBL

(3) EUCAST uses ceftazidime 10 µg disks

(4) Ceftazidime is not advised for treatment of Enterobacteriaceae, it is included in the panel as screening for ESBL

(5) If diameter of meropenem < 28 mm: test for carbapenemase (see Appendix 3: Testing for carbapenemase production). Isolates with 25-27 mm only need to be investigated for carbapenemase-production if they are resistant to piperacillin-tazobactam and/or temocillin (temocillin contributes more to the specificity). Investigation for carbapenemases is always warranted if zone diameter of meropenem is <25 mm.
(6) MIC-values and disc breakpoints only known for Shigella flexneri, MIC values are known for Shigella sonnei

| Quality control | E. coli ATCC 25922 | E. coli ATCC 35218 | S. aureus ATCC 29213 | K. pneumoniae ATCC 700603 |  |
| --- | --- | --- | --- | --- | --- |
|  | All NEO Sensitabs negative control ESBL | Beta-lactam and beta-lactamase inhibitor combinations | E-test; azithromycin + ciprofloxacin | ESBL testing |  |

### Pseudomonas aeruginosa

- Medium	Müller-Hinton 2 agar

- Inoculum	Grown broth or a suspension of 0,5 McF

- Incubation	Air incubator at 35 ± 2 °C, for 16-18 hours

|  | Antibiotic | Code | Content | Remarks | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | R | I | S | R | S |
| A | piperacillin/ tazobactam | PI+TZ | 100/10µg |  | ≤ 14 | 15 – 20 | ≥ 21 | < 18 | ≥ 18 |
| A | ceftazidime | CAZ30 | 30 µg |  | ≤ 14 | 15 – 17 | ≥ 18 | < 17 | ≥ 17 |
| B | ciprofloxacin | CIPR5 | 5 µg |  | ≤ 15 | 16 – 20 | ≥ 21 | < 26 | ≥ 26 |
| A | gentamicin | GEN10 | 10 µg |  | ≤ 12 | 13 – 14 | ≥ 15 | < 15 | ≥ 15 |
| B | amikacin | AMI30 | 30 µg |  | ≤ 14 | 15 – 16 | ≥ 17 | < 15 | ≥ 18 |
| B | meropenem | MRP10 | 10 µg |  | ≤ 15 | 16 – 18 | ≥ 19 | > 18 | ≤ 24 |

| Quality control | P. aeruginosa ATCC 27853 | E. coli ATCC 35218 |
| --- | --- | --- |
|  | All NEO Sensitabs | piperacillin-tazobactam |

### Acinetobacter

- Medium:  	Müller-Hinton 2 agar

- Inoculum: 	Grown broth or a suspension of 0,5 McF

- Incubation: 	Air incubator at 35 ± 2 °C, for 20-24 hours

|  | Antibiotic | Code | Content | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| B | piperacillin/ tazobactam | PI+TZ | 100/10µg | ≤ 17 | 18 – 20 | ≥ 21 | -1 | - |
| A | ceftazidime | CAZ30 | 30 µg | ≤ 14 | 15 – 17 | ≥ 18 | - | - |
| B | ceftriaxone | CTR30 | 30 µg | ≤ 13 | 14 – 20 | ≥ 21 | - | - |
| A | ciprofloxacin | CIPR5 | 5 µg | ≤ 15 | 16 – 20 | ≥ 21 | < 21 | ≥ 50 |
| A | gentamicin | GEN10 | 10 µg | ≤ 12 | 13 – 14 | ≥ 15 | < 17 | ≥ 17 |
| B | amikacin | AMI30 | 30 µg | ≤ 14 | 15 – 16 | ≥ 17 | < 17 | ≥ 19 |
| A | meropenem | MRP10 | 10 µg | ≤ 14 | 15 – 17 | ≥ 18 | < 15 | ≥ 21 |
| B | doxycycline | DOX30 | 30 µg | ≤ 9 | 10 – 12 | ≥ 13 | - | - |
| B | tetracycline | TET30 | 30 µg | ≤ 11 | 12 – 14 | ≥ 15 | - | - |
| B | co-trimoxazole | SXT | 1.25/23.75 | ≤ 10 | 11 – 15 | ≥ 16 | > 11 | ≤ 14 |

- EUCAST considers susceptibility result of penicillins unreliable as in most cases Acinetobacter spp. are resistant to penicillins.

| Quality control | E. coli ATCC 25922 | P. aeruginosa ATCC 27853 | E. coli ATCC 35218 |
| --- | --- | --- | --- |
|  | For tetracyclines and SXT | NEO sensitabs | Beta-lactam and beta-lactamase inhibitor combinations |

### Stenotrophomonas maltophilia

- Medium 	Müller-Hinton 2 agar

- Inoculum 	Grown broth or a suspension of 0,5 McF

- Incubation 	Air incubator at 35 ± 2 °C, for 20-24 hours

|  | Antibiotic | Code | Content | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| A | co-trimoxazole1 | SXT | 1.25/23.75 µg | ≤ 10 | 11 – 15 | ≥ 16 | < 16 | ≥ 16 |
| B | minocycline | MIN30 | 30 µg | ≤ 14 | 15-18 | ≥ 19 | - | - |

1: EUCAST: Ignore haze or fine growth within inhibition zone

Other antibiotics cannot be tested in a standardized way through agar diffusion methods.

| Quality control | E. coli ATCC 25922 |
| --- | --- |
|  | NEO sensitabs |

### Burkholderia cepacia (note: quality control not updated)

- Medium 	Müller-Hinton 2 agar

- Inoculum 	Grown broth or a suspension of 0,5 McF

- Incubation 	Air incubator at 35 ± 2 °C, for 20-24 hours

|  | Antibiotic | Code | Content | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| A | co-trimoxazole | SxT | 1.25/23.75 µg | ≤ 10 | 11-15 | ≥ 16 |  |  |
| C | chloramphenicol |  | E-test | ≥ 32 |  | ≤ 8 | - | - |
| B | ceftazidime | CAZ30 | 30 µg | ≤ 17 | 18-20 | ≥ 21 | - | - |
| A | meropenem | MRP10 | 10 µg | ≤ 15 | 16-19 | ≥ 20 | - | - |
| B | minocycline | MIN30 | 30 µg | ≤ 14 | 15-18 | ≥ 19 | - | - |

- Other antibiotics cannot be tested in a standardized way through disk diffusion methods.

- EUCAST: Burkholderia cepacia complex cannot be reliably tested through disk diffusion methods or E-testing. Furthermore, there is no evidence of a relationship between MIC values (as determined through microbroth dilution) and clinical response.

| Quality control | E. coli ATCC 25922 | P. aeruginosa ATCC 27853 | S. aureus ATCC 29213 |
| --- | --- | --- | --- |
|  | Neo-sensitabs | Neo-sensitabs | E-test |

Other non-fermenters than Pseudomonas aeruginosa, Acinetobacter spp., Stenotrophomonas maltophilia and Burkholderia cepacia cannot be tested reliably with disk diffusion methods.

### Staphylococcus

- Medium 	Müller-Hinton 2 agar

- Inoculum 	Suspension of 0,5 McF

- Incubation	Air incubator at 35 ± 2 °C, for 16-18 hours

  - In case of CNS, read vancomycin and cefoxitin after 24 hours of incubation

  - Clindamycin: Place the 2µg clindamycin disk at 15 to 26 mm from the 15 µg erythromycin disk (edge to edge) on MHA plate (D-zone test; see below)

|  | Antibiotic | Code | Content | Remarks | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | R | I | S | R | S |
| A | cefoxitin1,2 | CFO30 | 30 µg | S. aureus, S. lugdunensis3 | ≤ 21 |  | ≥ 22 | < 223 | ≥ 223 |
|  |  |  |  | CNS2, 3 | ≤ 24 |  | ≥ 25 | < 253 | ≥ 253 |
| A | penicillin4,5 | PEN10 | 10 µg | S. aureus4 | ≤ 28 |  | ≥ 29 | < 26 | ≥ 26 |
| A | co-trimoxazole | SXT | 1.25/23.75 |  | ≤ 10 | 11 – 15 | ≥ 16 | < 14 | ≥ 17 |
| C | ciprofloxacin | CIPR5 | 5 µg |  | ≤ 15 | 16 – 20 | ≥ 21 | < 20 | ≥ 20 |
| A | erythromycin | ERY15 | 15 µg |  | ≤ 13 | 14 – 22 | ≥ 23 | < 18 | ≥ 21 |
| A | clindamycin6 | CLI.2 | 2 µg |  | ≤ 14 | 15 – 20 | ≥ 21 | < 19 | ≥ 22 |
| B | linezolid7 | LINEZ | 30 µg |  | ≤ 20 |  | ≥ 21 | < 21 | ≥ 21 |
| B | doxycycline | DOX30 | 30 µg |  | ≤ 12 | 13 – 15 | ≥ 16 |  |  |
| U | nitrofurantoin9 | NI300 | 300 µg | For urine only | ≤ 14 | 15-16 | ≥ 17 | < 13 | ≥ 13 |
| B | vancomycin10,11 | E-test | Only for MRSA/MRSE | S. aureus | ≥ 16 | 4 – 8 | ≤ 2 | < 2 | ≥ 2 |
|  |  |  |  | CNS | ≥ 32 | 8 – 16 | ≤ 4 | < 4 | ≥ 4 |
| C | gentamicin12 | GEN10 | 10 µg | S. aureus | ≤ 12 | 13 – 14 | ≥ 15 | < 18 | ≥ 18 |
|  |  |  |  | CNS |  |  |  | < 22 | ≥ 22 |
| O | amikacin | AMI30 | 30 µg | S. aureus | - | - | - | < 16 | ≥ 18 |
|  |  |  |  | CNS | - | - | - | < 19 | ≥ 22 |

(1) Susceptibility for oxacillin is determined based on the results of cefoxitin. Results of cefoxitin are not reported.

(2) If cefoxitin is R, the isolate is methicillin (oxacillin) resistant. Oxacillin resistant staphylococci are resistant to all currently available beta-lactam antimicrobial agents, with exception of newer cephalosporins with anti-MRSA activity. If the isolate is cefoxitin resistant, report penicillin as resistant regardless of the result of penicillin testing

(3) EUCAST: cefoxitin cut-off diameter of 25 mm is only for S. epidermidis. Other CNS have the same cut-off as S. aureus and S. lugdunensis

(4) EUCAST: no currently available method can reliably detect production of penicillinase in CNS; report penicillin as resistant.

(5) EUCAST uses benzylpenicillin 1 unit instead of penicillin 10 units used by CLSI

(6) For clindamycin susceptibility assessment, read D-zone test (procedure: see 5.2 Detection of resistance mechanisms): if D-shaped flattened zone, report clindamycin resistant. Consider adding the following comment (EUCAST): "Clindamycin may still be used for short-term therapy of less serious skin and soft tissue infections as constitutive resistance is unlikely to develop during such therapy".

(7) When testing linezolid, disk diffusion zones should be examined using transmitted light. Organisms with resistant results by diffusion should be confirmed using a MIC method.

(8) EUCAST: norfloxacine can be used as screening for resistance to all fluoroquinolone antibiotics

(9) EUCAST uses nitrofurantoin 100 µg instead of 300 µg used by CLSI

(10) Always look for growth in the inhibition zone. If growth is present, report as resistant (see below).

(11) If vancomycin MIC ≥ 8 µg/ml for Staphylococcus aureus and ≥32 µg/ml for CNS: send strain to the reference laboratory
(12) CLSI: gentamicin is used only in combination with other agents that test susceptible

| Quality control | S. aureus ATCC 25923 | S. aureus ATCC 29213 |
| --- | --- | --- |
|  | Neo-sensitab disks | E-test vanco |

### Streptococcus pneumoniae

- Medium 	Müller-Hinton 2 agar with 5% sheep blood or MH-F agar (EUCAST)

- Inoculum 	Suspension of 0,5 McF

- Incubation 	5% CO2-incubator at 35 ± 2 °C, for 20-24 hours

Note: Do not measure the zone of inhibition of the haemolysis

|  | Antibiotic | Code | Content | Remarks | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | R | I | S | R | S |
| A | oxacillin1 | OXA.1 | 1 µg |  |  |  | ≥ 20 |  | ≥ 20 |
| A | co-trimoxazole | SXT | 1.25/23.75 |  | ≤ 15 | 16 – 18 | ≥ 19 | < 10 | ≥ 13 |
| A | erythromycin | ERY15 | 15 µg |  | ≤ 15 | 16 – 20 | ≥ 21 | < 19 | ≥ 22 |
| B | clindamycin | CLI.2 | 2 µg |  | ≤ 15 | 16 – 18 | ≥ 19 | < 19 | ≥ 19 |
|  |  |  | D-zone test: If D-shaped flattened zone, report clindamycin R |  |  |  |  |  |  |
| B | vancomycin | VAN30 | 30 µg |  |  |  | ≥ 17 | < 16 | ≥ 16 |
| B | tetracyclin | TET30 | 30 µg |  | ≤ 24 | 25 – 27 | ≥ 28 | < 22 | ≥ 25 |
| A | (benzyl)penicillin2 | E-test |  | Not meningitis | ≥ 8 | 4 | ≤ 2 | > 2 | ≤ 0.06 |
|  |  |  |  | Meningitis | ≥ 0.12 | - | ≤ 0.06 | > 0.06 | ≤ 0.06 |
| B | ceftriaxon2 | E-test |  | Not meningitis | ≥ 4 | 2 | ≤ 1 | - | - |
|  |  |  |  | Meningitis | ≥ 2 | 1 | ≤ 0.5 | > 2 | ≤ 0.5 |
| B | meropenem | E-test |  | Not meningitis | ≥ 1 | 0,5 | ≤ 0,25 | > 2 | ≤ 2 |
|  |  |  |  | Meningitis | - | - | - | > 1 | ≤ 0.25 |
| C | amoxicillin/ clavulanic acid | E-test |  | Not meningitis | ≥ 8/4 | 4/2 | ≤ 2/1 | - | - |

(1) oxacillin disk diffusion is used to screen for penicillin resistance; if diameter ≥ 20 mm, report penicillin as susceptible. If diameter < 20 mm, CLSI advises to determine MIC of penicillin and other beta-lactam antibiotics (do not report penicillin as resistant without performing MIC test). EUCAST advises the flow-chart shown below.

(2) If the sample is cerebrospinal fluid: interpret the results according to MIC ‘meningitis’.

For resistance to beta-lactam antibiotics, EUCAST proposes the following flow chart:

| Quality control | S. aureus ATCC 25923 | S. pneumoniae ATCC 49619 |
| --- | --- | --- |
|  | NEO Sensitabs | E-tests |

### Streptococcus pyogenes, agalactiae and other large β-hemolytic Streptococcus of gr. C & G

- Medium 	Müller-Hinton 2 agar with 5% sheep blood or MH-F (EUCAST)

- Inoculum 	Suspension of 0,5 McF

- Incubation 	5% CO2 incubator at 35 ± 2 °C, for 20-24 hours

|  | Antibiotic | Code | Content | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| A | penicillin1 | PEN10 | 10 units |  |  | ≥ 24 | < 18 | ≥ 18 |
| A | erythromycin2 | ERY15 | 15 µg | ≤ 15 | 16 – 20 | ≥ 21 | < 18 | ≥ 21 |
| A | clindamycin | CLI.2 | 2 µg | ≤ 15 | 16 – 18 | ≥ 19 | < 17 | ≥ 17 |
|  |  |  | D-test (see 5.2) | D-zone test (above): If D-shaped flattened zone, report clindamycin R3 |  |  |  |  |
| B | ceftriaxone1,4 | CTR30 | 30 µg |  |  | ≥ 24 | - | - |

(1) In case of resistance or intermediate susceptibility: repeat test. If resistance is confirmed, send to reference laboratory (penicillin resistance/intermediate susceptibility is rare or not yet reported). EUCAST uses benzylpenicillin 1 unit. Susceptibility to all beta-lactam antibiotics can be inferred from penicillin resistance

(2) Erythromycin can be used to determine susceptibility to azithromycin, clarithromycin and roxithromycin (EUCAST)
(3) If D-test positive, consider adding the following comment: "Clindamycin may still be used for short-term therapy of less serious skin and soft tissue infections as constitutive resistance is unlikely to develop during such therapy". (EUCAST)

(4) Only in case of blood or cerebrospinal fluid sample.

| Quality control | E. coli ATCC 25922 | S. pneumoniae ATCC 49619 |
| --- | --- | --- |
|  | NEO Sensitabs | E-tests |

### Streptococcus viridans (note: quality control not updated)

- Streptococcus anginosus group (S. anginosus, S. intermedius, S. constellatus)

- Small beta-hemolytic streptococcus of group A, C, F and G

- Streptococcus oralis, S. sanguinis, S. mitis)

- Streptococcus salivarius species, Streptococcus mutans species and Streptococcus bovis species

- Medium 	Müller-Hinton 2 agar with 5% sheep blood

- Inoculum 	Suspension of 0,5 McF

- Incubation 	5% CO2-incubator at 35 ± 2 °C, for 20-24 hours

|  | Antibiotic | Code | Content | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| A | penicillin1 | E-test |  | ≥ 4 | 0,25-2 | ≤ 0,12 | > 2 | ≤ 0.25 |
| C | erythromycin | ERY15 | 15 µg | ≤ 15 | 16 – 20 | ≥ 21 | - | - |
| C | clindamycin | CLI.2 | 2 µg | ≤ 15 | 16 – 18 | ≥ 19 | < 19 | ≥ 19 |
|  |  | D-test (see 5.2) |  | D-zone test : If D-shaped flattened zone, report clindamycin R |  |  |  |  |
| B | vancomycin | VAN30 | 30 µg |  |  | ≥ 17 | < 15 | ≥ 15 |
| B | ceftriaxone1 | CTR30 | 30 µg | ≤ 24 | 25 – 26 | ≥ 27 | < 27 | ≥ 27 |
| C | chloramphenicol | CLR30 | 30 µg | ≤ 17 | 18 – 20 | ≥ 21 |  |  |

(1) EUCAST: beta-lactam resistance can be screened with disk of penicillin 1 unit. A diameter of ≥ 18 mm indicates susceptibility to all beta-lactam antibiotics; no E-test needed. Diameter of < 18 mm: test individual beta-lactam agents (penicillin with E-test).

| Quality control | E. coli ATCC 25922 | S. pneumoniae ATCC 49619 |
| --- | --- | --- |
|  | Neo-sensitabs | E-tests and Neo-sensitabs |

### Enterococcus (note: quality control not updated)

- Medium 	Müller-Hinton 2 agar

- Inoculum 	grown broth or a suspension of 0.5 McF

- Incubation 	Incubator at 35 ± 2 °C, for 16-18 hours (24 hours for vancomycin!)

  - In case of urine sample add: levofloxacin, tetracycline, nitrofurantoin and fosfomycin.

|  | Antibiotic | Code | Content | Remarks | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | R | I | S | R | S |
| A | ampicillin1 | AMP10 | 10 µg |  | ≤ 16 |  | ≥ 17 | < 8 | ≥ 10 |
| U | tetracyclin | TET30 | 30 µg | Only for urine | ≤ 14 | 15 – 18 | ≥ 19 | - | - |
| U | nitrofurantoin1 | NI300 | 300 µg | Only for urine | ≤ 14 | 15 – 16 | ≥ 17 | < 15 | ≥ 15 |
| U | fosfomycin | FO200 | 200 µg | Only for urine | ≤ 12 | 13 – 15 | ≥ 16 | - | - |
| U | ciprofloxacin2 | CIPR5 | 5 µg | Only for urine | ≤ 15 | 16-20 | ≥ 21 | < 15 | ≥ 15 |
| - | gentamicin3 | GEN30 | 30 µg | Only for blood (high-level resistance) | - | - | - | < 8 | ≥ 8 |
| B | vancomycin4 | VAN30 | 30 µg |  | ≤ 14 | 15 – 16 | ≥ 17 | < 12 | ≥ 12 |
|  |  | E-test |  |  | ≥ 32 | 8 – 16 | ≤ 4 | > 4 | ≤ 4 |
| B | linezolid1 | LINEZ | 30 µg |  | ≤ 20 | 21 – 22 | ≥ 23 | < 19 | ≥ 19 |

(1) EUCAST uses different disk contents: Ampicillin 2 µg, Nitrofurantoin 100 µg and Linezolid 10 µg

(2) The norfloxacin disk diffusion test can be used to screen for fluoroquinolone resistance (EUCAST). In this case, a diameter of ≥ 12 mm indicates susceptibility

(3) Enterococcus species are intrinsically resistant to aminoglycosides, so this should never be given in monotherapy. There may however be synergistic action with beta-lactam antibiotics. To test for high-level aminoglycoside resistance, use gentamycin 30 µg disks.

(4) Growth in the inhibition zone or hazy edges of the zone always implies resistance

If diameter < 17mm, perform an additional E-test with vancomycin

| Quality control | S. aureus ATCC 25923 | S. aureus ATCC 29213 |
| --- | --- | --- |
|  | NEO sensitabs | E-test; vancomycin |

### Haemophilus influenzae and Haemophilus parainfluenzae

- Medium 	Haemophilus Test medium (HTM) or Müller-Hinton fastidious (MH-F) agar

- Inoculum 	Suspension of 0,5 McF

- Incubation 	CO2 Incubator at 35 ± 2 °C, for 16-18 hours

|  | Antibiotic | Code | Content | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| A | ampicillin1 | AMP10 | 10 µg | ≤ 18 | 19 – 21 | ≥ 22 | < 16 | ≥ 16 |
| C | amoxicillin/clav | AMC30 | 20/10 µg | ≤ 19 | - | ≥ 20 | < 15 | ≥ 15 |
| C | cotrimoxazole | SxT | 1.25/23.75 | ≤ 10 | 11 – 15 | ≥ 16 | < 20 | ≥ 23 |
| B | ceftriaxone2 | CTR30 | 30 µg |  |  | ≥ 26 | < 32 | ≥ 32 |
| B | meropenem2,3 | MRP10 | 10 µg |  |  | ≥ 20 | < 20 | ≥ 20 |
|  |  |  | E-test meningitis |  |  |  | > 1 | ≤ 0.25 |
| C | chloramphenicol | CLR30 | 30 µg | ≤ 25 | 26-28 | ≥ 29 | < 28 | ≥ 28 |

(1) Always test for the presence of a beta-lactamase (with nitrocefine). When positive, report ampicillin R.

EUCAST uses ampicillin 2 µg.

(2) In case of reduced susceptibility: repeat test

(3) EUCAST advises MIC for meropenem (meningitis) and clarithromycin

| Quality control | H. influenzae ATCC 49247 | E. coli ATCC 35218 |
| --- | --- | --- |
|  | Neo-sensitabs | Beta-lactam and beta-lactamase inhibitor combinations |

### Neisseria meningitidis

- Medium 	 MHA with 5% sheep blood or MH-F agar (EUCAST)

- Inoculum 	Suspension of 0,5 McF

- Incubation 	CO2 Incubator at 35 ± 2 °C, for 20-24 hours

  - NOTE; place a maximum of 2 disks per plate

  - Note; be careful while manipulation of Neisseria meningitidis, handle only in a safety cabinet in BSL-2 or BSL-3 labs.

|  | Antibiotic | Code | Content | CLSI |  |  | EUCAST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| C | penicillin1 |  | E-test | ≥ 0.5 | 0.12-0.25 | ≤ 0.06 | > 0.25 | ≤ 0,06 |
| C | cotrimoxazole3 | SxT | 1.25/23.75 | ≤ 25 | 26 – 29 | ≥ 30 | - | - |
| C | ceftriaxon,3,4 | CTR30 | 30 µg |  |  | ≥ 34 | > 0.125 | ≤ 0.125 |
| C | meropenem3,4 | MRP10 | 10 µg |  |  | ≥ 30 |  |  |
|  |  |  | E-test meningitis |  |  |  | > 0.25 | ≤ 0.25 |
| C | ciprofloxacin2 | CIPR5 | 5 µg | ≤ 32 | 33-34 | ≥ 35 | - | - |
|  |  |  | E-test | ≥ 0.03 | 0.06 | ≤ 0.12 | > 0.03 | ≤ 0.03 |
| C | chloramphenicol2, 3 | CLR30 | 30 µg | ≤ 19 | 20-25 | ≥ 26 |  |  |
|  |  |  | E-test |  |  |  | > 2 | ≤ 2 |

(1) EUCAST uses benzylpenicillin

(2) Only for assessing appropriateness of prophylaxis, not for therapy of patients

(3) EUCAST advises MIC

(4) Non-susceptible isolates are rare or not yet reported. The identification and antimicrobial susceptibility test result on any such isolate must be confirmed and the isolate sent to a reference laboratory.

| Quality control | E. coli ATCC 25922 | S. pneumoniae ATCC 49619 |
| --- | --- | --- |
|  | NEO sensitabs | E-test penicillin |

### Neisseria gonorrhoeae (note: quality control not updated)

- Medium 	DST (if possible: GC agar base and 1% defined growth supplement)

- Inoculum 	Suspension of 0,5 McF

- Incubation 	CO2-incubator at 36 ± 1 °C for 20-24 hours

  - Maximum 4 disks on a 10mm plate

|  | Antibiotic | Code | Content | CLSI |  |  | EU-cast |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| A | ceftriaxone | CTR30 | 30 µg |  |  | ≥ 35 |  |  |
|  |  |  | E-test |  |  |  | > 0.125 | ≤ 0.125 |
| A | cefixime | CFM.5 | 5 µg |  |  | ≥ 31 |  |  |
|  |  |  | E-test |  |  |  | > 0.125 | ≤ 0.125 |
| A | tetracycline | TET30 | 30 µg | ≤ 30 | 31-37 | ≥ 38 |  |  |
|  |  |  | E-test |  |  |  | > 1 | ≤ 0.5 |
| A | ciprofloxacin | CIPR5 | 5 µg | ≤ 27 | 28-40 | ≥ 41 |  |  |
|  |  |  | E-test |  |  |  | > 0.06 | ≤ 0.03 |
| O | penicillin1 | PEN10 | 10 units | ≤ 26 | 27 – 46 | ≥ 47 |  |  |
|  |  |  | E-test |  |  |  | > 1 | ≤ 0.06 |
| O | spectinomycin | SPECT | 100 µg | ≤ 14 | 15 – 17 | ≥ 18 |  |  |
|  |  |  | E-test |  |  |  | > 64 | ≤ 64 |

- A positive beta-lactamase test predicts resistance to penicillin, ampicillin and amoxicillin. EUCAST uses benzylpenicillin.

- EUCAST: a MIC-method should be used. Laboratories with few isolates are encouraged to send the isolates to reference laboratories for AST

| Quality control | E. coli ATCC 25922 |
| --- | --- |
|  | NEO sensitabs |

### Vibrio cholerae

- Medium 	Müller-Hinton 2 agar

- Inoculum 	Grown broth or a suspension of 0,5 McF

- Incubation 	Incubator at 35 °C, for 16-18 hours

|  | Antibiotic | Code | Content | CLSI |  |  | EU-cast |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | I | S | R | S |
| A | ampicillin1 | AMP10 | 10 µg | ≤ 13 | 14-16 | ≥ 17 | - | - |
| A | azithromycin1 | E-test |  |  |  | ≤ 2 | - | - |
| A | tetracyclin1,2,3 | TET30 | 30 µg | ≤ 11 | 12-14 | ≥ 15 | - | - |
| A | cotrimoxazole1 | SxT | 1.25/23.75 | ≤ 10 | 11 – 15 | ≥ 16 | - | - |
| A | chloramphenicol1 | CLR30 | 30 µg | ≤ 12 | 13 – 17 | ≥ 18 | - | - |
| A | ciprofloxacin2 | CIPR5 | 5 µg | ≤ 15 | 16-20 | ≥ 21 | - | - |
| A | cefotaxime2 | CTX30 | 30 µg | ≤ 22 | 23-25 | ≥ 26 | - | - |
| A | ceftazidime2 | CAZ30 | 30 µg | ≤ 17 | 18-20 | ≥ 21 | - | - |
| A | sulfonamides1 (V. cholerae only) |  | 250-300 µg | ≤ 12 | 13-16 | ≥ 17 |  |  |
| B | meropenem | MRP10 | 10 µg | ≤ 19 | 20-22 | ≥ 23 | - | - |
| B | gentamicin | GEN10 | 10 µg | ≤ 12 | 13-14 | ≥ 15 | - | - |

(1) Agents considered for primary testing of V. cholerae

(2) Agents considered for primary testing of other Vibrio spp.

(3) Tetracycline predicts doxycycline

| Quality control | E. coli ATCC 25922 | E. coli ASTT 35218 | P. aeruginosa ATCC 27853 |
| --- | --- | --- | --- |
|  | Neo-sensitabs | Beta-lactam and beta-lactamase inhibitor combinations | Neo-sensitabs |

# 5.  Detection of resistance mechanisms

## 5.1 ESBL testing using the combination disk test for K. pneumoniae, K. oxytoca, E. coli, P. mirabilis and Salmonella spp.

- If there is a difference of ≥ 5mm in 1 of the 2 combinations, the strain tested is an ESBL producer (see picture below).

- If a “champagne button” or “phantom zone” is detected on the plate, the isolate tested is an ESBL producer (see picture below).

- No changes in measured susceptibility should be made in response to the ESBL result. The presence or absence of ESBL has no clinical implications and is only important for infection control and public health purposes.

| Antibiotic | Code | Content |  |
| --- | --- | --- | --- |
|  |  | CLSI | EUCAST |
| ceftazidime | CAZ30 | 30 µg | 10 µg |
| ceftazidime + clavulanic acid1 | CAZ+C | 30/10 µg |  |
| cefotaxime | CTX30 | 30 µg | 5 µg |
| cefotaxime + clavulanic acid1 | CTX+C | 30/10 µg |  |

“Champagne button”

| Quality control | E. coli ATCC 25922 | K. pneumoniae ATCC 700603 |
| --- | --- | --- |
|  | ESBL negative control | ESBL positive control |

## 5.2 Inducible clindamycin resistance testing using the D-test

- Place the 2µg clindamycin disk at 15 to 26 mm (12 mm for Streptococcus spp.) from the 15µg erythromycin disk (edge to edge) on MHA plate

- Incubate for 16-18 hours at 35°C +/- 2°C

- The D-test is positive when you see a flattening of the zone of inhibition adjacent to the erythromycin disk (looks like letter D, see picture) = inducible clindamycin resistance

- When there is a hazy growth within the zone of inhibition around clindamycin, even without D-zone = clindamycin resistance

| Quality control | S. aureus ATCC BAA-976 | S. aureus ATCC BAA-977 |
| --- | --- | --- |
|  | D-zone negative | D-zone positive |

# Quality control

## First line quality control

- Appropriate quality control strains are mentioned in this SOP for all species.

- In case of batch testing always include a quality control at each tested batch and for each new LOT (plates as well as antibiotics) that is opened.

- For more specifics on quality control, see Appendix 1

## Second line quality control

- Not applicable

# Registration and processing results

- Susceptibility to antibiotics is reported in code of interpretation:

  - S	Susceptible

  - I	Intermediate

  - R 	Resistant

- In disk diffusion the diameter of the inhibition zone is registered in mm as well as the interpretation. Reporting to the clinician is done only by code of interpretation (S/I/R).

- MIC values are reported as well as the interpretation.

- For more information on reporting and expert rules, see Appendix 4: reporting of antibiotic susceptibility results

# Method verification literature

- Performance Standards for antimicrobial susceptibility testing; Twenty-Eighth Informational Supplement. CLSI document M100-S28. CLSI, 950 West Valley Road, Suite 2500, Wayne, Pennsylvania 19087 USA, 2018)

- Methods for Antimicrobial dilution and disk susceptibility testing of infrequently isolated or fastidious bacteria; approved guideline—Second Edition CLSI document M45-Ed3 CLSI, 940 West Valley Road, Suite 2500, Wayne, Pennsylvania 19087 USA, 2015.)

- Performance Standards for Antimicrobial Disk Susceptibility Tests; Approved Standard – Eleventh Edition M02-A12 (ISBN 1-56238-781-2) CLSI, 940 West Valley Road, Suite 2500, Wayne, Pennsylvania 19087 USA, 2015

- EUCAST Clinical breakpoint version 8.0 http://www.eucast.org/fileadmin/src/media/PDFs/EUCAST_files/Breakpoint_tables/v_8.0_Breakpoint_Tables.pdf

- User's Guide EUCAST and CLSI potency Neo Sensitabs: 2013

- Luber P, Wagner J, Hahn H, Bartelt E. Antimicrobial resistance in Campylobacter jejuni and Campylobacter coli strains isolated in 1991 and 2001-2002 from poultry and humans in Berlin, Germany. Antimicrob Agents Chemother. 2003 Dec;47(12):3825-30 http://aac.asm.org/cgi/reprint/47/12/3825

- The EUCAST guideline on detection of resistance mechanisms v2.0   http://www.eucast.org/fileadmin/src/media/PDFs/EUCAST_files/Resistance_mechanisms/EUCAST_detection_of_resistance_mechanisms_170711.pdf

- EUCAST guidance document: Burkholderia cepacia complex http://www.eucast.org/fileadmin/src/media/PDFs/EUCAST_files/General_documents/BCC_susceptibility_testing_130719.pdf

# Records and archives

- Order list materials for antibiotic susceptibility testing

- Internal quality control antibiotic susceptibility testing bacteriology

- E-test Reading Guide bioMérieux. AB Biodisk 2008. http://www.ilexmedical.com/files/ETEST_RG.pdf

- E-test Antimicrobial Susceptibility Testing. Package Insert. 9302553C - en - 2012/01. Available at http://www.ilexmedical.com/files/E-test-Package-Insert/AST_WW.pdf

- User's Guide EUCAST and CLSI potency Neo Sensitabs: 2013

- User guide McFarland Standard. December 2002. Available at http://www.dalynn.com/dyn/ck_assets/files/tech/TM53.pdf

| Appendices & Forms for completion |  |
| --- | --- |
| Number | Title |
| 1 | AST panels for reference laboratory and ITM |
| 2 | Quality control for AST |
| 3 | Testing for carbapenemase production |
| 4 | Reporting of antibiotic susceptibility results |

| Revision |  |
| --- | --- |
| Changes with respect to the previous version | Initial version |
| TLM_PRO_099_v02 | 02/02/2015; AP_ update according to CLSI guideline M100_S25 and EU-cast breakpoint table V5, 2015 |
| TLM_PRO_099_v05 | Update according to CLSI M100 S26; QC reviewed and updated |
| TLM_PRO_099_v06 | Comment added about 80% inhibition MIC reading with E-test added (1.2: MIC testing); Comment added about fosfomycin colonies in disk diffusion (1.2: Disk diffusion); Comment added about sampling different colonies for AST added (3.1: Preparing the inoculum); Carbapenemase screening in Enterobacteriaceae adapted (3.6.4: Enterobacteriaceae) |
| TLM_PRO_099_v07 | Updated according to CLSI M100 – S28 and EUCAST v8.0 |
| TLM_PRO_099_v08 | Disk abbreviations updated according to ROSCO Neo-Sensitabs Addition of penicillin testing for Entercoccus sp. and of amoxi/clav for Haemophilus sp. 6/09/2018 |
| TLM_PRO_099_v09 | Levofloxacin removed from all antibiograms (2nd line therapy for TB) Clarithromycin removed from Haemophilus and Neisseria antibiograms (2nd line therapy for TB) Penicillin removed for Enterococcus spp Tobramycin removed from all antibiograms Norfloxacin removed from Staphylococcus and Pseudomonas aeruginosa antibiograms Ampicillin-sulbactam removed from Acinetobacter antibiogram Vancomycin testing for Staphylococcus: only in case of oxacillin resistance Vancomycin disk testing removed for beta-hemolytic streptococci Azithromycin disk added for Salmonella spp Tetracycline, sulphonamide and trimethoprim added for Enterobacteriaceae Sulphonamide and trimethoprim added for Salmonella Shigella added as part of Enterobacteriaceae, instead of combination with Salmonella Azithromycin MIC value specified for Shigella species Disk diameters updated to CLSI M100 – S29 Disk diameters updated to EUCAST v9.0 |

| Name and function | Date | Signature |
| --- | --- | --- |
| Author |  |  |
| Sien Ombelet | 14/05/2019 |  |
| Reviewed by |  |  |
| Jan Jacobs | 14/05/2019 | See mail 14/05/2019 |
| Approved by |  |  |
| Jan Jacobs | 14/05/2019 | See mail 14/05/2019 |