| Logo partner institute | Department: Clinical Sciences | Unit: Tropical Laboratory Medicine |
| --- | --- | --- |
|  | Title: Biochemical Identification of Enterobacteriaceae |  |
|  | Project/study: N.A. |  |

# Scope and application

This document provides a basic method for biochemical identification of Enterobacteriaceae. Make sure the following characteristics apply to the isolate: - Gram negative rods - Glucose fermenting - Oxidase negative Perform any of these tests again if the SOP result is indeterminate.

# Responsibilities

| Function | Activities |
| --- | --- |
| Laboratory technician TLM member | - Complies with this procedure |

# Procedures

3.1 Materials

- Perform the tests below on fresh and pure cultures

- The following test results are always needed:

  - Gram-stain                                                  (see procedure TLM_PRO_0104)

  - Oxidase				(see procedure TLM_PRO_0055)

  - Kligler Iron Agar:			(see procedure TLM_PRO_0056)

    - Glucose-fermentation

    - Lactose-fermentation

    - H2S-production

    - Gas-production

  - Citrate					(see procedure TLM_PRO_0057)

  - Motility				(see procedure TLM_PRO_0058)

  - Urease					(see procedure TLM_PRO_0057)

  - Indole					(see procedure TLM_PRO_0058)

Additional tests that may be performed depending on the table used

  - PGUA 					(see procedure TLM_PRO_0057)

  - ONPG					(see procedure TLM_PRO_0057)

  - Lysine decarboxylase (LDC)		(see procedure TLM_PRO_0058)

  - Ornithine decarboxylase (ODC)		(see procedure TLM_PRO_0057)

  - Arginine dihydrolase (ADH)		(see procedure TLM_PRO_0057)

  - D-Mannitol (see API E)

  - D-Sorbitol (see API E)

  - Polyvalent Salmonella spp. antiserum	(see procedure TLM_PRO_0059)

- Record all test reactions as

  - (+) for a positive reaction  or

  - (-) for a negative reaction

(see respective table in addendum for correct interpretation)
Note: results of the KIA agar are written with abbreviations, e.g. “G” for “Glucose-fermenting”, see list of abbreviations at the end of this SOP.

3.2 Method

3.2.1. DAY 1

Start identification with inoculation of the following tests:

- KIA

- Citrate

- MIU

Make sure to  (i) use one single colony to inoculate the three tubes (ii) screw the tube cap open to allow air flow during incubation (iii) inoculate enough inoculum into the Citrate agar (iv) adhere to incubation timing.

3.2.2. DAY 2

1. Read and note down the test results of the KIA, MIU and Citrate tubes (see results in Addendum)

Make sure to read the result of mobility and Urease first before to add Kovac’s reagents to the MIU medium.

2. Inoculate the following DIATABS with colonies grown on the KIA Agar:

- Urease

- LDC

- ONPG

Make sure to add three drops of paraffin to the LDC diatab before incubation. 
Note: the speed by which there is a change of colour (Urease pos.) may have a diagnostic interest for the Urease DIATAB. 
- Positive reaction within 30 minutes: most cases: Morganella morganii

- Positive reaction within 4 hours: usually: Proteus spp., M. morganii, Enterobacter gergoviae.

- Positive reaction overnight: Klebsiella pneumoniae/Klebsiella oxytoca and Yersinia spp.

3.2.3 DAY 3

1. Read and note down the test results

2. Use one of the following tables to identify the isolate, according to the results obtained:

Table 1: H2S positive (including Moustache) - see General Comment 4.6.

Table 2: H2S negative and Motility positive

Table 3: H2S negative and Motility negative

The tables contain the percentages in which the test is positive for that species.

Table 1 : H2S + isolates

Table to differentiate between isolates that are H2S +

Step 1:  If Lactose negative, swarming on blood agar and Urease positive: identify as Proteus sp. 
Differentiate  between P. mirabilis and P. vulgaris with the Indole result.

Step 2:  Read Indole result:

If Indole is positive: identify as Edwardsiella tarda (very rare, see general comment 3).

If Indole is negative, perform step 3.

Step 3: Read ONPG and LDC results:

- If ONPG is negative and LDC is positive, identify as Salmonella sp.

Read Citrate result to differentiate between Salmonella Typhi (negative) and Non-Typhoid (positive) serotypes.

Confirm this result with testing of ODC (negative for Salmonella Typhi) and perform slide agglutination of the Salmonella isolate to differentiate between the various serotypes.

Note: Salmonella sp. has a clearly edged H2S (“shoulder” – see comment 6).

Note: Salmonella Typhi has a trace of H2S on Kligler Iron Agar (“moustache”- see General Comment 4.6).

- If ONPG is positive and LDC is negative, identify as Citrobacter freundii.

Note: Citrobacter freundii has less defined (“dirty” – see comment 6) H2S production compared to Salmonella sp.

|  | Lactose | H2S | gas | Citrate | Motility | Urease | Indole | LDC | ODC | ADH | ONPG | PGUA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proteus mirabilis | - (2%) | + (98%) | + (96%) | +/- (65%) | + (95%) | + (98%) | - (2%) | - (0%) | + (99%) | - (0%) | - (0%) | - (0%) |
| Proteus vulgaris | - (2%) | + (95%) | + (85%) | - (15%) | + (95%) | + (95%) | + (98%) | - (0%) | - (0%) | - (0%) | -(1%) | - (0%) |
| Edwardsiella tarda | - (0%) | + (100%) | + (100%) | - (1%) | + (98%) | - (0%) | + (99%) | + (100%) | +(100%) | - (0%) | - (0%) | - (0%) |
| Salmonella Typhi | - (1%) | Moustache | – (0%) | - (0%) | + (97%) | - (0%) | - (0%) | + (98%) | - (0%) | - (3%) | - (0%) | - (0%) |
| Salmonella Non-Typhi | - (1%) | + (100%)* | + (96%) | + (95%) | + (95%) | - (1%) | - (1%) | + (98%) | + (97%) | + (70%) | - (2%) | - (0%) |
| Citrobacter freundii | + (78%) | + (78%)** | + (89%) | + (78%) | + (89%) | +/- (44%) | -(33%) | -(0%) | -0(%) | +(67%) | +(89%) | - (0%) |

*Moustache possible when isolates originate from DR Congo

**Moustache also possible for Citrobacter freundii

Table 2: H2S - and Motility +

Table to differentiate between different isolates that are: H2S- and Motility +

Step 1: Read Indole result

If Indole is negative, read ONPG result:

- If ONPG is negative,  identify as Salmonella sp. 
Use LDC to differentiate between Salmonella Choleraesuis (LDC positive) and Salmonella Paratyphi A (LDC negative) but confirm with table 2.  
Perform slide agglutination of the Salmonella isolate to differentiate between the various serotypes.

- If ONPG is positive, continue as follows:

  - If Citrate is negative and LDC is positive, identify as Hafnia alveii

  - If Citrate is positive, identify as Enterobacter sp.

Differentiate between the Enterobacter species with the use of table 2.

Use DNase if differentiation between Enterobacter sp. (DNase negative) and Serratia sp. (DNase positive) is needed.

If Indole is positive:

- Perform PGUA and ODC; if PGUA is positive identify as Escherichia coli

- If PGUA is negative, read ONPG and ODC

  - If ONPG and ODC are positive, identify as Citrobacter diversus.

  - If ONPG is negative, but ODC is positive, identify as Morganella morganni .

  - If ONPG and ODC are negative, identify as Providencia sp.

Refer to table 2 when further differentiation between the Providencia species is needed.

Table 2: H2S - and Motility +

|  | Lactose | H2S | gas | Citrate | Motility | Urease | Indole | LDC | ODC | ADH | ONPG | PGUA | D-sorbitol | D-mannitol |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salmonella Choleraesuis | - (0%) | - (25%) | + (95%) | - (25%) | + (95%) | - (0%) | - (0%) | + (95%) | + (100%) | +/- (55%) | - (0%) | - (0%) | + (90%) | + (98%) |
| Salmonella Paratyphi A | - (0%) | - (10%) | + (99%) | - (0%) | + (95%) | - (0%) | - (0%) | - (0%) | + (95%) | -(15%) | - (0%) | - (0%) | + (95%) | + (100%) |
| Serratia marcescens** | - (2%) | - (0%) | +/- (55%) | + (98%) | + (97%) | - (15%) | - (1%) | + (99%) | + (99%) | - (0%) | + (95%) | - (0%) | + (99%) | + (99%) |
| Hafnia alvei | - (5%) | - (0%) | + (85%) | - (10%) | + (85%) | - (4%) | - (0%) | + (100%) | + (98%) | -(6%) | + (90%) | - (0%) | - (0%) | + (99%) |
| Enterobacter aerogenes | + (95%) | - (0%) | + (100%) | + (95%) | + (97%) | - (2%) | - (0%) | + (98%) | + (98%) | - (0%) | + (100%) | - (0%) | + (100%) | + (100%) |
| Enterobacter cloacae | + (93%) | - (0%) | + (100%) | + (100%) | + (95%) | +/- (65%) | - (0%) | - (0%) | + (96%) | + (97%) | + (99%) | - (0%) | + (95%) | + (100%) |
| Enterobacter agglomerans* | +/- (40%) | - (0%) | - (20%) | +/- (50%) | + (85%) | - (20%) | - (20%) | - (0%) | - (0%) | - (0%) | + (90%) | - (0%) | +/- (30%) | + (100%) |
| Enterobacter gergoviae | +/- (55%) | - (0%) | + (98%) | + (99%) | + (90%) | + (93%) | - (0%) | + (90%) | + (100%) | - (0%) | + (97%) | - (0%) | - (0%) | + (99%) |
| Enterobacter sakazakii* | + (99%) | - (0%) | + (98%) | + (99%) | + (96%) | - (1%) | - (11%) | - (0%) | + (91%) | + (99%) | + (100%) | - (0%) | - (0%) | + (100%) |
| Escherichia coli | + (95%) | - (1%) | + (95%) | - (1%) | + (95%) | - (1%) | + (98%) | + (90%) | +/- (65%) | - (17%) | + (95%) | + (100%) | + (94%) | + (98%) |
| Citrobacter diversus | +/- (50%) | - (0%) | + (99%) | + (99%) | + (95%) | +/- (44%) | + (99%) | - (0%) | + (99%) | + (80%) | + (99%) | - (0%) | + (99%) | + (99%) |
| Morganella morganii | - (1%) | - (20%) | + (90%) | - (0%) | + (95%) | + (95%) | + (95%) | - (1%) | + (95%) | - (0%) | - (10%) | - (0%) | - (0%) | - (0%) |
| Providencia rettgeri | - (5%) | - (0%) | - (10%) | + (95%) | + (94%) | + (98%) | + (99%) | - (0%) | - (0%) | - (0%) | - (5%) | - (0%) | - (0%) | + (100%) |
| Providencia stuartii | - (2%) | - (0%) | - (0%) | + (93%) | + (85%) | - (30%) | + (98%) | - (0%) | - (0%) | - (0%) | - (10%) | - (0%) | - (0%) | - (10%) |
| Providencia alcalifaciens | - (0%) | - (0%) | + (85%) | + (98%) | + (96%) | - (0%) | + (99%) | - (0%) | - (1%) | - (0%) | - (1%) | - (0%) | - (0%) | - (2%) |

This table reports only the most common species of Enterobacter and Citrobacter.

* Enterobacter sakazakii and Enterobacter agglomerans have a yellow pigment in  respectively 98% and 75%.

** If DNAse positive and red pigment: identify Serratia rubidaea.

Table 3: H2S- and Motility -

Table to differentiate between different isolates that are H2S - and Motility -

- If Citrate +,Urease +, LDC + and ODC -  (Diatab): identify as Klebsiella sp.

Read Indole result:

- If Indole is negative: identify as Klebsiella pneumoniae

- If Indole is positive: identify as Klebsiella oxytoca

Notes:

Be careful to not confuse Klebsiella spp. with Enterobacter spp. Verify all biochemical reactions mentioned in the table below.

  - If the biochemical reactions do not fit with Klebsiella sp., repeat the motility test (the majority of Enterobacter spp. are motile).

  - If after repetition of the motility test, the isolate is still not motile; and ODC + and LDC -: Identify as Enterobacter asburiae (part of Enterobacter cloacae complex)

- If Lactose -, Gas -, Citrate - : think about Shigella sp. or Escherichia paracoli (see table 3 and general comment n°4).

|  | Lactose | H2S | gas | Citrate | Motility | Urease | Indole | LDC | ODC | ADH | ONPG | PGUA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Klebsiella oxytoca* | + (100%) | - (0%) | + (97%) | + (95%) | - (0%) | + (95%) | + (100%) | + (99%) | - (0%) | - (0%) | + (100%) | - (0%) |
| Klebsiella pneumoniae* | + (98%) | - (0%) | + (97%) | + (98%) | - (0%) | + (95%) | - (0%) | + (98%) | - (0%) | - (0%) | + (99%) | - (0%) |
| Enterobacter asburiae | +/- (75%) | - (0%) | + (95%) | + (100%) | - (0%) | +/- (60%) | - (0%) | - (0%) | + (95%) | -(21%) | + (100%) | - (0%) |
| Klebsiella ozaenae* | +/- (30%) | - (0%) | +/- (50%) | - (30%) | - (0%) | - (10%) | - (0%) | +/- (40%) | - (3%) | -(6%) | + (80%) | - (0%) |
| Escherichia paracoli | - (25%) | - (1%) | - (5%) | - (1%) | - (5%) | - (1%) | + (80%) | +/- (40%) | -(20%) | - (3%) | +/- (45%) | + (95%) |
| Shigella dysenteriae | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | +/- (45%) | - (0%) | - (0%) | -(2%) | +/- (30%) | + (100%) |
| Shigella flexineri | - (3%) | - (0%) | - (3%) | - (0%) | - (0%) | - (0%) | +/- (50%) | - (0%) | - (0%) | - (5%) | - (1%) | + (100%) |
| Shigella sonneii | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | +(98%) | -(2%) | + (90%) | + (100%) |

*typical mucoid colonies

# General Comments

- Most identification schemes in textbooks are made some years ago,  for bacteria collected in Western countries. As such, there may be regional and time-related differences. Likewise, bacteria do not read textbooks and even 100% and 0% may have exceptions.

If needed perform confirmatory testing e.g. API and mass spectrometry.

- Some test characteristics may vary according to the type and even brand of medium (ex. MacConkey agar I – III)

- Some IDs are very rare. Please confirm those! Rare things exist but are rare. Rule out mixed flora e.g. Bacillus sp. and Aeromonas sp./Plesiomonas sp./Vibrio sp. (oxidase positive).

- Paracoli : some E. coli have a low metabolic activity and may be negative for some key reactions including lactose fermentation, lysine decarboxylase, gas production and motility. They are referred to as “non-reactive E. coli” or “E. paracoli” and (in older literature) “Alkalescens Dispar”. Biochemically, they may overlap with Shigella spp. and Shigella-serotyping and/or acetate utilization test can be used to differentiate. Most E. coli are positive for acetate utilization, most Shigella spp. are negative for acetate utilization. Acetate utilization test is not described in this procedure.

- Salmonella serotypes negative for H2S production are Salmonella Choleraesuis and Salmonella Paratyphi A – they can be distinguished by Lysine Decarboylase (positive resp. negative, see Table 2. Note that low (“moustache”) or absent production of H2S has been described for many Salmonella serotypes, see also point 6 below.

- H2S production:

  - Salmonella sp.: clearly edged H2S shoulder (fig. 1 left)

  - Citrobacter freundii: dirty edged H2S production (fig. 1 right)

  - Salmonella Typhi produces a trace (“moustache”) of H2S, but other serotypes may also produce low H2S quantities (fig.2)

Figure 1: Kligler reaction of Salmonella Typhimurium (left) and Citrobacter freundii (right)

Figure 2: Salmonella spp. Kligler reaction (side and front view): From left to right Salmonella Typhimurium, Salmonella Typhi, 2x Salmonella Enteritidis – note the Salmonella Enteritidis has an atypicially low H2S production.

# Abbreviations

| KIA | Kligler Iron Agar |
| --- | --- |
| G | Glucose fermenting |
| L | Lactose fermenting |
| H2S | H2S production |
| M | H2S as “moustache” |
| MIU | Motility Indole Urease |
| LDC | Lysine Decarboxylase |
| ONPG | Beta-Galactosidase |
| ODC | Ornithine Decarboxylase |
| PGUA | Beta-Glucuronidase |
| ADH | Arginine Dihydrolase |
| API | Analytic Profile Index |
| TLM | Tropical Laboratory Medicine |
| ID | Identification |

# References

- Manual of  Clinical Microbiology 8th Edition, Editor in Chief: Patrick R. Murray ISBN 1- 55581-255-4, ASM Press, Washington D.C., USA

- Manual of Clinical Microbiology 10th Edition, Editor in Chief: James Versalovic; ISBN 978-1-55581-463-2, ASM Press, Washington D.C., USA

-Diatabs, User’s guide Diagnostic tablets for bacterial identification. ROSCO Diagnostica, Denmark, 2009.

- Difco and BBL Manual. Manual of microbiological culture media. Beckton, Dickinson and company,  2nd Edition, 2009.

- API20E product insert; bioMérieux, Marcy-L'Etoile, France, 07584E - 2004/05

# Records and archives

| Revision |  |
| --- | --- |
| Version 1.0 | Initial version |
| Version 2.0 | - Percentages inluded - Chart adjusted for easy use - Tables presented based on particular common characteristics |
| Version 3.0 | - Tables combined according to H2S and Motility result - Flowchart removed - Lactose removed as identifying characteristic - Addenda, general comments and abbreviations added |
| Version 3.5 | - Comment added at table 3 on confusion of Klebsiella spp. with Enterobacter spp. - C. freundii added to table with all isolate results (Addendum) - Addition of API20E product insert as reference for biochemical reaction results. - General comment 4: acetate utilization test added for differentiation E. coli and Shigella spp. |

| Name and function | Date | Comments |
| --- | --- | --- |
| Author |  |  |
| Barbara Barbé | 09/05/2019 |  |
| Reviewed by |  |  |
| Sien Ombelet Marjan Peeters | 04/07/2019 |  |
| Approved by |  |  |
| Jan Jacobs | 05/07/2019 |  |

# Addendum

- Tables with test results

Table 1: Results observed on Kligler Iron Agar (KIA)

|  | Observation |  | Interpretation | Result Recorded |
| --- | --- | --- | --- | --- |
| 1. | Butt | Alkaline (red) | No fermentation of glucose | Glucose negative |
|  |  | Acid (yellow) | Fermentation of glucose | Glucose positive |
| 2. | Slant | Alkaline (red) | No fermentation of Lactose | Lactose negative |
|  |  | Acid (yellow) | Fermentation of Lactose | Lactose positive |
| 3. | Cracks, splits or bubbles |  | Production of gas | Gas positive |
| 4. | Black precipitate |  | Hydrogen sulfide (H2S) production | H2S positive |

Table 2: Results observed on Citrate agar

|  | Observation | Interpretation | Result Recorded |
| --- | --- | --- | --- |
| 1. | Alkaline (blue) | Metabolization of Citrate | Citrate positive |
| 2. | Neutral (green) | No metabolization of Citrate | Citrate negative |

Table 3: Results observed on MIU (Motility Indole Urease)

|  | Observation | Interpretation | Result Recorded |
| --- | --- | --- | --- |
| 1. | Cloudy, growth extending from the inoculation line | Motile | Positive |
|  | Clear or growth only on the inoculation line | Non-Motile | Negative |
| 2. | Alkaline (Pink colour through-out the medium)* *Think of Klebsiella sp. when the pink is defined to the top op the medium and when not motile. | Hydrolysis of urea | Positive |
|  | No colour change | No hydrolysis of urea | Negative |
| 3. | Top colours strong red * *Hold the tube slightly horizontal: the test is positive when the liquid running out of the medium is coloured red | Indole production | Positive |
|  | No colour change | No Indole production | Negative |

Table 4: Results observed on DIATABS

| DIATAB | Observation |  | Interpretation | Remarks |
| --- | --- | --- | --- | --- |
|  | + | - |  |  |
| LDC | Blue/violet | Yellow, green, grey, light blue | Decarboxylation of lysine | Add 3 drops of paraffin before incubation |
| ODC | Blue/violet | Yellow, green, grey, light blue | Decarboxylation of ornithine | Add 3 drops of paraffin before incubation |
| ADH | Pink | Yellow, yellow-orange | Enzymatic break-down of arginine | Add 3 drops of paraffin before incubation |
| ONPG | Yellow | Colourless | Enzymatic release of nitrophenol |  |
| PGUA | Yellow | Colourless | Enzymatic release of nitrophenol |  |
| Urease | Purple | Yellow | Hydrolysis of urea |  |

Table 5: results on Mobility Indole Lysine medium

|  | Observation | Interpretation | Result Recorded |
| --- | --- | --- | --- |
| 1. | Cloudy, growth extending from the inoculation line | Motile | Positive |
|  | Clear or growth only on the inoculation line | Non-Motile | Negative |
| 2. | Purple colour through-out the medium | Lysine decarboxylase activity | Positive This color may vary in intensity and may be bleached out to a pale light color due to reduction of the indicator. Lysine-negative cultures produce a yellow medium that may be purple or red on the top. Tubes that show a purple reaction with a red color on top should be incubated for a longer period of time. |
|  | Red to red-brown colour in the top centimeter of the medium | Lysine deaminase activity | Positive |
|  | No colour change | No lysine activity | Negative |
| 3. | Top colours strong red * *Hold the tube slightly horizontal: the test is positive when the water running out of the medium is red | Indole production | Positive |
|  | No colour change | No Indole production | Negative |

- Table with results of all isolates

|  | Lactose | H2S | gas | Citrate | Motility | Urease | Indole | LDC | ODC | ADH | ONPG | PGUA | D-sorbitol | D-mannitol |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Citrobacter diversus | +/- (50%) | - (0%) | + (99%) | + (99%) | + (95%) | +/- (44%) | + (99%) | - (0%) | + (99%) | + (80%) | + (99%) | - (0%) | + (99%) | + (99%) |
| Citrobacter freundii | + (78%) | + (78%)** | + (89%) | + (78%) | + (89%) | +/- (44%) | -(33%) | -(0%) | -0(%) | +(67%) | +(89%) | - (0%) | + (99%) | + (99%) |
| Edwardsiella tarda | - (0%) | + (100%) | + (100%) | - (1%) | + (98%) | - (0%) | + (99%) | + (100%) | +(100%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) |
| Enterobacter aerogenes | + (95%) | - (0%) | + (100%) | + (95%) | + (97%) | - (2%) | - (0%) | + (98%) | + (98%) | - (0%) | + (100%) | - (0%) | + (100%) | + (100%) |
| Enterobacter agglomerans* | +/- (40%) | - (0%) | - (20%) | +/- (50%) | + (85%) | - (20%) | - (20%) | - (0%) | - (0%) | - (0%) | + (90%) | - (0%) | +/- (30%) | + (100%) |
| Enterobacter asburiae | +/- (75%) | - (0%) | + (95%) | + (100%) | - (0%) | +/- (60%) | - (0%) | - (0%) | + (95%) | -(21%) | + (100%) | - (0%) | + (100%) | + (100%) |
| Enterobacter cloacae | + (93%) | - (0%) | + (100%) | + (100%) | + (95%) | +/- (65%) | - (0%) | - (0%) | + (96%) | + (97%) | + (99%) | - (0%) | + (95%) | + (100%) |
| Enterobacter gergoviae | +/- (55%) | - (0%) | + (98%) | + (99%) | + (90%) | + (93%) | - (0%) | + (90%) | + (100%) | - (0%) | + (97%) | - (0%) | - (0%) | + (99%) |
| Enterobacter sakazakii* | + (99%) | - (0%) | + (98%) | + (99%) | + (96%) | - (1%) | - (11%) | - (0%) | + (91%) | + (99%) | + (100%) | - (0%) | - (0%) | + (100%) |
| Escherichia coli | + (95%) | - (1%) | + (95%) | - (1%) | + (95%) | - (1%) | + (98%) | + (90%) | +/- (65%) | - (17%) | + (95%) | + (100%) | + (94%) | + (98%) |
| Escherichia paracoli | - (25%) | - (1%) | - (5%) | - (1%) | - (5%) | - (1%) | + (80%) | +/- (40%) | -(20%) | - (3%) | +/- (45%) | + (95%) | + (75%) | + (93%) |
| Hafnia alvei | - (5%) | - (0%) | + (85%) | - (10%) | + (85%) | - (4%) | - (0%) | + (100%) | + (98%) | -(6%) | + (90%) | - (0%) | - (0%) | + (99%) |
| Klebsiella oxytoca* | + (100%) | - (0%) | + (97%) | + (95%) | - (0%) | + (95%) | + (100%) | + (99%) | - (0%) | - (0%) | + (100%) | - (0%) | + (99%) | + (99%) |
| Klebsiella ozaenae* | +/- (30%) | - (0%) | +/- (50%) | - (30%) | - (0%) | - (10%) | - (0%) | +/- (40%) | - (3%) | -(6%) | + (80%) | - (0%) | +/- (65%) | + (100%) |
| Klebsiella pneumoniae* | + (98%) | - (0%) | + (97%) | + (98%) | - (0%) | + (95%) | - (0%) | + (98%) | - (0%) | - (0%) | + (99%) | - (0%) | + (99%) | + (99%) |
| Morganella morganii | - (1%) | - (20%) | + (90%) | - (0%) | + (95%) | + (95%) | + (95%) | - (1%) | + (95%) | - (0%) | - (10%) | - (0%) | - (0%) | - (0%) |
| Proteus mirabilis | - (2%) | + (98%) | + (96%) | +/- (65%) | + (95%) | + (98%) | - (2%) | - (0%) | + (99%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) |
| Proteus vulgaris | - (2%) | + (95%) | + (85%) | - (15%) | + (95%) | + (95%) | + (98%) | - (0%) | - (0%) | - (0%) | -(1%) | - (0%) | - (0%) | - (0%) |
| Providencia alcalifaciens | - (0%) | - (0%) | + (85%) | + (98%) | + (96%) | - (0%) | + (99%) | - (0%) | - (1%) | - (0%) | - (1%) | - (0%) | - (0%) | - (2%) |
| Providencia rettgeri | - (5%) | - (0%) | - (10%) | + (95%) | + (94%) | + (98%) | + (99%) | - (0%) | - (0%) | - (0%) | - (5%) | - (0%) | - (0%) | + (100%) |
| Providencia stuartii | - (2%) | - (0%) | - (0%) | + (93%) | + (85%) | - (30%) | + (98%) | - (0%) | - (0%) | - (0%) | - (10%) | - (0%) | - (0%) | - (10%) |
| Salmonella Choleraesuis | - (0%) | - (25%) | + (95%) | - (25%) | + (95%) | - (0%) | - (0%) | + (95%) | + (100%) | +/- (55%) | - (0%) | - (0%) | + (90%) | + (98%) |
| Salmonella Non-Typhi | - (1%) | + (100%)* | + (96%) | + (95%) | + (95%) | - (1%) | - (1%) | + (98%) | + (97%) | + (70%) | - (2%) | - (0%) | + (95%) | + (100%) |
| Salmonella Paratyphi A | - (0%) | - (10%) | + (99%) | - (0%) | + (95%) | - (0%) | - (0%) | - (0%) | + (95%) | -(15%) | - (0%) | - (0%) | + (95%) | + (100%) |
| Salmonella Typhi | - (1%) | Moustache | – (0%) | - (0%) | + (97%) | - (0%) | - (0%) | + (98%) | - (0%) | - (3%) | - (0%) | - (0%) | + (99%) | + (100%) |
| Serratia marcescens** | - (2%) | - (0%) | +/- (55%) | + (98%) | + (97%) | - (15%) | - (1%) | + (99%) | + (99%) | - (0%) | + (95%) | - (0%) | + (99%) | + (99%) |
| Shigella dysenteriae | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | +/- (45%) | - (0%) | - (0%) | -(2%) | +/- (30%) | + (100%) | -(30%) | - (0%) |
| Shigella flexineri | - (3%) | - (0%) | - (3%) | - (0%) | - (0%) | - (0%) | +/- (50%) | - (0%) | - (0%) | - (5%) | - (1%) | + (100%) | -(29%) | + (95%) |
| Shigella sonneii | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | - (0%) | +(98%) | -(2%) | + (90%) | + (100%) | -(2%) | + (99%) |