| Logo partner institute | Department: Clinical Sciences | Unit: Tropical Laboratory Medicine |
| --- | --- | --- |
|  | Work-up of grown blood cultures |  |
|  | Project/study: NA |  |

# Scope and application

The grown blood culture bottle is processed further to ultimately get an identification and antibiotic susceptibility result. This SOP describes which steps are taken to correctly process the blood culture when it has signs of growth.

# Responsibilities

| Function | Activities |
| --- | --- |
| Staff working with blood cultures (lab technician) | Complies with this procedure Records the results of the subcultures and final identification on the laboratory form for blood cultures Contacts the clinician immediately once the identification of the isolate is known |

# Materials

- Disposable gloves

- Sterile plastic pasteur pipettes, disposable

- 21G needle and 5 ml syringes

- Sharps container

- 70% isopropyl alcohol for disinfection

- Plastic or glass sterile tubes of approximately 5 ml

- Cryotubes of 2 ml

- Microscopic slides with frosted ends

- Dyes for Gram staining (see TLM_PRO_104 Gram stain)

- Microscope (100 x objective)

- Immersion oil

- Blood agar (or GC agar base with dehydrated haemoglobin powder)

- Chocolate agar

- McConkey III agar

- Mannitol salt agar (MSA)

- Optochin disks

- Bacitracin disks

- Colistin disks

- Nalidixic acid disks

- Sterile tweezers

- Sterile inoculation loops

- Bunsen burner

- Incubator at 36±1°C

- Candle Jar/ CO2 jar (GaspakTM EZ CO2 Pouch)

- Labels/marker

# Procedures

- If a blood culture bottle shows signs of growth, document the date and time of positivity on the laboratory form

- If a safety cabinet is present, perform the preparation of the Gram stain and subculture in the safety cabinet.

- If no safety cabinet is present, follow the normal precautions procedures (see TLM_RD_004 Biosafety level guidelines).

- Take extra precautions when working with samples suspected of Brucella (see also TLM_RD_004):

  - Use biosafety cabinet if available

  - If no biosafety cabinet: use disposable gloves, (surgical) mask and safety goggles or face shield (PPE)

  - Put the disposable gloves and cultures for waste in an autoclavable plastic bag. Autoclave this bag, then dispose it as infectious waste

  - Only use disposable (plastic) loops, not metal loops and no Bunsen burner

4.1 Preparation of the Gram stain and conservation of blood culture broth

- Label a slide with a frosted end with the blood culture bottle number  and the date

- Take a tube and label it with the blood culture bottle number and the date

- Also label 2 cryotubes with “BCF” (for blood culture fluid), the blood culture bottle number and the date

- Disinfect the rubber membrane of the bottle with 70% isopropyl alcohol

- Gently mix the bottle by inverting it 3 times

- Put on gloves

- Turn the screw cap of the bottle (preferred) or insert a 21G needle attached to a 5ml syringe through the septum if bottle can’t be opened with screw cap

  - Screw cap: insert a plastic pipette into the bottle (Pasteur pipette)

  - Needle: After perforating the septum, gently remove the syringe to release excess gas. Re-attach the syringe to the needle. Tilt the bottle and cautiously aspirate +/- 5 ml of blood culture suspension in the syringe. Remove the syringe without removing the needle from the blood culture bottle

- Transfer a few drops of blood culture broth from the pipette or the syringe into the labelled tube. Transfer approximately 2 ml of blood culture broth in each of the 2 labelled cryotubes.

- Put one drop of the blood culture broth on the microscopic slide

- Discard the pipette and close the blood culture bottle in case of screw cap; otherwise, connect  the syringe again to the needle in the blood culture bottle; remove the syringe with the needle and discard in a sharps container

- Perform the Gram stain as described in TLM_PRO_104

- Document the result of the Gram stain on the laboratory form.

- Report the Gram stain result to the clinician as soon as possible (preferably by telephone call). Record the conversation with the clinician on the laboratory form (see section 4.5).

4.2 Subculture on solid media

- If you see bacteria in the Gram stain:

  - Do not put the bottle back in the incubator

  - Verify Table 1 to select the correct agar media to inoculate, depending on the Gram stain result

  - Perform subcultures following the procedure below

  - Put the bottle on the bench until 1 week after the report of the result

- If you don’t see bacteria in the Gram stain:

  - Perform subculture on blood agar following the procedure below

  - Transfer the bottle back into the incubator.

Procedure for performing subculture

- Label the agar plates (blood agar, chocolate agar, McConkey agar, MSA) on the bottom with the blood culture bottle number, patient hospital number and the date of inoculation

- Transfer one drop of blood culture suspension from the labelled tube (used for Gram stain in section 4.1) on blood agar, chocolate agar and McConkey agar (according to Table 1), and streak the blood culture suspension onto the agar media as described in Annex 1.

- If needed (see Table 1), place a disk containing optochin/bacitracin/colistin/nalidixic acid on the inoculated agar with sterile tweezers

- Incubate the agar plates overnight at 36±1°C: MacConkey agar under aerobic conditions, blood agar and chocolate agar in CO2 – atmosphere (candle jar)

Table 1: Subculture of blood culture broth on agar plates according to Gram stain result

| Gram stain result | Agar media to inoculate |
| --- | --- |
| Gram-positive cocci in grape-like clusters | Blood agar + mannitol salt agar |
| Gram-positive cocci in chains | Blood agar + add a disk of optochin and a disk of bacitracin in section A (see Annex 1) |
| Gram-positive rods | Blood agar |
| Gram-negative rods | McConkey III agar + blood agar; add disk of colistin on blood agar in regions with high prevalence of Burkholderia |
| Gram-negative cocci | Chocolate agar |
| Yeasts | Blood agar |
| Mixture Gram-negative – Gram-positive | Blood agar + McConkey III agar On Blood agar: add a disk of colistin and a disk of nalidixic acid in section A (see Annex 1) |
| Gram stain negative: no bacteria found | Blood agar |

4.3 Storage of blood culture broth and bottles

- Store the 2 cryotubes with 1.5 ml blood culture broth as described in TLM_PRO_112.

- Store all grown blood culture bottles for at least 2 months (for QC purposes) and do not discard before the QC results of the grown bottles have been received by the reference laboratory in the country.

4.4 Reading of the plates

- After overnight incubation of the plates, read the plates and document the growth characteristics on the laboratory form.

- If no growth on the plates: re-incubate the plates and read after another overnight incubation.

  - In case of Gram-negative rods on Gram stain and no growth on blood agar or McConkey agar, inoculate a chocolate agar plate from the blood culture broth, using the procedures described above (section 4.1 & 4.2). Do not repeat Gram stain. Incubate the chocolate agar overnight at 36±1°C in CO2 atmosphere (candle jar/CO2 jar).

  - If there is growth of colonies on chocolate agar after 24h-48h of incubation, consider the strain as fastidious organism (most probably Haemophilus species) and send to reference laboratory

- If there is growth on one or more of the agar plates:

  - Use the grown colonies to perform further steps of identification according to the identification SOPs (see flowchart in Appendix 1).

  - Define the following bacterial species as contaminants if growth retrieved from a single bottle:

      - Coagulase-negative staphylococci

      - Bacillus species

      - Corynebacterium species

      - Cutibacterium species (former Propionibacterium)

      - Lactobacillus spp.

  - Perform susceptibility testing according to TLM_PRO_099: Susceptibility testing.

  - Store each isolate on 2 TSA tubes, as described in TLM_PRO_081.

- Label the TSA tubes with blood culture set N° and blood culture bottle ID.

- Send 1 TSA tube to the reference laboratory (each month), keep the 2nd TSA tube as back-up on site.

  - Store the isolate at the reference laboratory on microbank at -80°C. Refer to TLM_PRO_082.

- If there is no growth after two overnight incubations, see section 4.4

4.4 No growth on agar

If there is no growth on any of the agars after two overnight incubations, follow one of the following options (depending on the initial Gram stain result):

- Bottle showed signs of growth AND initial Gram stain showed bacteria:

  - Repeat Gram stain from the bottle

  - If you see bacteria in the Gram stain again:

    - Repeat subculture of bottle on the agar plates (see Table 1)

    - If there is growth on the agar plates this time, perform further steps of identification (see Appendix 1)

    - If there is again no growth on the agar plates:

      - Report Gram stain result with the following comment on the laboratory form: “identification of bacteria impossible due to insufficient growth”

      - Re-incubate the bottle until the end of the incubation time (day 8);

      - Perform a subculture on blood agar on day 8;

        - If no growth on blood agar after overnight incubation: remove the blood culture bottle from the incubator and record the date of removal on the laboratory form.

        - If there is growth on blood agar, inform the clinician immediately of this result and perform further steps of identification

  - If you don’t see bacteria in the Gram stain:

    - Re-incubate the bottle until the end of the incubation time (day 8);

    - Perform a subculture on blood agar on day 8 and remove the bottle from the incubator;

    - Record the date of removal on the laboratory form

    - If no growth on the blood agar after overnight incubation: report as “no growth”.

    - If there is growth on the blood agar, perform further steps of identification (see Appendix 1)

  - If you are unsure about the Gram stain result:

    - Take a picture through the ocular of the microscope (see TLM_PRO_111)

    - Send the picture to jjacobs@itg.be or sombelet@itg.be

- Bottle showed signs of growth AND you did not see bacteria on the Gram stain:

  - Re-incubate the bottle until the end of the incubation time (day 8);

  - Perform a subculture on blood agar on day 8 and remove the bottle from the incubator;

  - Record the date of removal on the laboratory form

  - If no growth on the blood agar after overnight incubation: report as “no growth”

  - If there is growth on the blood agar, perform further steps of identification (see Appendix 1)

4.5  Reporting of results

- Record the results of the Gram stain and subcultures on the laboratory form

- Inform the clinician as soon as you see bacteria in the Gram stain:

  - Always communicate the Gram stain results from blood culture to the clinician immediately, preferably by telephone call

  - Record who you have called and when you called and sign off with your name (or initials) and paragraph. Depending on the site, you may record this on the worksheet, the laboratory log book or on a separate form or logbook.

# Bibliography

- “Essential procedures for Clinical Microbiology”. Henry D. Isenberg. 1998 American Society for Microbiology, Washington DC.

- Reimer GL et al. Update on detection of bacteremia and fungemia. Clin Microbiol Rev. 1997; 10(3): 444-465

- Bruins et al. Does electronic clinical microbiology results reporting influence medical decision making: a pre- and post-interview study of medical specialists. BMC Med Inf & Decision Making. 2011;11:19

| Appendices & Forms for completion |  |
| --- | --- |
| Number | Title |
| Appendix 1 | Initial identification flowchart for bacteria isolated from an aerobic blood culture |

| Revision |  |
| --- | --- |
| TLM_PRO_109 Version 1.1 | Initial version |
| TLM_PRO_109 Version 1.2 | Small revision: added MSA and bacitracin to Table 1 (14/02/2018) |
| TLM_PRO_109 Version 1.3 | Small revision: addition of “time of growth” on the laboratory form (29/03/2018) |
| TLM_PRO_109 Version 1.4 | Addition of list of contaminants defined by Leber 2016 (p.3.4.1.6) (07/08/2018) |
| TLM_PRO_109 Version 1.5 | Addition of referral to Appendix 1 (27/05/2019) |
| TLM_PRO_109 Version 1.6 | Small revision: no growth on blood agar but growth on chocolate agar is indicative of fastidious species (24/06/2019) |

| Name and function | Date | Comment |
| --- | --- | --- |
| Author |  |  |
| Sien Ombelet | 07/08/2017 | Latest revision 24/06/2019 |
| Reviewed by |  |  |
| Barbara Barbé | 12/01/2018 |  |
| Approved by |  |  |
| Jan Jacobs | 26/01/2018 | Refer to mail BB 26/01/2018 |

Annex 1: How to streak blood culture broth/blood mixture on agar plate (optochin/nalidixic acid disks should be put in section A)