Western Blot Protocol

* Warm tris-glycine gel to room temperature before use

* Thaw protein samples on ice

| Biotin Ladder - 4 µl biotin - 4 µl 4x buffer - 4 µl H2O (milliQ) | 5% Milk - 2.5g blotting grade blocker - 50 mL TBST * It is helpful to add some of the TBST to the 50 mL conical tube before adding the blotting grade blocker so that the blotting grade blocker does not stick to the tube |
| --- | --- |
| Running Buffer (store at 4°C) - 100 mL 10x T/G buffer - 900 mL RO H2O - 5 mL SDS | Transfer Buffer (store at 4°C) - 100 mL 10x T/G buffer - 200 mL methanol - 700 mL RO H2O |

Gel Loading

- Vortex and briefly spin protein samples

- Make biotin ladder & dilute samples with lysis buffer according to BCA calculations

- Vortex & spin samples & biotin ladder

- Heat samples at 95°C for 10 minutes, heat biotin ladder at 95°C for 2 minutes

- While samples are heating, prepare the gel.

  - Open gel and peel off white tag

  - Place gel into voltage box so that the gel and box align at the bottom

  - Fill voltage box with running buffer

  - Blow out the wells

- Briefly spin down samples and biotin ladder

- Load samples into gel

  - 12µl Biotin ladder

  - 6 µl Kaleidoscope

  - 20 µl protein sample

Running Conditions: programmed under methods→ “westgel”

- 75V for 30 minutes

- 150V until complete (~ 1 hr.)

Transfer

- Mark one side of the membrane in pencil to differentiate top and bottom side

- Soak sponges and whatman paper (4/gel) into transfer buffer

- Place sponges and 2 whatman paper into transfer box. Gently roll over the whatman paper

- Dip the membrane in methanol → water → transfer buffer (order specific!)

- Carefully place the membrane on top of the gel in the correct orientation so that when the gel transfers to the membrane, it can be read from left to right

- On top of the membrane, add 2 more whatman paper and gently roll

- Add enough sponges to fill the transfer box

- Place the other half of the transfer box on top, and squeeze it to remove excess transfer buffer and place it into the voltage box

- Fill the inside (containing the membrane) with transfer buffer and the outside with water

- Begin transfer

Transfer Conditions: programmed under methods→ “transfer”

- 20V for 2 hours

Remove membrane from transfer box and leave it in 5% milk (enough to cover membrane) for an overnight block

- Add 12 mL of primary antibody to membrane and put on shaker for 1 hour

- Wash with TBST 3X for 10 minutes each

- Add secondary antibody to membrane and put on shaker for 1 hour

- Wash with TBST 3x for 10 minutes each

Develop

- Set FluorChemQ machine to (1) no filter “chemiwesterns”

- Make sure aperture is opened (turned all the way right)

- Hit acquire → live

- Move black plate to top spot

- Bring paper into focus by using the top knob

- Mix 500 µl peroxide buffer with 500 µl luminol enhancer

- Tap off excess liquid from membrane → place on another plate → apply peroxide/ luminol mixture evenly to the membrane surface → place into FluorChemQ and close door

- Click

  - Load protocol → chemi-high media

  - Display → chemi display

  - Emission → chemi channel assign