Western Blotting Protocol

Protein prep

-Obtain ~4 million cells from each cell line growing in culture.

-Spin down at ~1000 rpm for 5 min, remove supernatant.

-Wash cells twice with 5 mLs of cold 1xPBS, spin down cells again after each wash at ~1000 rpm for 5 min and remove supernatant. Can store cell pellet at -20 C.

-Lyse with RIPA buffer + 1x Halt Protease Inhibitor + EDTA.

-Prepare lysing solution- For every 1 mL of RIPA buffer add 10 uL of the Halt Protease Inhibitor Cocktail and 10 uL of EDTA solution (for inhibiting metalloproteases).

-For 10 samples: 10 x 0.5 mL = 4.9 mLs of RIPA + 50 uL of Halt Cocktail + 50 uL of EDTA.

-Add 0.5 mL of RIPA buffer for 5 million cells. Pipette up and down to suspend the pellet.

-Shake mixture gently for 15 min on ice.

-Centrifuge mixture at ~14,000 x g for 15 min to pellet cell debris.

-Transfer supernatant to new tube. Store at -20 C or -80 C.

Prepare protein standards

-Protein standard is at a concentration of 1.59 mg/mL. Prepare 1:2 serial dilutions.

-Take 40 uL of the 1.59 mg/mL protein standard from the bottle.

-20 uL of 1.59 mg/mL+ 20 uL of RIPA buffer= 0.8 mg/mL

-20 uL of 0.8 mg/mL + 20 uL of RIPA buffer= 0.4 mg/mL

-20 uL of 0.4 mg/mL + 20 uL of RIPA buffer= 0.2 mg/mL

-Should have 4 different protein concentrations: 0.2, 0.4, 0.8, 1.6 mg/mL.

Measure protein concentration

-Perform DC protein assay.

-Use 96 well microplate.

-Prepare reagent A’ (for use with detergents)- Add 20 uL of reagent S to each 1 mL of reagent A.

-Add 5 uL of each protein standard and sample onto a well in triplicate.

-Add 25 uL of reagent A’ into each well.

-Add 200 uL of reagent B into each well.

-Gently agitate the plate to mix the reagents for 5 sec. If bubbles form, pop them.

-After 15 min, read absorbance at 750 nm with a plate reader. The absorbances will be stable for ~1 hour.

-Graph the absorbance values obtained from the protein standards (absorbance on the x-axis and protein concentration on the y-axis) to arrive at an equation (from the line of best fit) to calculate the protein concentration from the absorbance values of each of the samples. Since each standard and sample was done in triplicate, average out the absorbance values first.

Make resolving SDS-PAGE gels

-Set-up the glass gel plates on the plastic base with the grey rubber gasket to form a tight seal. Check for leaks by adding ddH2O. Remove ddH2O by tilting the whole apparatus.

-Set-up 2 15 mL conical vials for the resolving and stacking gels.

-The recipe for 2 12% SDS PAGE gels:

-Resolving (~10 mLs) portion

-4 mL ddH2O

-3.3 mL 30% acylamide bis:tris 37:1 mix

-2.5 mL 1.5M Tris pH 8.8

-100 uL 10% SDS

-100 uL 10% Ammonium persulfate

-4 uL TEMED

-Stacking (~10 mLs) portion

-6.8 mL Water

-1.7 mL 30% acylamide bis:tris 37:1 mix

-1.25 mL 1M Tris pH 6.8

-100 uL 10% SDS

-100 uL 10% Ammonium persulfate

-10 uL TEMED

-Mix all the reagents together for each portion except for the 10% Ammonium persulfate and TEMED. These should be added just prior to pouring.

-First pour the resolving gel up to ~1 cm below comb. Add isopropanol on top to even out the top edge of the gel. Allow to solidify for ~20 min.

-Pour out the isopropanol by tilting the whole apparatus. Next pour the stacking gel leaving some room to fit the comb.

-Place 10 well comb. Allow to solidify for ~20 min.

Reduce and denature proteins

-From the protein concentrations calculate volume needed to load ~20-60 ug of protein. Approximately equal amounts of protein should be added to each well. Each well can hold up to ~50 uL.

-Prepare tubes for each sample to be loaded with 2x the calculated volume of each sample. Add 2x or 4x Laemmli buffer containing beta-mercaptoethanol (BME) to each sample. Prepare a blank as extra for loading.

-For 250 uL of 4x Laemmli buffer, use 225 uL of 4x Laemmli buffer + 25 uL of BME.

-For 200 uL of 1X Laemmli in RIPA buffer (blank)- 50 uL of 4x Laemmli buffer + 150 uL of RIPA buffer

-Place cap lock on each tube and boil samples for 10 min.

Load and run gels

-Place gels in tank and add running buffer (25mM Tris, 192mM Glycine, 0.1% SDS, pH 8.3) to the indicated line on the tank. Load each sample and 10 uL of BioRad prestained Precision Plus Dual Color Protein Standard.

-To equalize the volumes loaded, calculate the difference between the volume in the well with the most sample and the volume of sample present in each well. Load that calculated volume difference with the blank sample in each well.

-Make sure the lid is placed on the tank in the correct orientation.

-Run gels at 90 V for ~2 hrs until the bromophenol band arrives at the bottom and there is adequate separation of marker bands. Check that there are bubbles flowing from the bottom up in the running buffer.

Transfer protein band from gel to membrane

-Place transfer buffer (25mM Tris, 192mM Glycine, pH 8.3, 20% MeOH) in the refrigerator some hours prior to cool down.

-Open the glass containing the gels and carefully remove and discard the small gel pieces separating the wells. Place the gel in cold transfer buffer for 10-15 min.

-Cut the PVDF membrane and blotting paper in the size of the gel. Cut 1-2 corners of the PVDF membrane for orientation.

-Wet the PVDF membrane in methanol for at least 5 min. Handle only with tweezers at the corners.

-Wet the blotting/filter paper and sponges with the transfer buffer.

-Assemble the sandwich in the cassette in this order- sponge, blotting paper, membrane, gel, blotting/filter paper, sponge. Make sure it is in the correct orientation when placed in the tank.

-Place ice pack in tank and place tank on ice. Fill tank with transfer buffer.

-Run at 100 V for 1 hr.

-Can ensure proper transfer of proteins to the membrane by immersing in Ponceau S red dye for 5 min on rocker. Wash extensively in water until it runs clear and protein bands are well defined. Protein bands should appear pink-red. Destain the membrane with repeated washing in TBST. Reactivate PVDF membrane with methanol for 5 min and wash again in TBST.

Block membrane

-Place membrane in ~25 mL of TBST with 5% milk powder on a rocker at RT for 1 hr.

-Wash 3 times for 5 min each time in TBST on a rocker at RT.

Primary antibody incubation

-Incubate membrane in primary antibody diluted in ~25 mL of TBST at 4 C O/N on the rocker. Use antibody manufacturer recommended dilution.

-Wash 3 times for 5 min each time in TBST on a rocker at RT.

Secondary antibody incubation

-Incubate membrane in the appropriate secondary antibody diluted in ~25 mL of TBST at RT for 1 hr on a rocker.

-Wash 3 times for 5 min each time in TBST on a rocker at RT.

Develop and image membrane

-Develop using Millipore Immobilon Western Chemiluminescent HRP Substrate. Bring to room temperature, keep separate and away from light. Mix an equal volume of solution 1 and 2 (0.125 mL/cm2 final volume, ~3 mL for an average sized membrane).

-Drain excess wash buffer from the washed membranes and place them protein side up on a clean surface.

-Pipette the mixed detection reagent onto the membrane. Incubate for 1 min at RT.

-Drain off excess detection reagent by holding the membrane gently with forceps and touching the edge against a tissue.

-Wrap the membranes in plastic wrap and gently smooth out any air bubbles.

-Image the membrane on the BioRad ChemiDoc Imaging System. Click on Image Lab program  New Protocol  Gel Imaging

-Application  Blots  Chemiluminescence

-ensure filter is in correct “no filter” position

-Select gel type  Bio-Rad Ready Gel

-Image Exposure- Automatic optimization or manual set exposure time

-To visualize the marker bands-

-Application  Custom  White light epi illumination

-Save and print desired images.

-Use ImageLab software to merge images and make annotations.

Stripping and reprobing the membrane

-Make up stripping buffer solution (20 mL 10% SDS, 12.5 mL Tris HCl pH 6.8, 67.5 mL H2O, 0.8 mL mercaptoETOH).

-Submerge the membrane in ~20 mL of stripping buffer and incubate at RT for 20 min on the rocker.

-Depending on the affinity of the antibodies used, stripping may require additional time and higher temperature. Can test for removal of the primary and secondary antibodies by incubating the membrane in secondary antibody and detection agent and then imaging.

-Wash in 1xTBST for 10 min

-Reblock membrane in ~25 mL of TBST with 5% milk powder on a rocker at RT for 1 hr.

-Wash 3 times for 5 min each time in TBST on a rocker at RT.

-Continue with primary and secondary antibody incubation and imaging as described above.

Storage

The membranes may be stored wet and wrapped in a refrigerator (2–8°C) after each immunodetection.