Title: Whole Blood Processing (SepMate)

Purpose: To purify PBMCs from 50mL whole blood (per donor). SepMate tubes will be used with Lymphoprep to ease processing time and effort.

Materials

- SepMate-50 (STEMCELL)

- Lymphoprep (STEMCELL)

- EasySep Buffer (STEMCELL)

- DPBS (Ca/Mg free; Fisher)

- 15mL conical tubes

- 50mL conical tubes

- 1.5mL low bind tubes

- Freezing media A (100% FBS)

- Freezing media B (20% DMSO in FBS)

- 2.0mL Cryovials

- Freezing container

Protocol

Plasma separation

- For each donor, 50mL blood will be collected in 5 BD heparin tubes (green top).

- Centrifuge blood samples at 1,200xg for 10min at RT with swinging-out rotor and brake OFF within 6 hours of collection.

- Carefully transfer 3mL of plasma from each tube in a 15mL tube avoiding the buffy coat, use remaining blood for PBMC isolation.

- Centrifuge plasma at 16,000xg in fixed-angle rotor for 10min at 4°C (if a high-speed centrifuge for 15mL tubes is not available, aliquot plasma in multiple 1.5mL tubes and centrifuge at 16,000xg). Transfer plasma to new tubes avoiding the pellet.

- Aliquot 1mL plasma in multiple 1.5mL low bind tubes, freeze at -20°C on the day of collection and move plasma samples to -80°C after 24 hours.

PBMC isolation

- Prepare SepMate tubes with Lymphoprep

  - Prepare 3 SepMate tubes per donor and add 15mL Lymphoprep below the insert by pipetting into the center hole.

    - Take care to minimize any air bubbles below the plastic divider.

    - Video from SepMate manufacturers (~2 min)

https://www.youtube.com/watch?v=K9IlBxjgLt4

- Dilute whole blood 1:2 with DPBS (Ca/Mg free).

  - 105mL total (35mL residual blood and 70mL DPBS)

- Slowly pipette 35mL diluted blood down the side of the tube.

  - Some mixing may occur between the medium and blood, but take care not to mix under the divider.

  - Unlike conventional gradient separation, do not tilt the tube when adding diluted blood.

- Spin at 1,200xg for 15min at 20°C with brakes ON.

  - Prepare 3 destination 50mL tubes (~30 mL per Sepmate tube will be yielded.)

- When the spin finishes, pour off the top layer from the SepMate tubes into your prepared destination tubes.

  - Do not invert the tubes for more than 2 seconds.

- Spin 450xg for 10min at RT with brake ON

  - First wash at higher RCF because gradient medium is mixed with the cells.

- Pour off supernatant and combine pellets in three tubes to one, add EasySep buffer to 50mL.

- Spin 350xg for 10minutes at RT with brakes ON.

  - If yield is critical, ensure that the supernatant is clear; opaque supernatant indicates incomplete centrifugation of cells. If necessary, perform the spin at 450xg.

- Pour off supernatant, wash in 50mL EasySep buffer for a second wash.

- Spin 350xg for 10 min at RT with brakes on.

- Pour off supernatant and resuspend in 20mL EasySep buffer.

- Perform cell count.

- Spin at 350xg for 10 minutes and resuspend in appropriate volume of freezing media A to 10e6 cells/mL.

- Prepare freezing tubes with 500µL freezing media B.

- Add 500µL cell suspension from step 18 to cryovials and gently mix by pipetting.

- Place in freezing container for 1 day at -80°C and move to LN2 tank for long term storage on the following day.