Microscopy checklist generated with Rebecca Senft’s app:

https://rebecca-senft.shinyapps.io/MicCheck/

Montero Llopis P, Senft RA, Ross-Elliott TJ, Stephansky R, Keeley DP, Koshar P, et al. Best practices and tools for reporting reproducible fluorescence microscopy methods. Nat Methods. 2021 Dec;18(12):1463–76.

| Microscope Stand and Motorized Components |  |
| --- | --- |
| Microscope Stand manufacturer and Model | Leica microsystems DMI8 |
| Illumination Shutter Manufacturer and Model | Leica internal shutter |
| Stage Manufacturer and Model | iTK LMT200 |
| Linear encoded stage? | Yes |
| Focusing device manufacturer and model | Leica SuperZ Galvo stage (118507410) |
| Focusing device type | Galvanometric stage |
| Hardware-based Focus maintenance device manufacturer and model | Leica AFC |
| Software based focusing maintenance set up (wavelength, range, step size, algorithm) | No |
| Excitation/Emission filter wheel Manufacturer and Model | No excitation wheel, emission filter wheels are part of the spinning disk system (Yokogawa) |
| Excitation/Emission filter wheel Location in the Lightpath | Emission filter wheels positioned just before the camera |
| *** Upright or inverted | Inverted |
| Spinning Disk Confocal-Specific Hardware and Settings |  |
| Scan unit Manufacturer and Model | Yokogawa CSU-W1-T2 |
| Pinhole size | 50 µm |
| *** Number of disks | Single-disk |
| *** Disk speed | 4000 rpm |
| Illumination |  |
| Laser launch/combiner manufacturer and model | Gataca Systems / iLaunch |
| Laser line/wavelength | 405nm / 488nm / 561nm / 642nm |
| Laser type | Solid state |
| Laser modulation | Direct control |
| Wavelength Selection |  |
| Filter manufacturer and product number | SEMROCK |
| Filter center wavelength and bandwidth (FWHM), cut on or cut off wavelength | FF01 452/45 FF03 525/50 FF01 609/54 FF01 708/75 |
| Filter coating method | Hard-sputtered |
| Optics |  |
| Objective manufacturer | Leica microsystems |
| Objective correction | PLAN APOCHROMAT |
| Objective magnification | x20 |
| Objective numerical aperture | 0.75 N.A |
| Specified immersion medium | Oil |
| Detection |  |
| Camera manufacturer and model | Teledyne Photometrics / Prime 95B |
| Camera type | sCMOS |
| Binning | 1 x 1 |
| Bit Depth and associated gain | 16 |
| EM gain (if EMCCD used) | NA |
| *** Pixel size | 0.55 µm |
| *** Chip size | 1200x1200 pixels |
| *** Pixel readout rate | 100 mHz |
| Acquisition Software |  |
| Software manufacturer, name, and version | Molecular devices / MetaMorph, 7.10.4.407 |
| State of the shutter during acquisition | Always open |
| Order of experimental acquisition | Z-stack was acquired, 1 channel at a time, before moving to another x, y position |
| Sequential, simultaneous, or triggered channel/color acquisition | Images were acquired sequentially |
| Zstack focusing device | SuperZ Galvo stage |
| Zstack range and step size | ~20-30µm range, 1µm step-size |
| Time interval | Exposure times: 200ms (405), 250ms (488), 250ms (555), 300ms (642) |
| Sample Preparation |  |
| Sample holder type, manufacturer and product number | Leica / Universal insert SuperZ type S |
| Coverslip grade | #1.5 |
| Coverslip coating (type, concentration, detailed protocol) | None |
| Detail protocol: fixative, concentration of fixative, fixation conditions (buffers, time, temperature), blocking, binding and hybridization buffer composition, Ab manufacturer, lot number, concentration, probe concentration, binding or hybridization conditions (time, temperature, sequential/simultaneous) | See detailed protocol for staining |
| Mounting/imaging medium name, manufacturer and product number | Fluoromount-G™, Sigma Aldrich |
| Organic dye name, manufacturer and product number | See detailed protocol for staining |
| Organic dye concentration and solvent | See detailed protocol for staining |
| *** Mounting/imaging medium RI if not specified and curing time. | 1.4 RI, 1 week curing time |

FPbase Spectra Viewer link:

https://www.fpbase.org/spectra/?s=222,223,4601,4602,266,267,250,251,829,905,996,1057,1330,$cl0_405,$cl1_488,$cl2_561,$cl3_642&showY=0&showX=1&showGrid=0&areaFill=1&logScale=0&scaleEC=0&scaleQY=0&shareTooltip=1&palette=wavelength

Lambert TJ. FPbase: a community-editable fluorescent protein database. Nat Methods. 2019 Apr;16(4):277–8.