A flexible low-hazard protocol for sample preparation and high-molecular weight DNA extraction from large lepidopterans.

Authors: Jay K Goldberg1,2,3*, Carson W. Allan4, Jayson D Talag5, David A Kudna5, Dario Copetti5, and Luciano Matzkin2,3,4

1 Department of Crop Genetics, John Innes Centre, Norwich, Norfolk, UK

2 Department of Ecology and Evolutionary Biology, University of Arizona, Tucson, AZ, USA

3 BIO5 Institute, University of Arizona, Tucson, AZ, USA

4 Department of Entomology, University of Arizona, Tucson, AZ, USA

5 Arizona Genomics Institute, University of Arizona, Tucson, AZ, USA

*Correspondence: jaykgold@arizona.edu

Abstract: Large insects are an optimal system for genomics studies given the ease with which large quantities of genetic material can be obtained from single individuals. This avoids the need to generate inbred/clonal genotypes or complications associated with sequencing pooled samples of heterogenous individuals. The advent of third generation long-read sequencing has made it easier than ever to produce high quality genome assemblies for non-model species, but acquiring suitable high-molecular weight (HMW) DNA can be a difficult process that often requires the use of hazardous chemicals such as phenol or cetyltrimethylammonium bromide (CTAB). Here, we describe a protocol for extracting HMW DNA from large insects (specifically Manduca sexta) with minimal use of hazardous materials. This protocol is suitable for use on small insects, but larger insects must be dissected prior to DNA extraction. As such, we have also included a detailed protocol for preparing tissues of either adult or larval samples.

Tissue preparation

Supplies:

- 1.7mL microcentrifuge tubes

- Liquid nitrogen (LN2)

- Ice (for caterpillars)

- Razor blade/scalpel

- Forceps

- Plastic bit to protect the bench

- DI water in a small cup or beaker

Protocol:

- Put samples in the fridge immediately and let cool for minimum 30 minutes to anesthetize. Insects should be completely immobile before proceeding.

- Prepare (label tubes, fill ice/ln2 buckets, etc) while the specimens cool

- Cut off adult head or caterpillar head capsule using a razor blade, place in 1.7mL tube and submerge in LN2. Proceed to step 4 before freezing additional tissues.

NOTE: Adult head tissue is unsuitable for DNA extraction using this protocol due to eye pigments and can be discarded.

- Separate somatic tissues into separate tubes and submerge in LN2

  - 4a: adult moths – 1) Remove legs/wings by hand and discard if not desired for RNA-sequencing. 2) Separate thorax and abdomen. Discard abdomen if not desired for RNA-sequencing 3) Gently remove scales from thorax by hand. 4) cut thorax into 2-4 pieces depending on sample size. Tissue pieces should not be larger than 0.5mL.

  - 4b: caterpillars – 1) Separate caterpillar into three sections foregut, midgut, and tail (cut behind the true legs and in front of the last prolegs). 2) Remove gut contents from tail using forceps and dunking in DI water, if necessary, and submerge in LN2. 3) Pull fore/midgut tissue out of cuticle, remove plant material, and submerge in LN2. Gut tissue will be bright yellow, plant material will be dark green. Fat bodies (white) should also be avoided, if possible.4) Place gut-less cuticle pieces into separate 1.7mL tubes and submerge in LN2. Tissue pieces should not be larger than 0.5mL.

- Remove samples from LN2, place in cardboard freezer box, and store at -80C

Genomic DNA Extraction

Solutions:

- Fly-A

Tris HCI buffer 0.1M pH 8.0

EDTA 0.1M pH 8.0

SDS 1%

- Qiagen ProK solution (20mg/mL)

- 4M Potassium acetate

- Chloro+isoamyl 24:1

- 2-Propanol

- 70% Ethanol

- 1X TE

- Qiagen RNaseA

- NaCl (if cleanup is necessary)

Protocol:

NOTE: Do not vortex for any mixing step

- Tissue of individual moths/caterpillars previously prepared are stored at -80°C.

- With tube on ice add 700μL Fly-A solution at room temp and 7.5μL of ProK.

- Use a fresh blue pestle to crush tissue by pressing to the bottom of the tube and twisting until the tissue breaks down a bit and the solution gets some color.

NOTE: Do not be too rough or try to completely grind the tissue to a pulp

- Let sit on ice till all tubes are ready.

- Incubate at 57°C for 60 minutes, gently mix halfway through.

- Cool at 37°C for 3 minutes.

- Add 70μL of 4M potassium acetate and gently mix.

- Incubate on ice for 30 minutes.

- Centrifuge at 4°C for 30 minutes at 18,000 rcf.

- Transfer supernatant to new 1.5mL tube be careful not to transfer debris.

- Add equal volume of chloro+isoamyl 24:1 and mix by rocking back and forth 40 times.

- Centrifuge at 4°C for 5 minutes at 10,500 rcf.

- Transfer the upper phase to a new 1.5mL tube. Be very careful not to get any of the interface.

- Add equal volume of 2-Propanol and slowly rock back and forth. Should see threads of precipitate at this point.

- Centrifuge at 4°C for 5 minutes at 10,500 rcf.

- Remove the liquid leaving the pellet.

- Wash pellet with 1mL of fresh room temperature 70% ETOH and rock back and forth to wash all surfaces.

- Centrifuge at 4°C for 2 minutes at 10,500 rcf.

- Remove all ETOH with a P1000 tip and then use a P10 tip to insure full removal. Dry the pellet in the hood for 10-15 minutes.

- Resuspend with 30μL 1X TE.

20a) If re-precipitating to remove contamination: use 60uL of 2.5M NaCl TE buffer

- Combine all tubes from each sample into a 1.5mL tube.

21a) If re-precipitating: add 2 volumes of 100% EtOH and gently mix. Let sample cool on ice for 30min and repeat steps 15-20.

NOTE: Do not cool NaCl solution in freezer. Lower temperatures result in contaminants co-precipitating.

- Add 3μL of Qiagen RNaseA, mix gently and incubate at 37°C for 30 minutes.

- Nanodrop to check purity.

23a) if 260/230 is low, precipitate by adding 2 volumes of 100% ethanol and cooling to -20C for 30minutes. Return to step 18 and follow steps 20a/21a to remove contamination.