IJESRT

International Journal of Engineering Sciences & Research Technology

PHYSICOCHEMICAL PROPERTIES AND FATTY ACID COMPOSITION OF MANGO SEED KERNEL OIL

Fajriyati Mas’ud*1,2, Meta Mahendradatta3, Amran Laga3, Zainal Zainal3

*1Post-Graduate Students, Hasanuddin University, Makassar, South Sulawesi, Indonesia

2Department of Chemical Engineering, State Polytechnic of Ujung Pandang, Makassar, South Sulawesi, Indonesia

3Department of Agricultural Technology, Hasanuddin University, Makassar, South Sulawesi, Indonesia

DOI: 10.5281/zenodo.1147523

ABSTRACT

Mango seed kernel oil has been extracted from Arumanis mango by multistage extraction with hexane (stage 1) followed with ethanol (stage 2) with the aim that all non-polar and polar components can be extracted. In the present paper, we provide data on physicochemical properties and fatty acid composition of mango seed kernel oil needed by the food, pharmaceutical, and cosmetics industries who are interested in using mango seed oil as its raw material. Extraction was performed at 50oC, for 5 h with mango seed kernel and solvent ratio of 1:4. The physicochemical properties, include water content, specific gravity, refractive index, melting point, density, color, acid value, iodine value, peroxide value and saponification value were examined. Oleic, stearic, linoleic and palmitic acids were found dominant in first stage, it is had 27.27% of saturated fatty acid and 72.73% of unsaturated fatty acid as monounsaturated fatty acid 67.51% and 5.22% of polyunsaturated fatty acid. In the second stage extracted with ethanol, palmitic, linoleic, stearic, and lauric acid were dominant extracted. The saturated fatty acid contained of 77.26%, monounsaturated fatty acid of 1.87%, and polyunsaturated fatty acid of 20.87%. In addition, antioxidant activity of mango seed kernel oil has also been shown.

KEYWORDS: Edible oil, extraction oil, solvent extraction, physicochemical properties, fatty acid

- INTRODUCTION

Mango (Mangifera indica L.) is one of the most important fruit of the nutrition and economic aspect. Mango is consists of edible peel and seed. The seed consists of a tenacious coat enclosing the kernel [1]. It is present about 18% of the total fruit [2]. The kernel content of the seed ranges from 45.7% to 72.8% [3]. MSK contain almost 15% of oil [4]. Variation in oil content in some reports due to the differences in variety of plant, cultivation (soil, climate, etc), ripening stage, the harvesting time and the extraction method of oil used [5, 6]. This can be as a result of difference in climatic conditions of their geographical locations [7].

Mango seed kernel (MSK) is an important source of nutrients and oil [8]. It has mainly been on the lipid component of the kernel, and of its potential application in the confectionery industry as a source of cocoa-butter substitute [9], and could potentially be used in the formulations of food products such as chocolate and biscuits as a natural nutritional additive [10]. The fatty acid (FA) profile is a main determinant of the oil quality, it is important as nutritional substances and metabolites in living organisms, the high quality and nutritional value of mango seed kernel oil (MSKO) has potential application in human foods [11].

MSKO is a promising edible oil, safe and natural source of edible fat, it is not contain trans FA [12]. MSKO is a promising and a safe source of edible oil and was founded to be nutritious and non-toxic, so that it could be substitute for any solid fat without adverse eﬀects [9]. Toxicity was not noticed in rats fed with diets containing 100g/kg crude of MSKO [13]. The lipids were extracted from several varieties of mangoes, free of toxic materials such as hydrocyanic acid [14].

Related to method of oil extraction, solvent extraction result in sesame revealed that polar solvent such as ethanol is better than non-polar solvent such as hexane. It can be explained by the interaction between unsaturated fatty acids and polar solvent, compared with non-polar solvents [15]. On the other hand, hexane is the most commonly used, and preferably for oil extraction from seed, due to its availability at reasonable cost and its functional characteristics suitable for oil extraction. Hexane is capable of dissolving triglycerides at sufficiently low temperatures, not reactive with oil and miscella, and requires simple equipment.

In a previous study, we have reported the optimum conditions of the MSKO extraction process [16] that are urgently needed by the food, pharmaceutical, and cosmetic industries that use MSKO as their raw material. Furthermore, to complement the MSKO study, in this paper, we provide data on the physicochemical properties and the fatty acid composition of MSKO obtained from multistage extraction with hexane followed by ethanol, so that all polar and non-polar components and especially long chains and short chain fatty acids of MSKO can be perfectly extracted. Throughout our literature studies on oil extraction, studies have been reported only on the use of one type of solvent and repeated several times, or using two types of solvent with a certain ratio, no studies have used multistage extraction using non-polar and polar solvents, especially on MSKO extraction. In addition, we also provide data on the antioxidant activity of MSKO in relation to the potential of MSKO to be processed as an oil-based functional food.

- MATERIALS AND METHODS

Arumanis mango, a local mango Indonesia, as samples were obtained from South Sulawesi. Standards of fatty acid methyl ester (FAME) were from Supelco Inc., Bellefonte, PA (Supelco 37 Component FAME Mix), all chemical/reagents were from Merck, Germany, and DPPH were from Sigma Aldric.

Preparation of MSK and extraction oil

Preparation of MSK according to Mahale and Giri [8] with minor modification. The seeds were washed, and the kernel enclosed in the hard cover was separated manually. The kernels were dried in the oven at 50°C for 12 h to a constant weight in order to reduce its moisture content. Separation of thin cover from the kernel was carried out using tray to blow away the cover in order to achieve very high yield. Stainless steel grinder was used to powdered form, sealed in a plastic container and stored in a freezer until extraction to prevention of its oxidation.

At experimental unit, 50 grams of MSK powder were weighed in the reactor 1.0 L four neck flasks, MSK was extracted with 400 mL solvent, using a heating mantle connected with the thermometer setting at 50oC for 5 h, agitator on the top, speed of 200 rpm, residue was separated by centrifugation (refrigerated AX-521 centrifuge) at a speed of 3500 rpm for 20 min. The liquid part accommodated in the flask evaporator, solvents removed on a rotary evaporator Buchi R-215 incorporates vacuum Pomp V-700. MSKO obtained was packaged in a dark glass bottle and stored in a freezer for analysis. The MSK residue from hexane extraction subsequently extracted with ethanol.

Preparation and analysis of fatty acid methyl esters (FAME)

FA composition of MSKO was verified by gas chromatography. Lipids were esterified by method adapted from Metcalfe [17], which consisted of lipid saponification with KOH 0.5 M in methanolic solution and catalyzed BF3-MeOH reagent. The sample was solubilized by dichloromethane, from which 1 µL was injected for GC analyses. To separate and quantify the esterified FA mixture, GC-MS QP 2010 by Shimadzu was used equipped with split/split less injector, capillary column RTX®-1 (30 mx0.25 mmIDx0.25 µm) and flame ionization detector (FID). Helium was used as the carrier gas at flows of 1.25 mL/min. The injector and detector temperature were set to 260oC. The chromatographic conditions for separation were column initial temperature of 50oC, raising to 200oC at a flow rate of 6oC/min, holding during 4 min, the second step consisted in increased at a heating rate of 2oC/min to 240oC, and held for 10 min. FAME peaks were identified by comparing their retention time and equivalent chain length with respect to standard FAME.

Determination of physicochemical properties and antioxidant activity of MSKO

Physical properties of oil included water content, speciﬁc gravity, refractive index, melting point, and chemical characteristics including free fatty acid (FFA), peroxide value, iodine value and saponiﬁcation value were determined according to AOCS [18]. Antioxidant activity of MSKO was assessed using 2,2-diphenyl-1picrylhydrazylhydrate (DPPH) free radical. The DPPH radical scavenging activity of oil extracts was quantified according to Hatano et al. [19]. 0.0016 g of DPPH weighed and diluted with methanol in a 50 mL volumetric flask to obtain a solution of 10 mM. Preparation of standard BHT solution: 100 mg of BHT dissolved with methanol in 100 mL volumetric flask, in order to obtain a solution of 1000 ppm. Create BHT stock solution with a concentration of 25, 50, 100 and 200 ppm. Preparation of polyphenol extract: 100 mg of oil was dissolved in 50 mL methanol, sonication for 20 min and concentrated until methanol evaporated.  Preparation of stock solution sample: 100 mg of polyphenol extract dissolved with methanol in a 100 mL volumetric flask, in order to obtain a 100 ppm standard solution. Created sample stock solution with a concentration of 80, 200, 400, 800 ppm. Measurement of antioxidant activity: 4 mL of BHT solution and 4 mL of sample solution were put into a test tube and added 1 mL of 1 mM DPPH solution, the mixture is homogenized and incubated for 30 min at 37°C. Absorbance was measured at a wavelength of 516 nm. The radical scavenger activity was expressed in terms amount of antioxidants necessary to decrease the initial DPPH absorbance by 50% (IC50). The IC50 value for each sample was determined graphically by plotting the percentage disappearance of DPPH as a function of the sample concentration.

- RESULTS AND DISCUSSION

Physicochemical properties of MSKO

Physical properties including refractive index, melting point, color and odor of MSKO were determined (Table 1). MSKO extracted with hexane was creamy pale yellow in color.  Extracted of MSKO was semi-solid, pale yellow colored fat [8]. Refractive index of both MSKO at 30oC was 1.46 and 1.53, respectively. The refractive index associated with yield. The longer the extraction time, the more the oil yield, so that the refractive index is higher. Refractive index is an indication of quality assurance, analyzing the stability of oil during thermal treatment and the level of saturation of oil [20]. The refractive index of MSKO at 40oC is about 1.359 to 1.559 [21].

Table 1. Physical properties of MSKOa

| Physical properties | Value |  |
| --- | --- | --- |
|  | Stage 1 | Stage 2 |
| Refractive index at 30oC | 1.46±0.01 | 1.53±0.01 |
| Melting point (oC) | 33±0.35 | 31±0.26 |
| Density (g/mL) | 0.83±0.007 | 0.85±0.003 |
| Color | Creamy yellow | Brown |
| Odor | Pleasant | Pleasant |
| Physical state at 27oC | Semi-solid | Semi-solid |

aResults are average of duplicate determinations ±S.D.

The density of both MSKO was 0.83 and 0.85 g/mL, respectively. Density is a measurement of the mass per unit volume. The greater the density of oil, the greater the mass of each volume.  The oil has a density of 0.8 g/mL [22]. Each type of oil has a density that is typical, depending on the FA composition of the oil. The melting point of both MSKO was 32oC and 31oC, respectively. The lower the melting point of oil, the better the oil is for making oil creams [23]. The highest melting point of oil due to the presence of long-chain SFA [24]. MSKO had the lowest melting point as it contains high amount MUFA and PUFA such as oleic and linoleic acids. The melting point of MSKO from different countries ranged from 25 to 47oC [25], 39 to 40oC [26], 25 to 33oC [27].

The chemical properties of oil are the most important properties that determine the present condition of the oil. Chemical properties including the total acid, iodine, peroxide, FFA and saponification values of MSKO were determined (Table 2). FFA and peroxide value are the major characterization parameters for oil quality [11]. The low FFA content is indicative of low enzymatic hydrolysis. The oil with high FFA can develop off flavor during storage.  Low FFA of oil indicate that the oil almost free from hydrolytic rancidity brought almost by lipases, and enables the direct use of such an oil in industries without further neutralization [5]. FFA was determined in both MSKO of 2.11 and 2.39%, respectively.  The FFA of MSKO was about 2.26-3.76% [27].

MSKO had a high quality due to the low level of peroxide value [11]. MSKO founded high quality, the peroxide value of both MSKO were 1.83 and 2.39 mg/kg, respectively. This value were lower than that expected of rancid oil which ranges from 20.00-40.00 mg/g oil [28].  Generally in the fresh oil, the peroxide value should be less than 10 mg/g oil [11].  The peroxide value of MSKO was 1.95 to 1.99 mEq/kg [26]. The peroxide value is a good indicate for the stability of the oil and use as indicator of deterioration of oil. It is measure of the concentration of peroxide and hydroperoxide formed in the initial stages of lipid oxidation. The peroxide value is also as a measure of the extent oxidation or oxidative rancidity of oil.  Oxidative rancidity is the addition of oxygen across the double bonds in unsaturated FA in the presence of enzymes or certain chemical compounds. High peroxide values are associated with higher rate of rancidity [7].

Table 2. Chemical properties of MSKOa

| Chemical properties | Value |  |
| --- | --- | --- |
|  | Stage 1 | Stage 2 |
| FFA, as oleic acid (%) | 2.11±0.2 | 2.39±0.2 |
| Peroxide value (mg/kg) | 1.83±0.2 | 2.35±0.2 |
| Acid value (mg KOH/g oil) | 3.44±0.16 | 4.33±0.59 |
| Iodine value (gram I2/100 g oil) | 53.19±0.45 | 31.27±0.44 |
| Saponification number (mg KOH/g oil) | 191.21±0.99 | 135.27±1.9 |

aResult are average of duplicate determinations ±S.D.

Acid value serves as an indicator for edibility of oil. It is the mg KOH required to neutralize the FFA in 1 g of oil. The acid value is used to measure the extent at which glycerides in oil are decomposed by lipases and other actions such as light and heat, that its determination is often used as general indication of the condition and edibility of oils.  Acid value was determined in both MSKO of 3,44 and 4.33 mg KOH/g oil, respectively, indicates that the oils were edible because it falls within the recommended codex of 0.6 and 10 for virgin and non-virgin edible fats and oil, respectively [20]. In general, the lower the acid value, the more its acceptability for edibility purpose. The acid value of MSKO is about 4.49-7.48 mg KOH/g [27]. The acid values varied from 1.22% for the Egyptian mango variety to 7.48% for the Kenyan Kent mango variety [25].

The iodine value of oil indicates the unsaturation of FA, it was found to be 53.19 and 57.59 g I2/kg oil in both MSKO, respectively. Iodine value is the amount of iodine necessary to saturate 100 g of oil and is a measure of the amount of unsaturation in oil.  It is used to determine in assessing the stability of oil in industrial applications [29].  The iodine value of MSKO ranged from 51.08-56.79g I2/kg oil [27], 40.90 to 56.79 g I2/kg oil [25]. The iodine value of the Kaew mango variety was 40.90 cultivated in Thailand, while the Kagege mango variety was 56.79 cultivated in Kenya, the iodine values of different mango varieties cultivated in India, Bangladesh, Egypt, Mexico and Malaysia were comparable [27, 30]. The MSKO of Arumanis mango had the lowest iodine value, which reflected its characteristic such as higher resistance to oxidation, longer shelf life and higher quality.  Iodine value maybe due to the differences of FA compositions [11]. A highly SFA level of oil is confirmed to be of benefit in terms of storage ability when compared to more unsaturated oil. The iodine value is also an index for assessing the ability of the oil to be rancid [7]. The iodine value obtained from this study indicates that the oil contained appreciable level of unsaturated FA present especially oleic acid.

MSKO Arumanis mango had saponification value of 191.21 and 206.73 mg KOH/g oil, respectively.  The saponification values ranged from a low of 185.4 mg KOH/g for the Thailand Kaew mango variety to a high of 197.0 mg KOH/g for the Bangladesh Ranipas mango variety [25]. The saponification value is the milligram of KOH necessary to saponify 1 g of oil and shows the capacity of forming soaps of oil, it is shows the average mass long-chain molecules or FFA [31]. The saponification value was a useful tool for the evaluation of the chain length or molecular weight of FA occurring in the triacylglycerol in oil [11].  It is value represents the average molecular weight, or chain length of all the FA. Saponification value is inversely proportional to the molecular weight of the glycerides on the oil. The lower saponification value indicates a very high content of low molecular weight triacylglycerol. Saponification value indicates the size or nature of FA chains esterified to glycerol. It gives information about characteristic of the FA and the solubility of their soaps in water. The higher the saponification value of fat free from moisture and unsaponifiable matter, the more the solubility of the soap produced from it [7].

Fatty acid composition of MSKO

The FA profile of MSKO was analyzed by GC-MS (Table 3). The results showed that the group of saturated fatty acid (SFA) was predominantly of MSKO, there were valeric, lauric, myristic, palmitic, margaric, stearic, arachidic, behenic, tricosanoic and lignoceric acids. The data showed that the main FA of MSKO extracted with hexane (first step) was oleic (67%) and stearic acids (19.22%), they together constituted 86.68% of total FA. MSKO had 27.27% of SFA and 72.73% of unsaturated fatty acid (UFA) as MUFA 67.51% and PUFA 5.22%.  On the other hand, all of FA could be well extracted though the result of the second stage with ethanol was still founded.

In general, stearic acid was the main SFA in MSKO extracted with hexane, while oleic acid was the major UFA. The ratio of SFA to UFA were 1:2.67, this ratio indicated that the oil stable and tolerant to rancidity due to high content of UFA.  In the second stage extracted with ethanol, MSKO had 77.26% of SFA and 22.74% of UFA as MUFA 1.87% and PUFA 20.87%. The data showed that the main FA of total lipid was palmitic (37.64%), linoleic (20.87%) and lauric acid (11.97%), they together constituted 70.48% of total FA. In addition, valeric, lauric, myristic, palmitoleic, margaric, arachidic, behenic, tricosanoic and lignoceric acids were minor FA present in MSKO.

Table 3. Fatty acid composition on MSKO extracted by multistage extraction

| Fatty acid | Lipid Numbers | Group | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Stage 1(mg/L) | Stage 2 (mg/L) |
| Valeric | C5:0 | SFA | 0.759 | 0.139 |
| Lauric | C12:0 | SFA | 3.614 | 3.818 |
| Myristic | C14:0 | SFA | 2.744 | 1.82 |
| Palmitic | C16:0 | SFA | 213.511 | 12.006 |
| Palmitoleic | C16:1 | MUFA | 1.698 | 0.131 |
| Margaric | C17:0 | SFA | 2.86 | 0.243 |
| Stearic | C18:0 | SFA | 699.803 | 6.343 |
| Oleic | C18:1 | MUFA | 2456.043 | 0.464 |
| Linoleic | C18:2 | PUFA | 190.178 | 6.658 |
| Arachidic | C20:0 | SFA | 48.981 | 0.222 |
| Behenic | C22:0 | SFA | 8.395 | 0.02 |
| Tricosanoic | C23:0 | SFA | 1.952 | 0.011 |
| Lignoceric | C24:0 | SFA | 10.07 | 0.025 |

MSKO is more stable than many other vegetable oils, because it is rich in PUFA. Such oils seem to be suitable for blending with other vegetable oils, stearin manufacturing, confectionery industry or in the soap industry [32]. The PUFA is very important for the stability of oil because of the chemical reactions, due to occurring at the double bonds. The rates of those oxidation reactions depend on the number of double bonds in the carbon chain. Therefore, oils with a high proportion of oleic acid are more stable than others. Oleic acid is less susceptible to the oxidation than PUFA from the n-6 series (linoleic acid).  Linoleic acid is important for the stability of oil, due to the chemical reactions occurring at the double bonds.  It contributes to health benefits of human body and was preferred by industries when oil hydrogenation was required [11].

The mango fats were rich source of stearic, oleic, palmitic, and linoleic acids [25]. The Egyptian mango contained stearic and oleic acids of total lipids and neutral lipids constituted 84.4% and 86.2% of total FA, respectively [21]. The major FA in MSKO were stearic acid (31.3-41.3%), oleic acid (38.7-42.3%) and palmitic acid (8.311.3%) [33]. The major FA in all the three mango varieties (Saigon, Edward and Julie) evaluated were stearic (37.57-38.80%), palmitic (8.61-9.06%), oleic (43.71-44.91%), linoleic (5.87-7.00%) and linolenic (0.63-0.95%), Saigon had the highest stearic acid content while Edward had the highest palmitic, linoleic and linolenic acid [1].

Oleic acid is an 18-carbon MUFA, essential in human nutrition and helps to reduce triglycerides, LDL-cholesterol, total cholesterol and glycemic index [11]. The increase in stability over oxidation of vegetable oil is attributed to oleic acid [34].  Stearic acid, a long C18 straight-chain SFA, has been found to bind and plasticize composites [35], human serum albumin [36] and α-helical sites in bio-molecules [37], indicating the usefulness of the kernels of these mangoes for those purposes. The high quality and nutritional value of MSKO has potential application in human foods. The high content of oleic and linoleic acids make MSKO a potential source of nutrient rich food oil.

Antioxidant activity of MSKO

The radical scavenging activity is expressed as median inhibitory concentrations (IC50) value which is inversely proportional to the antioxidant activity [38, 39]. Antioxidants are major ingredients that protect the quality of oils by retarding oxidation [40]. The mechanism of action of flavonoids are through scavenging or chelating process [41]. Study on the antioxidant activity in MSKO, it was obtained IC50 value of hexane and ethanol extract of 30.5 µg/mL and 32 µg/mL, respectively, while BHT was used as a comparison had IC50 values of 34.5 µg/mL, higher than the IC50 value of hexane and ethanol extract. The smaller the value of IC50 indicates that the compound has a large antioxidant activity. MSKO could be categorized as oil with considerable antioxidant potential. The DPPH radical scavenging activity of hexane extract was higher than that of ethanol extract, because hexane more extracting other antioxidant components soluble in oil, such as β-carotene and vitamin E (tocopherols and tocotrienols). Studies reported that the IC50 value of selected vegetable oil extracts, such as soy, sunflower, rapeseed and corn ranged from 29.7 to 34.0 µg/mL [42].

Synthetic antioxidants are use at legal limits to reduce deterioration, rancidity and oxidative discoloration. BHA and BHT are volatile and decompose easily at high temperatures [43]. There are some serious problems concerning the safety and toxicity of such synthetic antioxidants related to their metabolism and possible absorption and accumulation in body organ and tissues. Therefore, the search for preparation of useful natural antioxidants is highly desirable. Many studies showed that natural antioxidants,  as ﬂavonoids and other phenolic phytochemicals, present in plants are associated with reduced chronic  disease  risk [44]. The ability of phenolic compounds to quench free radicals arises because of both their ability to donate protons and to transfer electrons while remaining relatively stable [45].

The extract of MSK exhibited contained the highest degree of free-radical scavenging and tyrosinase-inhibition activities compared with methyl gallate and phenolic compounds [11]. Many studies showed that natural antioxidants, as ﬂavonoids and other phenolic phytochemicals, contained in plants were associated with reduced chronic disease risk [46]. The type of phenolic compounds has been demonstrated to inhibit lipid peroxidation of human LDL in vitro [47]. Some studies indicated that MSKO contained diﬀerent phenolic compounds and consisted of stable fat-rich with SFA, so it could be good source of natural antioxidants [48]. MSKO can be used as natural antioxidants in a variety of foods, due to the content of some phenolic compounds, FA component that was rich with the SFA and MUFA oleic acid besides tocopherols, squalene and diﬀerent sterol fractions The antioxidant property of MSKO is attributed to some extent to its richness in SFA and MUFA as well as fractions of tocopherols, sterols, their esters and other phenolic compounds are present as unsaponifiable matter at MSKO [49].

MSKO can be used as antioxidant in edible oil. Oxidative stability of buffalo fat with the addition of 5% MSK similar to sunflower seed oil stability with added 300 ppm TBHQ during 12 months of storage in darkness, peroxide and anisidine values are at the lowest concentrations [50]. Oxidative stability of refined sunflower seed oil after adding 1% crude MSKO and placing it at 90°C incubator for 36 hours was equal to the antioxidant capacity of added 200 ppm BHT [49]. Comparison of the peroxide value between fat with containing different concentrations of MSKO showed that the addition of crude MSKO could be reduced the oxidation process and as the concentration of the MSKO increased the oxidation chain reaction and consequently peroxide formation was reduced. Gallic acid present in MSKO has been known as an anti-inflammatory, anti-mutant and anti-oxidative agent [51].

- CONCLUSION

Based on the results of physicochemical properties and fatty acid profile of the MSKO, it could be concluded that the MSKO could be become valuable resource to produce high value of vegetable oil. The MSKO extracted with hexane has better quality. The MSKO was analyzed for refractive index, acid value, saponification value, and iodine value. Most of the values comply with the standard specifications. The oil was good quality and could be recommended suitable for the food, pharmaceutical and cosmetics industries. MSKO is rich in oleic and stearic acid indicating that oil was stable and tolerant to rancidity. This oil may be considered as an important source of UFA and has the potential to be used as nutrient rich food oil.  Furthermore, the results also verified that MSKO have IC50 value that lower than BHT as reference, enabling their application as ingredient of functional or enriched food. The results of present study provide useful information for edible oil and food industry, due to its special composition, rich in PUFA as oleic acids, and in antioxidant compounds.

- ACKNOWLEDGEMENTS

## Thanks to the Directorate of Higher Education, Ministry of Research and Technology and Higher Education of Indonesia for a Doctoral Grant that supports the funding of this study with No. Contract of 052/SP2H/LT/DRPM/IV/2017, and we do appreciate and thanks to the Department of Chemical Engineering, State Polytechnic of Ujung Pandang for supporting the laboratory works by providing GC-MS instrument and other analysis equipment.

##### REFERENCES

[1] 	Akinyemi SOS, PE Akin-Idowu, OO Oduntan, KO Egbekunle, "Chemical Composition of the Seed Kernel Flour of Some Mango (Mangifera indica L.) Varieties".  J. BIOL. CHEM. RESEARCH, Vol.32, No.1, pp. 160-173, 2015.

[2] 	Dhingra S, Kapoor AC, "Nutritive value of mango seed kernel". JOURNAL OF THE SCIENCE OF FOOD AND AGRICULTURE, Vol.36, No.8, pp. 752-756, 1985. Doi: 10.1002/jsfa.2740360817.

[3]	Prabhakar JV, Hemavathy J., "Composition of Polar Lipids of AIphonso (Mangifera Indica) Kernel Mango". JOURNAL OF FOOD SCIENCE, Vol.52, No.3, pp. 833-834. 1987. Doi: 10.1111/j.1365-2621.1987.tb06742.x.

[4]	Nzikou JM, Kimbonguila A, Matos L, Loumouamou B, Pambou-Tobi NPG, Ndangui CB, Abena AA, Silou T, Scher J, Desobry S. "Extraction and characteristics of seed kernel oil from mango (Mangiferaindica)". RJ ENVIRON EARTH SCI, Vol.2, pp. 31-35, 2010.

[5]	Arogba SS., "Physical, chemical and functional properties of Nigerian mango (Mangifera indica) kernel and its processed flour". J SCI FOOD AGRIC, Vol.73, No.3, pp. 321-328, 1997. Doi: 10.1002/(SICI)1097-0010(199703)73:3<321::AID-JSFA722>3.0.CO;2-4.

[6]	Yamoneka J, Malumba P, Blecker C, Gindo M, Richard G, Fauconnier M, Lognay G, Danthine S.  "Physicochemical properties and thermal behaviour of African wild mango (Irvingia gabonensis) seed fat". LWT-FOOD SCIENCE AND TECHNOLOGY, Vol. 64, No.2, pp. 989-996. 2015. Doi: 10.1016/j.lwt.2015.06.035.

[7]	Sani I. "Soxhlet extraction and physicochemical characterization of Mangifera indica L. Seed kernel oil". Research and Reviews: J FOOD DAIRY TECH, Vol. 2, No.1, pp. 20-24, 2014.

[8]	Mahale SM, Goswami-Giri AS. "Composition and characterization of reﬁned oil compared with its crude oil from waste obtained from Mangifera indica". ASIAN J RES CHEM, No.4, pp.1415-1419, 2011.

[9]	Rukmini C, Vijayaraghavan M. "Nutritional and toxicological evaluation of mango kernel oil". J OF THE AMERICAN OIL CHEMISTS' SOCIETY, Vol.61, No.4, pp.789-792, 1984. Doi: 10.1007/BF02672138.

[10]	Jafari JM, Gharachorloo AH, Hemmaci. "The Stabilizing Effect of Three Varieties of Crude Mango Seed Kernel Oil on Tallow". J OF FOOD BIOSCIENCES AND TECHNOLOGY, Vol.4, No.1, pp. 31-36, 2014.

[11]	Kittiphoom S, Sutasinee S. "Mango seed kernel oil and its physicochemical properties". INTERNATIONAL FOOD RESEARCH JOURNAL, Vol.20, No.3, pp. 1145-1149, 2013.

[12]	Solis-Fuentes JA, Duran-de-Bazua M, Del C. "Mango (Mangifera indica L.) Seed and Its Fats". NUTS AND SEEDS IN HEALTH AND DISEASE PREVENTION, PP. 741-748, 2011. Doi: 10.1016/B978-0-12-375688-6.10088-X.

[13]	Rukmini C. "Toxicity studies on unconventional oils". PROC. NUTR. SOC. INDIA. Vol.33, pp. 49-55, 1987.

[14]	Rashwan MRA. "Fatty acids composition, neutral lipids and phospholipids fractionation in the kernel lipids of the mango varieties". ASSIST JOURNAL OF AGRICULTURAL SCIENCE, Vol.21, No.1, pp.105-117, 1990.

[15]	Peeres VF, J Saffi, MIS Melecchi, FC Abad, RA Jacques, MM Martinez, EC Oliveira, EB Caramaao. "Comparison of soxhlet, ultrasound-assisted and pressurized liquid extraction of terpenes, fatty acids and Vitamin E from Piper Gaudichaudianum Kunth". JOURNAL OF CHROMATOGRAPHY A, Vol. 1105, pp. 115-118, 2006. Doi:10.1016/j.chroma.2005.07.113

[16]	Mas'ud, F., Meta M, Amran L, Zainal Z. "Optimization of Mango Seed Kernel Oil Extraction using Response Surface Methodology". OILSEEDS & FATS CROPS AND LIPIDS. 2017. Doi: 10.1051/ocl/2017041.

[17]	Metcalfe. "The rapid preparation of fatty acid esters for gas chromatographic analysis". ANALYTICAL CHEMISTRY, Vol.33, No.3, pp. 363-364. 1961. Doi: 10.1021/ac60171a016.

[18]	AOCS. "Official methods and recommended Practices (15th ed.)". Champaign, NewYork: American Oil Chemist’s Society. 1989.

[19]	Hatano T, Kagawa H, Yasuhara T, Okuda T. "Two new flavonoids and other constituents in licorice root. Their relative astringency and radical scavenging effects". CHEMICAL & PHARMACEUTICAL BULLETIN, Vol.36, No.6, pp. 2090-2097. 1988. Doi: 10.1248/cpb.36.2090.

[20]	Olajumoke OE. "Extraction and Characterization of Vegetable Oil from Mango". IOSR JOURNAL OF APPLIED CHEMISTRY, Vo.5, No.3, pp. 6-8. 2013.

[21]	Abdalla EM, Darwish SM, Ayad EHE, EL-Hamahmy RM. "Egyptian mango by-product 1". Compositional quality of mango seed kernel. FOOD CHEM, Vol.103, pp.1134-1140, 2006. Doi: 10.1016/j.foodchem.2006.10.017.

[22]	Rojas-Grau MA, Raybaudi-Massilia RM, Soliva-Fortuny RC, Avena-Bustillos RJ, McHugh TH, and Martín-Belloso O. "Apple puree-alginate edible coating as carrier of antimicrobial agents to prolong shelf-life of fresh-cut apples". POSTHARVEST BIOLOGY AND TECHNOLOGY, Vol.45, No.2, pp. 254-264. 2007. Doi: 10.1016/j.postharvbio.2007.01.017

[23]	Ekundayo CA, Idzi E. "Mycoflora and nutritional value of shelled melon seeds (Citrulus vulgaris Schrad.) in Nigeria". PLANT FOODS FOR HUMAN NUTRITION, Vol.40, pp. 15-222. 1990.

[24]	Fauzi SHM, Rashid NA, Omar Z. "Effects of Enzymatic Interesterification on the Physicochemical, Polymorphism and Textural Properties of Palm Stearin, Palm Kernel Oil and Soybean Oil Blends". INTERNATIONAL JOURNAL OF BIOSCIENCE, BIOCHEMISTRY AND BIOINFORMATICS, Vol. 3, No.4, pp. 398-403. 2013. Doi: 10.7763/IJBBB.2013.V3.242.

[25]	Jahurul MHA, Zaidul ISM, Ghafoor K, Al-juhaimi FY, Nyam K, Norulaini NAN, Sahena F Omar AKM. "Mango (Mangifera indica L.) by-products and their valuable components : A review". FOOD CHEM, 183: 173-180. 2015. Doi: 10.1016/j.foodchem.2015.03.046.

[26]	Joseph JK. "Short Communication, Physico-chemical Attributes of Wild Mango (Irvingia gabonensis) Seeds". BIORESOURCE TECHNOLOGY, Vol.53, pp. 179-181, 1995.

[27]	Muchiri DR, Mahungu SM, Gituanja SN. "Studies on mango (Mangifera indica, L.) kernel fat of some Kenyan varieties in Meru". JAOCS, JOURNAL OF THE AMERICAN OIL CHEMISTS’ SOCIETY, Vol.89, No.9, pp. 1567-1575. 2012. Doi: 10.1007/s11746-012-2054-6.

[28]	Ishida H, Suzuno H, Sugiyama N, Innami S, Tadokoro T, Maekawa A. "Nutritive evaluation on chemical components of leaves, stalks and stems of sweet potatoes (Ipomoea batatas poir)". FOOD CHEM, Vol,68, No.3, pp. 359-367. 2000. Doi: 10.1016/S0308-8146(99)00206-X.

[29]	Xu YX, Hanna MA, Josiah SJ. "Hybrid hazelnut oil characteristics and its potential oleochemical application". INDUSTRIAL CROPS AND PRODUCTS, Vol.26, No.1, pp. 69-76. 2007. Doi: 10.1016/j.indcrop.2007.01.009.

[30]	Sonwai S, Kaphueakngam P, Flood A. "Blending of mango kernel fat and palm oil mid-fraction to obtain cocoa butter equivalent". JOURNAL OF FOOD SCIENCE AND TECHNOLOGY, Vol.51, No.10, pp. 2357-2369. 2012. Doi: 10.1007/s13197-012-0808-7.

[31]	Kittiphoom S. "Utilization of mango seed". INTERNATIONAL FOOD RESEARCH JOURNAL, Vol.19, No.4, pp. 1325-1335. 2012.

[32]	Fahimdanesh M, Bahrami ME. "Evaluation of Physicochemical Properties of Iranian Mango Seed Kernel Oil". INTERNATIONAL CONFERENCE ON NUTRITION AND FOOD SCIENCES, pp. 44-49. 2013. Doi: 10.7763/IPCBEE.

[33]	Jeyarani T, Subramanian S, Sneha R, Sudha ML, Negi PS. "Characterisation of Mango Kernel Fat and Preparation of Tran’s-Free Margarine for Use in Muffins". J NUTR FOOD SCI, Vol.5, pp.357. 2015. Doi: 10.4172/2155-9600.1000357.

[34]	Abdulkarim SM, Long K, Lai OM, Muhammada SKS, Ghazali HM. "Frying quality and stability of high-oleic Moringa oleifera seed oil in comparison with other vegetable oils". FOOD CHEM, Vol.105, No.4, pp. 1382-1389. 2007.

[35]	Chabba S, Netravali AN. "Green, composites using modified soy protein concentrate resin and flax fabrics and yarns". JSME INTERNATIONAL JOURNAL, SERIES A: SOLID MECHANICS AND MATERIAL ENGINEERING, Vol.47, No.4, pp. 556-560. 2004. Doi: 10.1299/jsmea.47.556.

[36]	Langer K, Balthasar S, Vogel V, Dinauer N, von Briesen H, Schubert D, de Duve C, Seglen PO, Grinde B, Solheim AE, Bhattacharya AA, Grune T, Curry S. "Crystallographic analysis reveals common modes of binding of medium and long-chain fatty acids to human serum albumin". EUROPEAN JOURNAL OF BIOCHEMISTRY/FEBS, Vol. 257, No.3, pp. 391-397. 2003. Doi: 10.1006/jmbi.2000.4158.

[37]	Vila JA, Ripoll DR, Villegas, ME, Vorobjev YN, Scheraga HA. "Role of hydrophobicity and solvent-mediated charge-charge interactions in stabilizing alpha-helices". BIOPHYSICAL JOURNAL, Vol.75, No.6, pp. 2637-46. 1998. Doi: 10.1016/S0006-3495(98)77709-4.

[38]	Thaipong K, Boonprakob U, Crosby K, Cisneros-Zevallos L, Hawkins BD. "Comparison of ABTS, DPPH, FRAP, and ORAC assays for estimating antioxidant activity from guava fruit extracts". JOURNAL OF FOOD COMPOSITION AND ANALYSIS, Vol.19, No.6-7, pp. 669-675. 2006. Doi: 10.1016/j.jfca.2006.01.003.

[39]	Kamiloglu O, Ercisli S, Sengul M, Toplu C, Serce S. "Total phenolics and antioxidant activity of jujube (Zizyphus jujube Mill.) genotypes selected from Turkey". AFRICAN JOURNAL OF BIOTECHNOLOGY, Vol.8, No.2, pp. 303-307. 2009.

[40]	Wanasundara U, Amarowicz R, Shahidi’itj F. "Isolation and Identification of an Antioxidative Component in Canola Meal". J. AGRIC. FOOD CHEM, Vol.42, pp. 1285-1290. 1994. Doi: 10.1021/jf00042a006.

[41]	Pourmorad F, Hosseinimehr SJ, Shahabimajd N. "Antioxidant activity, phenol and flavonoid contents of some selected Iranian medicinal plants". AFRICAN JOURNAL OF BIOTECHNOLOGY, No.5, pp. 1142-1145. 2006. Doi: 10.1055/s-2007-987042.

[42]	Siger A, Nogala-Kalucka M, Lampart-Szczapa E. "The content and antioxidant activity of phenolic compounds in cold-pressed plant oils". JOURNAL OF FOOD LIPIDS, No.15, pp. 137-149. 2008. Doi: 10.1111/j.1745-4522.2007.00107.x

[43]	Branen AL. "Toxicology and biochemistry of butylated hydroxyanisole and butylated hydroxytoluene". JOURNAL OF THE AMERICAN OIL CHEMISTS’ SOCIETY, Vol.52, No.2, pp. 59-63. 1975. Doi: 10.1007/BF02901825.

[44]	Cieslik E, Grȩda A, Adamus W. "Contents of polyphenols in fruit and vegetables". FOOD CHEM, Vol.94, No.1, pp. 135-142. 2006. Doi: 10.1016/j.foodchem.2004.11.015.

[45]	Pietta PG. "Flavonoids as antioxidants". JOURNAL OF NATURAL PRODUCTS, Vol. 63, No.7, pp. 1035-1042. 2000. Doi: 10.1021/np9904509.

[46]	Singh G, Marimuthu P, De Heluani CS. Catalan CAN. "Antioxidant and biocidal activities of Carum nigrum (Seed) essential oil, oleoresin, and their selected components". JOURNAL OF AGRICULTURAL AND FOOD CHEMISTRY, Vol.54, No.1, pp. 174-181. 2006. Doi: 10.1021/jf0518610.

[47]	Goncalves B, Landbo AK, Let M, Silva AP, Rosa E, Meyer AS. "Storage affects the phenolic profiles and antioxidant activities of cherries (Prunus avium L) on human low-density lipoproteins". JOURNAL OF THE SCIENCE OF FOOD AND AGRICULTURE, Vol.84, No.9, pp. 1013-1020. 2004. Doi: 10.1002/jsfa.1752.

[48]	Puravankara D, Boghra V, Sharma RS. "Effect of antioxidant principles isolated from mango (Mangifera indica L) seed kernels on oxidative stability of buffalo ghee (butter-fat)". JOURNAL OF THE SCIENCE OF FOOD AND AGRICULTURE, Vol.80, No.4, pp. 522-526. 2000. Doi:10.1002/(SICI)1097-0010(200003)80:4<522::AID-JSFA560>3.0.CO;2-R.

[49]	Toscano G, Maldini E. "Analysis of the Physical and Chemical Characteristics of Vegetable Oils As Fuel". JOURNAL OF AG. ENG., No.3, pp. 39-47. 2007.

[50]	Youssef AMM.  "Utilization of  the seed of mango processing wastes as a secondary source of oil and protein". ALEXANDRIA JOURNAL OF AGRICULTURAL RESEARCH, Vol.44, No.3, pp. 149-166. 1999.

[51]	Soong YY, Barlow PJ. "Quantification of gallic acid and ellagic acid from longan (Dimocarpus longan Lour.) seed and mango (Mangifera indica L.) kernel and their effects on antioxidant activity". FOOD CHEM, Vol. 97, No.3, pp. 524-530. 2006. Doi: 10.1016/j.foodchem.2005.05.033.49

CITE AN ARTICLE

It will get done by IJESRT Team