SOP : Analysis of the Derivatives of Sialic Acid in Pig Tissues

- Introdction：Sialic acid is a generic term for the N- or O-substituted derivatives of neuraminic acid, a monosaccharide with a nine-carbon backbone. N-glycolylneuraminic acid (Neu5Gc or NGNA) is not synthesized in the human body, so most of the sialic acid in the body is N-acetylneuraminic acid (Neu5Ac or NANA), while other animals have two kinds of sialic acid (NANA and NGNA). There may be NGNA in biotech products manufactured from nonhuman animal source, and a detectable method is needed to determine the derivatives of sialic acid in the product.

- Purpose： The detection method of sialic acid by HPLC Analysis, and further distinguish the NGNA and NANA of sialic acid in biotechnology products.

- Material/reagent and device：

  - Devices：

    - HPLC system, Thermo Scientific™ (Waters 2475 Multi-wavelength Fluorescence Detector, Waters 717 plus Autosampler and Waters 600 Controller)

    - Reversed phase C18 column (DiscoveryR BIO wide Pore C18, 5m, 4.6 x 25 cm)

    - Dry Bath Incubator (Violet BioScience Inc. Model VH01)

    - Centrifuge (Sigma 3K30)

    - 0.22m membrane (Millipore GVWP04700)

  - Reagents

    - DMB (1, 2-diamino 4, 5-methylenedioxybenzene) reagent

- Prepare reagent buffer : Add 99.8% of Acetic acid 800L , 540L 2-ME and 31 mg Sodium hydrosulfite dissolved in 7 mL deionized water and add water to the final volume of 10 mL.  (1.4 M acetic acid, 0.75M 2-mercaptoethanol and 18 mM Sodium hydrosulfite solution)

- Add 1.6 mg DMB ( purchase from Sigma-Aldrich, Inc.) into 1 mL of reagent buffer.

  - HPLC elution Buffer (MeOH / Acetonitrile /H2O=7:9:84) :

Added 100% methanol 70 mL and 90 mL of Acetonitrile into 1 L volumetric flask then add deionized water to final volume of 1000 mL.  The solution is filtered through 0.22m membrane.

- Procedure

  - Sample preparation

    - About 100 mg samples, including ear or tail tissues and small intestines, in MQ water are cut into small pieces by scissors.

    - The mixture is incubated at 95 oC for 30 min.

    - After cooling to room temperature, 0.5 M H2SO4 was added to the final concentration of 25 mM.

    - The mixtures were incubated at 80 oC for 1 h to release the sialic acids from the samples.

    - After centrifugation at 10,000 x g for 10 min, the supernatant was collected.

    - Prepare the standard: 1 mg of NANA and NGNA is dissolve in 1 mL of deionized water, respectively.

    - 200 L of DMB reagent are added to	200 L of samples and standard, respectively, the mixture are incubated at 80 oC for 2 h to label fluorescent agent.

    - After labelling, the mixtures are filtrated through 0.22m membrane.

  - HPLC analysis

    - The DMB-labeled sample was injected onto Waters™ HPLC system (Waters 2475 Multi-wavelength Fluorescence Detector, Waters 717 plus Autosampler and Waters 600 Controller) with the DiscoveryⓇ BIO wide Pore C18 (5 m, 4.6 x 25 cm) column.

    - The analysis was performed by an isocratic mobile phase of Methanol: Acetonitrile: H2O ratio are 7: 9: 84 with a flow rate of 0.6 mL/min and the fluorescence detector was set at excitation wavelength of 373 nm and emission wavelength of 448 nm.

    - Usually, the samples are diluted 100 folds and the standards are diluted 5000-10000 folds with deionized water for HPLC analysis.

- References

  - Toshiyuki H., Keiko T.A., Naoko I. T., Hidenao T.,Atsushi S., Toshihiko T., Toshio I., Toshiaki A.,Yukishige Y. and Ineo I. (2000). Determination of N-acetyl- and N-glycolylneuraminic acids in gangliosides by combination of neuraminidase hydrolysis and fluorometric high-performance liquid chromatography Using a GM3 Derivative as an Internal Standard. Analytical Biochemistry, 281, 193-201.

  - Veronique S., Julien M., Sean A. (2010). Determination of sialic acids in milks and milk-based products. Analytical Biochemistry, 405, 28–40.