Artificial Cerebrospinal Fluid I (ACSF.I)

- Scope and Applicability: Artificial Cerebrospinal Fluid I (ACSF.I) is used for applications including transcardial perfusion prior to fresh mouse brain slicing and as a bathing solution during fresh mouse brain slicing.

- Materials:

  - Calcium Chloride, dihydrate (CaCl2·2 H2O) (Sigma C3306, C3881 or equivalent)

  - D-Glucose (C6H12O6; aka Dextrose) (Sigma G7021, G8270 or equivalent)

  - HEPES (C8H18N2O4S) (Sigma H3375 or equivalent)

  - Magnesium Sulfate, Heptahydrate (MgSO4·7 H2O) (Sigma M1880 or equivalent)

  - Sodium Phosphate monobasic monohydrate (NaH2PO4 • H2O) (Sigma S9638 or equivalent)

  - Potassium Chloride (KCl) (Sigma P9333, Sigma P5405, P4504 or equivalent)

  - Sodium Pyruvate (CH3COCOONa) (JT Baker 3354 purchased as VWR JT3354 or equivalent)

  - Sodium L-Ascorbate (C6H7NaO6 ) (Sigma A7631, A4034 or equivalent)

  - Sodium Bicarbonate (NaHCO3) (Sigma S5761, S8875 or equivalent)

  - Thiourea (NH2CSNH2) (Sigma T7875 or equivalent)

  - N-Methyl-D-glucamine (C7H17NO5) (Fluka 66930 purchased as Sigma M2004 or equivalent)

  - 5N HCl (VWR BDH7419-1 or equivalent)

  - Concentrated (36-38%) HCl (VWR JT9535-1 or equivalent)

  - Myo inositol (C6H12O6) (Sigma I7508, I5125 or equivalent)

  - N-Acetyl-L-cysteine (HSCH2CH(NHCOCH3)COOH) (Sigma A7250 or equivalent)

  - Taurine (NH2CH2CH2SO3H) (Sigma 86329, T0625 or equivalent)

  - MilliQ Water

  - 1.7 ml microcentrifuge tubes (Genesee 24-282 or equivalent)

  - 1 Liter polystyrene bottle (VWR 28199-762 or equivalent)

  - 500 ml polystyrene bottle (VWR 28199-760 or equivalent)

  - 250 ml polystyrene bottle (VWR 28199-758 or equivalent)

  - 1000 ml graduated cylinder

  - 100 ml graduated cylinder

  - Pipettor

  - 50 ml serological pipette (VWR 53106-441 or equivalent)

  - 25 ml serological pipette (VWR 53300-567 or equivalent)

  - 10 ml Serological pipette (VWR 53300-523 or equivalent)

  - P200 pipetteman

  - P200 pipette tips (Rainin GPL200F or equivalent)

  - 1 liter filter system, 0.45 µm (Corning 430516 purchased as VWR 28199-814 or equivalent)

  - Weigh paper

  - Weigh boats

  - Disposable plastic spatulas

  - Stir bar

- Equipment:

  - Vacuum pump (Gast DOA-P104-AA or equivalent) with Tygon tubing

  - Osmometer (Fiske 210 or equivalent)

  - pH Meter

  - Stir plate

  - Magnetic stir bar remover

  - Calibrated balance

- Safety:

  - Gloves

  - Eye protection

  - Lab coat

  - Fume hood

- Output:

  - 500 ml or 1 liter of Artificial Cerebrospinal Fluid I (ACSF.I) pH 7.3, Osmolarity 300-310 mOsm containing:

    - 0.5 mM Calcium Chloride, dihydrate

    - 25 mM D-Glucose

    - 20 mM HEPES

    - 10 mM Magnesium Sulfate, Heptahydrate

    - 1.25 mM Sodium Phosphate monobasic monohydrate

    - 2.5 mM Potassium Chloride

    - 3 mM Sodium Pyruvate

    - 5 mM Sodium L-Ascorbate

    - 25 mM Sodium Bicarbonate

    - 2 mM Thiourea

    - 96 mM HCl

    - 96 mM N-Methyl-D-glucamine

    - 3 mM Myo inositol

    - 12 mM N-Acetyl-L-cysteine

    - 0.01 mM Taurine

- Reference Documents:

  - N/A

Warning: Personal Protective Equipment (PPE) should be used at all times while operating this protocol. If you are unsure what PPE you should be using, see your immediate supervisor.

- Setup:

  - Prepare working stocks of reagents, as necessary. Label the stock bottles with contents, preparer’s initials, preparation date, storage requirement and expiration date.

    - 200 mM Calcium Chloride, dihydrate: Prepare a 200 mM stock solution by dissolving 29.4 g of CaCl2·2 H2O in 1 Liter of MilliQ Water in a labeled 1 Liter polystyrene bottle. Store the stock solution at room temperature for up to 60 days.

    - 1 M HEPES: Prepare a 1M stock solution by dissolving 238.3 g of HEPES in 1 Liter of MilliQ Water in a labeled 1 Liter polystyrene bottle. Store the stock solution at room temperature for up to 60 days.

    - 1 M Magnesium Sulfate, heptahydrate: Prepare a 1 M stock solution by dissolving 246 g of MgSO4.7 H2O 1 M in 1 Liter of MilliQ Water in a labeled 1 Liter polystyrene bottle. Store the stock solution at room temperature for up to 60 days.

    - 250 mM Potassium Chloride, 125 mM Sodium Phosphate monobasic monohydrate, 100 mM Magnesium Sulfate Heptahydrate solution.

      - Prepare a 250 mM stock solution of Potassium Chloride by dissolving 18.6 g of KCl in 1 Liter of MilliQ Water in a labeled 1 Liter polystyrene bottle.

      - Add 17.3 g of NaH2PO4 to the Potassium Chloride solution.

      - Add 24.6 g of MgSO4.7 H2O to the Potassium Chloride / Sodium Phosphate Solution.

      - Store the stock solution at room temperature for up to 60 days.

    - Sodium Pyruvate: Prepare a 500 mM stock solution by dissolving 55 g of CH3COCOONa in 1 Liter of MilliQ Water in a labeled 1 Liter polystyrene bottle. Store the stock solution at 4°C for up to 60 days.

    - Taurine: Prepare a 100 mM stock solution by dissolving 0.625 g of Taurine in 50 ml of MilliQ Water in a labeled 50 ml conical tube. Store the stock solution at 4°C for up to 60 days.

    - Thiourea: Prepare a 1.6 M stock solution by dissolving 6.08 g of Thiourea in 50 ml of MilliQ Water. Aliquot 1.4 ml the solution into each of 35 labeled 1.7 ml microcentrifuge tubes. Store the aliquots at -20°C for up to 60 days. The solution must be warmed to room temperature prior to use.

  - Label a sterile 1 liter polystyrene bottle with the reagent name, date and the preparer’s initials.

  - Obtain a stir bar and place it in the container.

  - Set up the stir plate in the fume hood.

- Methodology/Procedures (record all lot numbers and volumes used in the appropriate reagent preparation notebook):

  - Consult the reagent request form to determine the volume of ACSF.I to prepare. Table 1 lists the reagent quantities required to make either 500 ml or 1 Liter of ACSF.I.

  - To make 1000 ml of Artificial Cerebrospinal Fluid.I (ACSF.I):

    - Using a graduated cylinder, measure 500 ml of MilliQ Water. Add the water to an appropriately labeled container containing a stir bar.

    - Place the container on a stir plate and begin stirring at a high setting (7 or higher).

    - Using a sterile serological pipette or graduated cylinder, add 20 ml of the 1 M HEPES stock solution.

    - Using a sterile serological pipette, add 10 ml of the 250 mM Potassium Chloride, 125 mM Sodium Phosphate monobasic monohydrate, 100 mM Magnesium Sulfate Heptahydrate solution.

    - Using a sterile serological pipette, add 6 ml of the 500 mM Sodium Pyruvate stock solution.

    - Using a P200 Pipetteman and a P200 pipette tip, add 100 µl of the 100 mM Taurine stock solution.

    - Using a sterile serological pipette, add 1.25 ml of the 1.6 M Thiourea stock solution. Note: the solution must be at room temperature to fully dissolve.

    - Using a calibrated balance, weigh out 4.51 g of D-Glucose. Slowly add weighed D-Glucose to the container while stirring.

    - Weigh out 540 mg of Myo inositol. Slowly add weighed Myo inositol to the container while stirring.

    - Weigh out 1.96 g of N-Acetyl-L-cysteine. Slowly add weighed N-Acetyl-L-cysteine to the container while stirring.

    - Weigh out 19.4 g of N-Methyl-d-glucamide. Slowly add weighed N-Methyl-d-glucamide to the container while stirring.

    - Weigh out 1.0 g of Sodium L-Ascorbate. Slowly add weighed Sodium L-Ascorbate to the container while stirring.

    - Weigh out 2.1 g of Sodium Bicarbonate. Slowly add weighed Sodium Bicarbonate to the container while stirring.

Reagent; 500 ml; 1000 ml. HEPES; 1 M; 10 ml; 20 ml. Hydrochloric Acid; 5N; 950 µl; 1.9 ml. Stock solution of: 250 mM Potassium Chloride 125 mM Sodium Phosphate monobasic monohydrate 100 mM Magnesium Sulfate Heptahydrate; 5 ml; 10 ml. Sodium Pyruvate; 500 mM; 3 ml; 6 ml. Taurine; 100 mM; 50 µl; 100 µl. Thiourea; 1.6 M; 675 µl; 1.25 ml. D-Glucose; 2.225 g; 4.51 g. Myo inositol; 270 mg; 540 mg. N-Acetyl-L-cysteine; 980 mg; 1.96 g. N-Methyl-D-glucamide; 9.7 g; 19.4 g. Sodium L-Ascorbate; 500 mg; 1.0 g. Sodium Bicarbonate; 1.05 g; 2.1 g. Calcium Chloride; 200 mM (add after adjusting pH); 1.25 ml; 2.5 ml. Magnesium Sulfate; 1 M (add after adjusting pH); 4.5 ml; 9 ml. Bring up to this final volume with MilliQ water (after adjusting pH); 500 ml; 1000 ml.

Table 1: Reagents Required for Preparation of 500 ml or 1 Liter of ACSF.I

    - Continue stirring at a high speed (setting 7 or higher) until all of the chemicals are fully in solution.

    - Using a calibrated pH meter, measure the pH of the ACSF solution. Adjust the pH to 7.3 by adding 5 N HCl dropwise. Up to 16 ml of HCl may be needed to reach the desired pH.

    - Using a sterile serological pipette, add 2.5 ml of the 200 mM Calcium Chloride stock solution.

    - Using a sterile serological pipette, add 9 ml of the 1 M Magnesium Sulfate stock solution.

    - Remove stir bar using a magnetic stir bar remover.

    - Adjust final volume to 1000 ml with MilliQ Water using the graduation marks on the side of the container (if calibrated) or a graduated cylinder.

  - Measure the osmolarity of a 10 µl sample of the solution using an osmometer. If the osmolarity is not 300-310 mOsm (295-305 mmol/kg), adjust as needed.

    - If the osmolarity is too high, add MilliQ water.

    - If the osmolarity is too low, add a volume (X) of 1 mg/ml NaCl solution determined by the following equation

      - D=desired osmolarity

      - O= observed osmolarity

      - X=volume of 1 mg/ml NaCl solution to add (in ml)

    - Confirm that the correct osmolarity has been reached after addition of NaCl or water.

    - Filter the ACSF through a Corning 1 liter 0.45 µm filter system.

      - Using Tygon tubing, attach the Corning filter system to the Gast vacuum pump. Turn on the pump.

      - Filter 500 ml at a time, stirring the remaining ACSF constantly on a stir plate.

  - Label the bottle with reagent name, date, time and preparer’s initials. ACSF expires one week from date of preparation.

  - Record the date of preparation, lot numbers of all components and osmolarity in the reagent preparation notebook.

- Take Down:

  - Dispose of excess reagent directly into municipal sewer system.

  - Wash the stir bar with MilliQ Water.

  - Rinse all beakers, graduated cylinders, and other equipment used to prepare ACSF solutions with MilliQ Water only (No Detergent). Invert all beakers and graduated cylinders and leave on clean absorbent pad to air dry.

- Technical Information:

  - Artificial Cerebrospinal Fluid (ACSF) must be made fresh the day it is to be used. Store at 4°C until use.

  - Use within one week from the day it is made, or at the discretion of the investigator.

  - The Magnesium Sulfate Heptahydrate is added to the solution twice; first as a 1 M stock and then again as part of the stock solution of Potassium Chloride/ Sodium Phosphate monobasic monohydrate/ Magnesium Sulfate Heptahydrate. The 1 M stock is used to bring the Magnesium Sulfate Heptahydrate concentration up to the final desired level of 10 mM.

  - The osmolarity of the ACSF normally should not need to be adjusted if all of the components were added correctly. If the osmolarity does need to be adjusted, add MilliQ Water to lower or Sodium Chloride to raise the value.

  - Stock Solution Storage Conditions:

| Component | Storage Temperature | Shelf Life |
| --- | --- | --- |
| Calcium Chloride; 200 mM | RT | 60 days |
| HEPES; 1 M | RT | 60 days |
| Magnesium Sulfate; 1 M | RT | 60 days |
| Potassium Chloride; 50 mM | RT | 60 days |
| Stock solution of: 250 mM Potassium Chloride 125 mM Sodium Phosphate monobasic monohydrate 100 mM Magnesium Sulfate Heptahydrate | RT | 60 days |
| Sodium Pyruvate; 500 mM | 4°C | 60 days |
| Taurine; 100 mM | 4°C | 60 days |
| Thiourea; 1.6 M | -20°C | 60 days |

- Appendix

  - Materials required for preparation of 1 Liter of ACSF.I

| Material | Supplier, Part Number | Amount Required for 1 Liter |
| --- | --- | --- |
| 1 Liter polystyrene bottle | VWR 28199-762 | 1 ea |
| 1 liter filter system, 0.45 µm | VWR 28199-814 | 1 ea |
| Calcium Chloride, dihydrate | Sigma C3881, C3306 | 73.5 mg (2.5 ml of 200 mM stock) |
| D-Glucose | Sigma G7021, G8270 | 4.5 g |
| HEPES | Sigma H3375 | 20 ml of 1 M stock(20 ml of 1 M stock |
| Magnesium Sulfate, Heptahydrate | Sigma M1880 | 2.5 g (10 ml of 1 M stock) |
| Sodium Phosphate monobasic monohydrate | Sigma S9638 | 173 mg (50 ml of 25 mM stock) |
| Potassium Chloride | Sigma P5405, P4504 | 186 mg (50 ml of 50 mM stock) |
| Sodium Pyruvate | VWR JT3354 | 330 mg (30 ml of 100 mM stock) |
| Sodium L-Ascorbate | Sigma A7631, A4034 | 990 mg |
| Sodium Bicarbonate | Sigma S5761, S8875 | 2.1 g |
| Thiourea | Sigma T7875 | 1.25 ml |
| N-Methyl-D-glucamine | Fluka 66930 | 18.7 g |
| 5N HCl | VWR BDH7419-1 | 1.9 ml |
| Myo inositol | Sigma I7508, I5125 | 540 mg |
| N-Acetyl-L-cysteine | Sigma A7250 | 1.95 g |
| Taurine | Sigma 86329, T0625 | 100 µl |

  - ACSF Formulation Comparison

| Component | ACSF.I | ACSF.II | ACSF.III | ACSF.IV | ACSF.V | ACSF.VI | ACSF.VII | ACSF.VIII | ACSF.IX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calcium Chloride, dihydrate | 0.5 mM | 2 mM | 2 mM | 2 mM | 1.8 mM | 2 mM | 0.5 mM | 2 mM | 1.3/2.0 mM |
| D-Glucose (Dextrose) | 25 mM | 25 mM | 12.5 mM | 25 mM | 0 | 20 mM | 25 mM | 25 mM | 25 mM |
| DL-AP5 | 0 | 0 | 0 | 0 | 0 | 50 µM | 0 | 0 | 0 |
| DNQX | 0 | 0 | 0 | 0 | 0 | 20 µM | 0 | 0 | 0 |
| HCl | 98 mM | 0 | 0 | 0 | 0 | 0 | 92 mM | 0 | 0 |
| HEPES | 20 mM | 16 mM | 0 | 20 mM | 5 mM | 0 | 20 mM | 20 mM | 0 |
| Kynurenic Acid | 0 | 0 | 1 mM | 0 | 0 | 0 | 0 | 0 | 0 |
| Magnesium Chloride | 0 | 0 | 0 | 0 | 1.0 mM | 2 mM | 0 | 0 | 0 |
| Magnesium Sulfate | 10 mM | 2 mM | 1 mM | 2 mM | 0 | 0 | 10 mM | 2 mM | 1.0/2.0 mM |
| Monosodium Phosphate | 1.25 mM | 1.25 mM | 1.25 mM | 1.25 mM | 0 | 1.25 mM | 1.2 mM | 1.2 mM | 1.2 mM |
| Myo inositol | 3 mM | 3 mM | 0 | 3 mM | 0 | 0 | 0 | 0 | 0 |
| N-acetylcysteine | 12 mM | 12 mM | 0 | 12.3 mM | 0 | 0 | 0 | 0 | 0 |
| N-methyl-d-glucamine | 96 mM | 0 | 0 | 0 | 0 | 0 | 92 mM | 0 | 0 |
| Picrotoxin | 0 | 0 | 0.1 mM | 0 | 0 | 0 | 0 | 0 | 0 |
| Potassium Chloride | 2.5 mM | 2.5 mM | 2.5 mM | 2.5 mM | 5.4 mM | 3 mM | 2.5 mM | 2.5 mM | 3mM |
| Sodium Bicarbonate | 25 mM | 25 mM | 26 mM | 25 mM | 0 | 20 mM | 30 mM | 30 mM | 18 mM |
| Sodium Chloride | 0 | 97 mM | 126 mM | 94 mM | 135 mM | 126 mM | 0 | 92 mM | 126 mM |
| Sodium L-Ascorbate | 5 mM | 5 mM | 0 | 5 mM | 0 | 0 | 5 mM | 5 mM | 0.16 mM |
| Sodium Pyruvate | 3 mM | 3 mM | 0 | 3 mM | 0 | 0 | 3 mM | 3 mM | 0 |
| Taurine | 0.01 mM | 0.01 mM | 0 | 0.01 mM | 0 | 0 | 0 | 0 | 0 |
| Thiourea | 2 mM | 2 mM | 0 | 2 mM | 0 | 0 | 2 mM | 2 mM | 0 |
| Tetrodotoxin (TTX) | 0 | 0 | 0 | 0 | 0 | 100 nM | 0 | 0 | 0 |