Artificial Cerebrospinal Fluid V (ACSF.V)

- Scope and Applicability: Artificial Cerebrospinal Fluid is used during surgical procedures to maintain tissue hydration and minimize tissue damage, adhesion formation and infection.

- Materials:

  - Sodium Chloride (NaCl; VWR JT4058 or equivalent)

  - Potassium Chloride (KCl; Sigma P9333 or equivalent)

  - Magnesium Chloride (MgCl2; VWR 80503-016 or equivalent)

  - Calcium Chloride dihydrate (CaCl2 • 2H2O; Sigma C3306 or equivalent)

  - HEPES (Sigma H3375 or equivalent)

  - Potassium Hydroxide (1N KOH in water; Acros Organics AC27840-0010 or equivalent)

  - MilliQ Water (RNase-free)

  - Labeled 1 Liter plastic bottle (or equivalent as directed by project lead's request)

  - 1000 ml graduated cylinder

  - Pipettor

  - 25 ml serological pipette

  - 1 liter filter system, 0.22 µm (Corning 431206 purchased as VWR 28199-812 or equivalent)

  - Weigh paper

  - Weigh boats

  - Disposable plastic spatulas

  - RNase-free stir bar

- Equipment:

  - Gast vacuum pump (Model # DOA-P104-AA or equivalent)

  - pH Meter

  - Stir plate

  - Magnetic stir bar remover

- Safety:

  - Gloves

  - Eye protection

  - Lab coat

  - Fume hood

  - KOH can cause burns. Wear proper PPE including gloves and eye protection while handling this chemical. Use this chemical in a fume hood.

- Output:

  - Artificial Cerebrospinal Fluid (ACSF) containing:

    - 135.0 mM NaCl

    - 5.4 mM KCl

    - 1.0 mM MgCl2

    - 1.8 mM CaCl2

    - 5.0 mM HEPES

- Reference Documents:

  - N/A

Warning:   Personal Protective Equipment (PPE) should be used at all times while operating this protocol.  If you are unsure what PPE you should be using, see your immediate supervisor.

- Setup:

  - Obtain a sterile 1 liter plastic bottle. Label the container with the reagent name, date and your initials.

  - Obtain an RNase free stir bar and add to the container.

  - Set up stir plate.

  - Ensure pH meter is calibrated.

  - Ensure balance is calibrated.

- Methodology/Procedures (record all lot numbers and volumes used in the appropriate reagent prep notebook):

  - To make 1000 ml of Artificial Cerebrospinal Fluid (ACSF):

    - Using a graduated cylinder, measure and add 900 ml of MilliQ water to an appropriately labeled RNase free container containing an RNase free stir bar.

    - Place container on stir plate and begin stirring at a high setting (7 or higher).

    - Using a calibrated balance, weigh out 7.889 g of Sodium Chloride (NaCl). Slowly add weighed NaCl to the container while stirring.

    - Using a calibrated balance, weigh out 403 mg of Potassium Chloride (KCl). Slowly add weighed KCl to the container while stirring.

    - Using a calibrated balance, weigh out 203 mg of Magnesium Chloride (MgCl2). Slowly add weighed MgCl2 to the container while stirring.

    - Using a calibrated balance, weigh out 265 mg of Calcium Chloride (CaCl2). Slowly add weighed CaCl2 to the container while stirring.

    - Using a calibrated balance, weigh out 1.192 g of HEPES.  Slowly add weighed HEPES to the container while stirring.

Table 1: Reagents Required for Preparation of 1 Liter of ACSF

Reagent; 1000 ml. MilliQ Water; 900 ml. NaCl; 7.889 g. KCl; 403 mg. MgCl2; 203 mg. CaCl2; 265 mg. HEPES; 1.192 g. Bring up to this final volume with MilliQ water (after adjusting pH); 1000 ml.

    - Continue stirring at a high speed (setting 7 or higher) until all of the chemicals are fully in solution.

    - In a fume hood, use a calibrated pH meter to adjust the pH to 7.4 using 1N KOH.

      - A fume hood is required when adjusting pH with KOH.

    - Adjust final volume to 1000 ml with MilliQ water using the graduation marks on the side of the container (if calibrated) or a graduated cylinder.

    - Remove stir bar using a magnetic stir bar remover.

    - Filter the ACSF through a Corning 1 liter 0.22 µm filter system.

      - Using Tygon tubing, attach the Corning filter system to the Gast vacuum pump.  Turn on the pump.

      - Filter 500 ml at a time, stirring the remaining ACSF constantly on a stir plate.

    - Label the bottle with reagent name, date and preparer’s initials.

    - Can be stored at 4°C and used for up to one month.

- Take Down:

  - Dispose of excess reagent directly into municipal sewer system.

  - Wash the stir bar with RNase Away and rinse with MilliQ water.

- Technical Information:

  - Artificial Cerebrospinal Fluid (ACSF) has a shelf life of one month at 4°C.

  - The osmolarity of this formulation of ACSF is normally not measured or adjusted. However, if the end user asks that it be made with a specific osmolarity, measure the osmolarity of a 10 µl sample of the solution using an osmometer prior to filtration. If the osmolarity is not in the desired range, adjust as needed.

    - If the osmolarity is too high, add MilliQ water.

    - If the osmolarity is too low, add a volume (X) of  1 mg/ml NaCl solution determined by the following equation

      - D=desired osmolarity

      - O= observed osmolarity

      - X=volume of 1 mg/ml NaCl solution to add (in ml)

    - Confirm that the correct osmolarity has been reached after addition of NaCl or water.

- Appendix

  - Materials required for preparation of 1 Liter of ACSF

| Material | Supplier, Part Number | Amount Required to Prepare 1 Liter |
| --- | --- | --- |
| 1 Liter plastic bottle or equivalent as directed by project lead's request | VWR 28199-762 | 1 ea |
| 25 ml serological pipette | VWR 53300-567 | 1 ea |
| 1 liter filter system, 0.45 µm | Corning 431206 | 1 ea |
| Weigh paper | VWR 12578-165 | 3 ea |
| Weigh boats | VWR 12577-053 | 2 ea |
| Disposable plastic spatulas | VWR 80081-188 | 5 ea |
| Sodium Chloride (NaCl) | VWR JT4058 | 7.889 g |
| Magnesium Chloride (MgCl2) | VWR 80503-016 | 203 mg |
| Potassium Chloride (KCl) | Sigma P9333 | 403 mg |
| Calcium Chloride dihydrate | Sigma C3306 | 265 mg |
| HEPES | Sigma H3375 | 1.192 g |
| Potassium Hydroxide (1N KOH in water) | Acros Organics AC27840-0010 | 5 ml |

  - ACSF Formulation Comparison

| Component | ACSF.I | ACSF.II | ACSF.III | ACSF.IV | ACSF.V | ACSF.VI | ACSF.VII | ACSF.VIII | ACSF.IX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calcium Chloride, dihydrate | 0.5 mM | 2 mM | 2 mM | 2 mM | 1.8 mM | 2 mM | 0.5 mM | 2 mM | 1.3/2.0 mM |
| D-Glucose (Dextrose) | 25 mM | 25 mM | 12.5 mM | 25 mM | 0 | 20 mM | 25 mM | 25 mM | 25 mM |
| DL-AP5 | 0 | 0 | 0 | 0 | 0 | 50 µM | 0 | 0 | 0 |
| DNQX | 0 | 0 | 0 | 0 | 0 | 20 µM | 0 | 0 | 0 |
| HCl | 98 mM | 0 | 0 | 0 | 0 | 0 | 92 mM | 0 | 0 |
| HEPES | 20 mM | 16 mM | 0 | 20 mM | 5 mM | 0 | 20 mM | 20 mM | 0 |
| Kynurenic Acid | 0 | 0 | 1 mM | 0 | 0 | 0 | 0 | 0 | 0 |
| Magnesium Chloride | 0 | 0 | 0 | 0 | 1.0 mM | 2 mM | 0 | 0 | 0 |
| Magnesium Sulfate | 10 mM | 2 mM | 1 mM | 2 mM | 0 | 0 | 10 mM | 2 mM | 1.0/2.0 mM |
| Monosodium Phosphate | 1.25 mM | 1.25 mM | 1.25 mM | 1.25 mM | 0 | 1.25 mM | 1.2 mM | 1.2 mM | 1.2 mM |
| Myo inositol | 3 mM | 3 mM | 0 | 3 mM | 0 | 0 | 0 | 0 | 0 |
| N-acetylcysteine | 12 mM | 12 mM | 0 | 12.3 mM | 0 | 0 | 0 | 0 | 0 |
| N-methyl-d-glucamine | 96 mM | 0 | 0 | 0 | 0 | 0 | 92 mM | 0 | 0 |
| Picrotoxin | 0 | 0 | 0.1 mM | 0 | 0 | 0 | 0 | 0 | 0 |
| Potassium Chloride | 2.5 mM | 2.5 mM | 2.5 mM | 2.5 mM | 5.4 mM | 3 mM | 2.5 mM | 2.5 mM | 3mM |
| Sodium Bicarbonate | 25 mM | 25 mM | 26 mM | 25 mM | 0 | 20 mM | 30 mM | 30 mM | 18 mM |
| Sodium Chloride | 0 | 97 mM | 126 mM | 94 mM | 135 mM | 126 mM | 0 | 92 mM | 126 mM |
| Sodium L-Ascorbate | 5 mM | 5 mM | 0 | 5 mM | 0 | 0 | 5 mM | 5 mM | 0.16 mM |
| Sodium Pyruvate | 3 mM | 3 mM | 0 | 3 mM | 0 | 0 | 3 mM | 3 mM | 0 |
| Taurine | 0.01 mM | 0.01 mM | 0 | 0.01 mM | 0 | 0 | 0 | 0 | 0 |
| Thiourea | 2 mM | 2 mM | 0 | 2 mM | 0 | 0 | 2 mM | 2 mM | 0 |
| Tetrodotoxin (TTX) | 0 | 0 | 0 | 0 | 0 | 100 nM | 0 | 0 | 0 |