cDNA Synthesis

- Scope and Applicability: Covers the synthesis of first-strand cDNA from total RNA using the Invitrogen SuperScript III Reverse Transcriptase system.

- Materials:

  - Invitrogen Superscript III First-Strand Synthesis System (Invitrogen 18080-051) at

-20ºC

    - Anchored dT (5 µg/µl)

    - 10 mM dNTP mix

    - 10X RT Buffer

    - 25 mM MgCl2

    - 0.1 M DTT

    - RNase OUT

    - Superscript III RT

  - Total RNA (~5 µg) at -20ºC

  - Nuclease-Free Water (Ambion AM9932 or equivalent)

  - 1.7 ml microcentrifuge tubes

  - 96-well PCR Plate (Eppendorf 951020401)

- Equipment:

  - Rainin Single Pipette: p10, p20, p200

  - Rainin 12-channel p20

  - Rainin Electronic Single Repeater p200

  - Filter tips: p20, p200

  - Benchtop cooler

  - Benchtop Centrifuge (5810 R or equivalent)

  - Thermal cycler (MJ or Bio-Rad)

- Safety:

  - Nitrile Gloves

  - Eye protection

  - Lab coat

- Output:

  - cDNA for QC and downstream processes such as “PCR from cDNA for IVT Reactions”.

- Reference Documents:

  - MB0004 2100 Bioanalyzer Operation for PCR, GTP, and IVT Products

    - https://www.protocols.io/view/2100-bioanalyzer-operation-for-pcr-gtp-and-ivt-pro-bfzjjp4n

  - MB0050 PCR Prep from cDNA for IVT Reactions

    - https://www.protocols.io/view/pcr-prep-from-cdna-for-ivt-reactions-bep7jdrn/abstract

Warning:  Personal Protective Equipment (PPE) should be used at all times while operating this protocol. If you are unsure what PPE you should be using, see your immediate supervisor.

- Methodology/Procedures: NOTE: unless otherwise noted, ‘spin down’ means to spin the tubes in a microcentrifuge for 5-10” at top speed (or plates in the benchtop centrifuge for 2’ at 1800rpm) in order to drive the liquid to the bottom of the tube/wells.

  - Pull out Invitrogen SuperScript III First-Strand Synthesis kit from -20ºC:

    - Thaw at RT for 15’: Anchored dT, 10 mM dNTP mix, NFdH20, 10x RT Buffer, 25 mM MgCl2, 0.1 M DTT

    - Place in benchtop cooler:  RNase OUT, SuperScript III RT

    - Thaw RNA aliquot at RT for 10’ (RNA information can be found on the cDNA worksheet in the upper right highlighted in yellow)

    - Once thawed, vortex and spin down all reagents except flick to mix RNase OUT, SuperScript III RT and RNA aliquot.  Spin down all reagents.

    - Label a PCR plate with the cDNA plate name listed at the top of the paperwork.  It will have the format: cDNA_YYMMDD_PP, where YY=year, MM=month, DD=day, PP=plate#.

    - Label a 1.7 ml tube as “cDNA Syn Premix”; this will be your cDNA Synthesis Premix tube.  Label 1.7 ml tubes as “01”, “02”, “03”, etc. for each RNA+Primer Premix.  Consult with the paperwork to determine how many premix tubes are needed.

  - Preparing the RNA+Primer Premix:

    - Follow the table for preparing the RNA+Primer Premix on the cDNA worksheet.  Volume of each component is calculated based on the entries in cDNA tab and includes waste volume.

      - Example table:

    - Make up the RNA+Primer Premix in the corresponding labeled tube.

    - Add the components listed in the premix table in order- nuclease free water, anchored dT, then Total RNA.

    - Vortex all premixes.

  - Use the p200 repeater pipette with filter tips to aliquot premixes into the labeled PCR plate according to plate map.

  - Preheat the thermalcycler to 65ºC.  Choose the program for 65ºC for 10’.

  - Spin down and incubate the plate for 5 minutes at 65ºC.

  - After incubation, immediately place the plate in the benchtop cooler for 1’.

  - Spin down plate.

- Prepare cDNA Synthesis Premix:

  - While the RNA+Primer Plate is incubating, prepare the cDNA Synthesis Premix:

    - Spin down each reagent.

    - Follow the table for preparing the cDNA Synthesis Premix on the cDNA worksheet.  Volume of each component is calculated based on the entries in cDNA tab and includes waste volume Add reagents in the order listed in the table:

    - Flick to mix ten times.

  - Add 10 µl of the cDNA Synthesis Premix to each well containing the RNA+Primer premix using the 12-channel p20 with filter tips. Change tips between each row.

    - Mix by pipetting 3-4 times and spin down.

  - Incubate cDNA plate in thermalcycler on preset program “cDNA-dT”. The reaction will run as follows:

    - 50ºC for 50 minutes

    - 85ºC for 5 minutes

    - 4ºC forever

  - QC and Pooling:

    - Remove plate from the thermalcycler and spin down.

    - Run samples on the Bioanalyzer using the mRNA Nano program. Refer to MB0004 2100 Bioanalyzer Operation for PCR, GTP, IVT Products.

      - DO NOT denature samples.

      - Determine that all duplicate samples show alike traces.

  - After confirming duplicate samples are alike, the duplicate samples can be pooled into 1.7 ml tubes—different 1.7 ml tubes for each premix.

    - Label tubes as: “cDNA_YYMMDD_PP_WW-WW”, where YY=year, MM=month, DD=day, PP=plate#, WW=wells

- Take Down:

  - Reassemble Invitrogen SuperScript III First-Strand Synthesis System by replacing all tubes that were removed from the box.

    - Return box to -20ºC.

  - Store samples at -20ºC in box labeled “cDNA”.

  - Paste BA traces in documentation