Preparation of PLL (or PDL*) coated slides:

Reagents:

- 100% EtoH

- RNAse free water

- PLL or PDL 100µg/ml

  - 4ml PDL + 800uL 10XPBS + 3.2ml DW

- 1% Bind-silane

  - 100% Ethanol 180ml+ 1M Acetic Acid 2ml+ RNase-free water 16ml+ Bind-silane 2ml

Materials:

- coverslips in the desired thickness

- Fine tweezers

- Sharp diamond pen to cut glass

- Glass slide holder

Procedure:

- Cut the left upper corner of the coverslip – this will be the side that is coated.

- Place coverslips in slide holder and fill with 100% EtoH to cover the coverslips completely

- Remove access EtoH and place in oven to dry completely (about 5-10min)

- Plasma clean coverslips:

  - Remove plastic cover from plasma oven, put coverslips on it.

  - Close black lid with closed air valve. Hold it firmly until vacuum is created.

  - Do not adjust the wheel

  - Power ‘ON’

  - Pump ‘ON’ (down)

  - When the micron indicator reaches 400, turn valve 90 clockwise – this allows a small amount of air flow to create the plasma

  - when the micron indicator reaches 900, turn the RF level to ‘high’ and set a timer for 5 min

  - when the 5 min are up, hold the lid firmly. turn RF level to ‘OFF’ turn pump ‘OFF’ and Power ‘OFF’

  - Turn the valve VERY SLOWLY 180 counter-clockwise to insert air.

  - Remove the lid and take the slides out gently

  - Return plastic cover

- Place coverslips with the plasma side facing up in a petri dish/on a clean surface covered with parafilm that you’re  comfortable working on

- Place coverslips in Bind silane for 1h RT.

- Discard the bind-silane solution and rinse coverslips 3x with 100% ethanol.

- Heat cure coverslips in oven for 30 min at 90ºC.

- Lay coverslips flat (functionalized side up; on parafilm or in petri dishes) and treat with PDL:

  - Cover each slide with ~1mL of PDL/PBS solution, ensuring that as much of the slide as possible is covered with solution, especially the area where samples will be placed.

  - Incubate with PDL for at least 3h, ideally overnight, at room-temperature.

- Remove PDL, wash coverslips with RNase-free water and let air dry.

- Attach coverslips to the glass-slides by placing a drop of ethanol on the slide and putting the coverslip on it (coated side up-this is where the tissue will be)

- Store in (-20). Use within 1 week.

After the coverslips are ready, slice the tissue in the desired thickness (usually 10-100 micron)