Methods

# Cardiovascular profile

A maternal cardiovascular profile was assessed in every pregnant woman combining three non-invasive techniques to obtain information about arteries, veins, heart and body fluid content (Table 1). A standardized protocol was used as reported in previous studies (28).

Blood pressure measurement

Blood pressures were measured twice in each participant, once in supine and once in standing position, in order to evaluate the cardiovascular orthostatic reflex response of which results have been reported elsewhere (56, 67). Arterial blood pressure was measured from the right brachial artery using an appropriate cuff size of the automated sphygmomanometer incorporated in the impedance cardiography monitoring device (see further). Measurements in the upright position were recorded after three minutes of adaptation following the postural change, which is reported to be representative for long lasting effects after posture change (5, 12). For this study, only standing values were considered of systolic (SBP), diastolic (DBP) and mean arterial pressure (MAP), as defined in Table 1.

# Bio-impedance spectrum analysis

The body composition and fluid balance were measured by a multiple frequency bioelectrical impedance analyser (Maltron Bioscan 920-II, Maltron International LTD, Essex, UK) in supine position with stretched arms and legs, without socks or shoes (57). Two electrodes, receiving the electrical signal, were placed on the dorsal surfaces of the wrist and ankle at the level of the process of the radial and ulnar resp. fibular and tibial bones. Two other electrodes, sending the electrical signal, were attached to the third metacarpal bone of the right hand and right foot. The applied current was 0.6 mA with a frequency of 5, 50, 100 and 200 kHz during 5 seconds. Total Body Water (TBW) is estimated by bio-impedance as explained briefly in Table 1. TBW totals intracellular water (ICW) and extracellular water (ECW), which in turn is considered the sum of interstitial, transcellular water and plasma volume.

Impedance Cardiography (ICG)

The Non-Invasive Continuous Cardiac Output Monitor (NICCOMO, Medis Medizinische Messtechnik GmbH, Ilmenau, Germany) was used for impedance cardiography assessment with four skin-resistance-eliminating electrodes, two on the axillary line under the thorax and two in the neck. The examination was performed after 3-minute stabilization of cardiovascular function in standing position. Parameters were classified into three groups. (a) Cardiac function parameters are heart rate (HR) in beats per minute and stroke volume (SV), as calculated from the ICG measurements using the formula of Bernstein (2). Cardiac output (CO) in L/min is calculated as HR x SV. (b) Characteristics of Aorta flow velocities are represented by Velocity index (VI), defined as the peak systolic flow expressed in 1/1000/s, and acceleration index (ACI), defined as maximum acceleration of systolic flow expressed in 1/100/s². (c) Total Peripheral Resistance (TPR) is calculated as MAP x 80/CO and expressed in dyn.sec/cm5 (22).

# Uterine Artery Doppler

As reported elsewhere, the automated algorithm of the Doppler ultrasound scanner was used to measure Resistivity index (RI = [Peak systolic velocity – minimal diastolic velocity]/Peak systolic velocity) and Pulsatility Index (PI = [Peak systolic velocity – minimal diastolic velocity]/Mean velocity) in the uterine arcuate arteries, at < 2cm of the insertion of the intra-uterine branching of the uterine artery (28, 42) . RI is a proxy for arterial wall counteraction against forward blood flow, altered by a combination of vascular compliance and resistance (8). PI merely reflects the intra-cycle variability of blood flow velocities in a vessel. Both parameters were measured three consecutive times at each side, and averaged as part of a standardized protocol (54).

# Hepatic and Renal venous ECG-Doppler Ultrasound

An electrocardiogram was used to assist Doppler ultrasonography of the maternal renal interlobar and hepatic veins using a 3.5 MHz transabdominal probe during interrupted breathing in supine position (Aplio Mx, Toshiba Medical Systems nv, Sint-Stevens-Woluwe, Belgium). Each parameter was measured three consecutive times and averaged as part of a standardized protocol, reducing intra-and inter-variability (54). Parameters were classified into 2 groups: heart rate corrected pulse transit and impedance indices.

The heart rate corrected venous pulse transit (VPT) is the time interval between the P-top from the ECG-wave and the A-wave of the Doppler pulse wave, and expressed in ms. The atrial contraction is responsible for a retrograde rebounce of blood into the venous system (13). This rebound can be observed by Doppler sonography up to the level of the renal interlobar veins in pregnancies complicated with early onset preeclampsia (28). The P-A time interval, labeled as venous pulse transit (VPT) is a measure for the velocity at which this rebouncing A-wave propagates through the venous system. As heart rate varies during the course of gestation, VPT is expressed as a fraction of heart rate, measured as the time interval in msec between the consecutive ECG R-waves of the corresponding heart cycle. As such, VPT is whithout units. VPT is considered the venous equivalent of arterial pulse transit time (or pulse wave velocity), and is inversely related to vascular wall stiffness: the shorter this time interval, the faster and more distant the rebound of atrial contraction travels in retrograde direction through the central veins (28, 68). Venous vascular stiffness depends on vaso-activity from autonomic nervous system induced or endothelium dependent smooth muscle contraction, as well as on (micro-) structural composition of the different layers of the vascular walls (20).

At the venous side, the maximum and minimum flow velocity is measured at the level of the renal interlobar and hepatic veins as explained elsewhere (26, 27). Venous flow velocities depend on the pressure gradient relative to the right atrium, intravascular filling, vascular tone and external compression (21). A venous impedance index is calculated using the formula [(Maximum Velocity-Minimum Velocity)/Maximum velocity] (27). Venous impedance index is the venous Doppler equivalent arterial PI, representing the intra-cycle variability of blood flow velocities in the veins.

# Statistics

An independent t-test at 5% significance level was used for intergroup comparison of continuous demographic data. Chi-square test was used for categorical demographic variables. Normality was checked via Shapiro-Wilk. Data were presented as mean ± SD or n (%).

Linear Mixed Models for repeated measurements were used to examine differences between UP and all types of GHD. A random patient effect was used to correct for the correlation between trimestrial measurements of a pregnancy, as multiple measurements were performed in 48% of the patients. Fixed effects of trimester and group, as well as their interaction term were specified. The fixed effects structure was simplified by using a significance level of 5%. Analyses were done in SAS (SAS 9.4, Institute Inc., Cary, NC, USA). A correction for demographical influences (BMI, nulliparity and age) on the cardiovascular parameters was implemented in the linear mixed model. Corrections for multiple testing were not implemented.

Interpretation of measured values

Hypertension is defined according to ISSHP criteria as systolic and/or diastolic pressure repeatedly ≥ 140 or 90 mm Hg respectively (7). As discussed elsewhere (67), blood pressures in GHD can be significantly higher than in UP, without meeting the criteria of hypertension.

Body water volumes

To allow comparison within and between groups, all measurements of TBW, ECW and ICW are corrected for age, parity and BMI with mean values estimated for a 30 year old primigravid with BMI 23.

Cardiac function parameters

Similar to body water volumes, all measurements of SV, HR, VI and ACI are standardized for age, parity and BMI, as such representing the values obtained in a 30 year old primigravid with BMI 23, which allows for intra- and intergroup comparison. CO is the product of SV and HR, which indicates that opposite abnormal trends of SV and HR can present with normal CO.

Aorta flow velocities

Reduced VI and/or AI reflect abnormal central arterial function. The latter relates to  cardiac systolic function, aorta or peripheral arterial pressure, intravascular filling and aorta compliance (43).

Total peripheral resistance

TPR represents the overall arterial resistance in the systemic circulation.

Uterine arterial flow

Uterine artery PI and RI relate directly to vascular wall resistance against uterine blood flow. Increased PI and RI are reported in GHD and/or poor fetal growth (10). Unilateral abnormal values in left or right uterine arteries are very common, and therefore some authors recommend averaging (32). For this study, we preferred to present unilateral left and right values, similar to the venous Doppler values of renal interlobar veins (see further).

Hepatic venous flow

In non-pregnant individuals, hepatic venous flow shows a triphasic waveform (Venous impedance index > 1), and this turns to a completely flat pattern in uncomplicated third trimester pregnancy (Venous Impedance Index = 0) (26). This shift is associated with increase of intra-abdominal pressure (62), a physiologic feature of uncomplicated pregnancy (55), and also with increase of VPT (61). In the clinical stage of early onset preeclampsia, triphasic HV patterns are frequently observed, together with short VPT (25, 61). The same is true for late onset preeclampsia, but to a much lesser degree (39).

Renal interlobar vein Doppler

During the course of uncomplicated pregnancy, VPT increases (61) and renal interlobar venous impedance index decreases (27). In early onset PE, VPT is short and rebound of atrial contraction is responsible for the so-called venous pre-acceleration nadir (24), with subsequent increase of Impedance Index in one or both kidneys. This can be observed already weeks before clinical onset of disease (39). In late onset PE, borderline reduced VPT and increased RIVI are reported, usually unilaterally and presenting only at onset of disease (39). Gestational hypertension presents without abnormalities of VPT and Impedance Index (28, 29).

Figure 1. Study protocol of the combined assessment of body volumes, cardiac function, central & peripheral arterial function, uterine arterial flow, hepatic and renal venous flow, as performed since 2011 in the ongoing Hasselt University Research Project of Maternal Venous Hemodynamics. In consecutive order (from left to right): Doppler ultrasound and bio-impedance spectrum analysis in supine position, sphyngomanometric blood pressure measurement after 2 min stabilization in standing position, with another 1 min interval before impedance cardiographic assessment.

# References

1.	Beale AL, Meyer P, Marwick TH, Lam CSP, and Kaye DM. Sex Differences in Cardiovascular Pathophysiology: Why Women Are Overrepresented in Heart Failure With Preserved Ejection Fraction. Circulation 138: 198-205, 2018.

2.	Bernstein DP, and Lemmens HJ. Stroke volume equation for impedance cardiography. Medical & biological engineering & computing 43: 443-450, 2005.

3.	Bernstein IM, Meyer MC, Osol G, and Ward K. Intolerance to volume expansion: a theorized mechanism for the development of preeclampsia. Obstetrics and gynecology 92: 306-308, 1998.

4.	Borges VTM, Zanati SG, PeraColi MTS, Poiati JR, Romao-Veiga M, PeraColi JC, and Thilaganathan B. Maternal hypertrophy and diastolic disfunction and brain natriuretic peptide concentration in early and late Pre-Eclampsia. Ultrasound in obstetrics & gynecology : the official journal of the International Society of Ultrasound in Obstetrics and Gynecology 2017.

5.	Borst C, van Brederode JF, Wieling W, van Montfrans GA, and Dunning AJ. Mechanisms of initial blood pressure response to postural change. Clinical science (London, England : 1979) 67: 321-327, 1984.

6.	Brown MA, Lindheimer MD, de Swiet M, Van Assche A, and Moutquin JM. The classification and diagnosis of the hypertensive disorders of pregnancy: statement from the International Society for the Study of Hypertension in Pregnancy (ISSHP). Hypertension in pregnancy 20: IX-XIV, 2001.

7.	Brown MA, Magee LA, Kenny LC, Karumanchi SA, McCarthy FP, Saito S, Hall DR, Warren CE, Adoyi G, and Ishaku S. Hypertensive Disorders of Pregnancy: ISSHP Classification, Diagnosis, and Management Recommendations for International Practice. Hypertension (Dallas, Tex : 1979) 72: 24-43, 2018.

8.	Bude RO, and Rubin JM. Relationship between the resistive index and vascular compliance and resistance. Radiology 211: 411-417, 1999.

9.	Chmielewski J, Lewandowski RJ, and Maddur H. Hepatorenal Syndrome: Physiology, Diagnosis and Management. Seminars in interventional radiology 35: 194-197, 2018.

10.	Cnossen JS, Morris RK, ter Riet G, Mol BW, van der Post JA, Coomarasamy A, Zwinderman AH, Robson SC, Bindels PJ, Kleijnen J, and Khan KS. Use of uterine artery Doppler ultrasonography to predict pre-eclampsia and intrauterine growth restriction: a systematic review and bivariable meta-analysis. CMAJ : Canadian Medical Association journal = journal de l'Association medicale canadienne 178: 701-711, 2008.

11.	de Haas S, Ghossein-Doha C, van Kuijk SM, van Drongelen J, and Spaanderman ME. Physiological adaptation of maternal plasma volume during pregnancy: a systematic review and meta-analysis. Ultrasound in obstetrics & gynecology : the official journal of the International Society of Ultrasound in Obstetrics and Gynecology 49: 177-187, 2017.

12.	Del Bene R, Barletta G, Mello G, Lazzeri C, Mecacci F, Parretti E, Martini E, Vecchiarino S, Franchi F, and La Villa G. Cardiovascular function in pregnancy: effects of posture. BJOG : an international journal of obstetrics and gynaecology 108: 344-352, 2001.

13.	Devine PJ, Sullenberger LE, Bellin DA, and Atwood JE. Jugular venous pulse: window into the right heart. Southern medical journal 100: 1022-1027; quiz 1004, 2007.

14.	Dupont M, Mullens W, and Tang WH. Impact of systemic venous congestion in heart failure. Curr Heart Fail Rep 8: 233-241, 2011.

15.	Duvekot JJ, Cheriex EC, Pieters FA, Menheere PP, and Peeters LH. Early pregnancy changes in hemodynamics and volume homeostasis are consecutive adjustments triggered by a primary fall in systemic vascular tone. American journal of obstetrics and gynecology 169: 1382-1392, 1993.

16.	Duvekot JJ, and Peeters LL. Renal hemodynamics and volume homeostasis in pregnancy. Obstet Gynecol Surv 49: 830-839, 1994.

17.	Egeland GM, Klungsoyr K, Oyen N, Tell GS, Naess O, and Skjaerven R. Preconception Cardiovascular Risk Factor Differences Between Gestational Hypertension and Preeclampsia: Cohort Norway Study. Hypertension (Dallas, Tex : 1979) 67: 1173-1180, 2016.

18.	Esposito R, Izzo R, Galderisi M, De Marco M, Stabile E, Esposito G, Trimarco V, Rozza F, De Luca N, and de Simone G. Identification of phenotypes at risk of transition from diastolic hypertension to isolated systolic hypertension. Journal of human hypertension 30: 392-396, 2016.

19.	Ferrazzi E, Stampalija T, Monasta L, Di Martino D, Vonck S, and Gyselaers W. Maternal hemodynamics: a method to classify hypertensive disorders of pregnancy. American journal of obstetrics and gynecology 218: 124.e121-124.e111, 2018.

20.	Fleenor BS, and Berrones AJ. Arterial Stiffness. In: Implications and InterventionsSpringer International Publishing, 2015, p. 61.

21.	Gelman S. Venous function and central venous pressure: a physiologic story. Anesthesiology 108: 735-748, 2008.

22.	GmbH mMM. IMPEDANCE CARDIOGRAPHY WITH ACM TECHNOLOGY. Medis.

23.	Guidelines QC. Term small for gestation age baby. edited by Committee QCGS, and (Queensland) SMaNCN2016.

24.	Gyselaers W. Hemodynamics of the maternal venous compartment: a new area to explore in obstetric ultrasound imaging. Ultrasound in obstetrics & gynecology : the official journal of the International Society of Ultrasound in Obstetrics and Gynecology 32: 716-717, 2008.

25.	Gyselaers W, Mesens T, Tomsin K, Molenberghs G, and Peeters L. Maternal renal interlobar vein impedance index is higher in early- than in late-onset pre-eclampsia. Ultrasound in obstetrics & gynecology : the official journal of the International Society of Ultrasound in Obstetrics and Gynecology 36: 69-75, 2010.

26.	Gyselaers W, Molenberghs G, Mesens T, and Peeters L. Maternal hepatic vein Doppler velocimetry during uncomplicated pregnancy and pre-eclampsia. Ultrasound in medicine & biology 35: 1278-1283, 2009.

27.	Gyselaers W, Molenberghs G, Van Mieghem W, and Ombelet W. Doppler measurement of renal interlobar vein impedance index in uncomplicated and preeclamptic pregnancies. Hypertension in pregnancy 28: 23-33, 2009.

28.	Gyselaers W, Staelens A, Mesens T, Tomsin K, Oben J, Vonck S, Verresen L, and Molenberghs G. Maternal venous Doppler characteristics are abnormal in pre-eclampsia but not in gestational hypertension. Ultrasound in obstetrics & gynecology : the official journal of the International Society of Ultrasound in Obstetrics and Gynecology 45: 421-426, 2015.

29.	Gyselaers W, Tomsin K, Staelens A, Mesens T, Oben J, and Molenberghs G. Maternal venous hemodynamics in gestational hypertension and preeclampsia. BMC pregnancy and childbirth 14: 212, 2014.

30.	Khalil A, Garcia-Mandujano R, Maiz N, Elkhouli M, and Nicolaides KH. Longitudinal changes in maternal hemodynamics in a population at risk for pre-eclampsia. Ultrasound in obstetrics & gynecology : the official journal of the International Society of Ultrasound in Obstetrics and Gynecology 44: 197-204, 2014.

31.	Kocemba J, Kawecka-Jaszcz K, Gryglewska B, and Grodzicki T. Isolated systolic hypertension: pathophysiology, consequences and therapeutic benefits. Journal of human hypertension 12: 621-626, 1998.

32.	Kofinas AD, Penry M, Swain M, and Hatjis CG. Effect of placental laterality on uterine artery resistance and development of preeclampsia and intrauterine growth retardation. American journal of obstetrics and gynecology 161: 1536-1539, 1989.

33.	Larciprete G, Valensise H, Vasapollo B, Altomare F, Sorge R, Casalino B, De Lorenzo A, and Arduini D. Body composition during normal pregnancy: reference ranges. Acta diabetologica 40 Suppl 1: S225-232, 2003.

34.	Marik PE, Baram M, and Vahid B. Does central venous pressure predict fluid responsiveness? A systematic review of the literature and the tale of seven mares. Chest 134: 172-178, 2008.

35.	McLaughlin K, Zhang J, Lye SJ, Parker JD, and Kingdom JC. Phenotypes of Pregnant Women Who Subsequently Develop Hypertension in Pregnancy. Journal of the American Heart Association 7: 2018.

36.	Melchiorre K, Sharma R, Khalil A, and Thilaganathan B. Maternal Cardiovascular Function in Normal Pregnancy: Evidence of Maladaptation to Chronic Volume Overload. Hypertension (Dallas, Tex : 1979) 67: 754-762, 2016.

37.	Melchiorre K, Sharma R, and Thilaganathan B. Cardiac structure and function in normal pregnancy. Curr Opin Obstet Gynecol 24: 413-421, 2012.

38.	Melchiorre K, Sharma R, and Thilaganathan B. Cardiovascular implications in preeclampsia: an overview. Circulation 130: 703-714, 2014.

39.	Mesens T, Tomsin K, Oben J, Staelens A, and Gyselaers W. Maternal venous hemodynamics assessment for prediction of preeclampsia should be longitudinal. The journal of maternal-fetal & neonatal medicine : the official journal of the European Association of Perinatal Medicine, the Federation of Asia and Oceania Perinatal Societies, the International Society of Perinatal Obstet 28: 311-315, 2015.

40.	Miller WL. Assessment and Management of Volume Overload and Congestion in Chronic Heart Failure: Can Measuring Blood Volume Provide New Insights? Kidney Dis (Basel) 2: 164-169, 2017.

41.	Myatt L, and Roberts JM. Preeclampsia: Syndrome or Disease? Current hypertension reports 17: 83, 2015.

42.	Nelson TR, and Pretorius DH. The Doppler signal: where does it come from and what does it mean? AJR American journal of roentgenology 151: 439-447, 1988.

43.	O'Rourke MF, and Adji A. Noninvasive studies of central aortic pressure. Current hypertension reports 14: 8-20, 2012.

44.	Poelzl G, and Auer J. Cardiohepatic syndrome. Current heart failure reports 12: 68-78, 2015.

45.	Redman CW, and Sargent IL. Latest advances in understanding preeclampsia. Science (New York, NY) 308: 1592-1594, 2005.

46.	Reslan OM, and Khalil RA. Molecular and vascular targets in the pathogenesis and management of the hypertension associated with preeclampsia. Cardiovasc Hematol Agents Med Chem 8: 204-226, 2010.

47.	Ronco C, and Di Lullo L. Cardiorenal syndrome. Heart failure clinics 10: 251-280, 2014.

48.	Sanghavi M, and Rutherford JD. Cardiovascular physiology of pregnancy. Circulation 130: 1003-1008, 2014.

49.	Scandiuzzi RM, Prado CA, Araujo Junior E, Duarte G, Quintana SM, da Silva Costa F, Tonni G, Cavalli Rde C, and Marcolin AC. Maternal uterine artery Doppler in the first and second trimesters as screening method for hypertensive disorders and adverse perinatal outcomes in low-risk pregnancies. Obstet Gynecol Sci 59: 347-356, 2016.

50.	Scholten RR, Lotgering FK, Hopman MT, Van Dijk A, Van de Vlugt M, Janssen MC, and Spaanderman ME. Low Plasma Volume in Normotensive Formerly Preeclamptic Women Predisposes to Hypertension. Hypertension (Dallas, Tex : 1979) 66: 1066-1072, 2015.

51.	Scholten RR, Oyen WJ, Van der Vlugt MJ, Van Dijk AP, Hopman MT, Lotgering FK, and Spaanderman ME. Impaired fetal growth and low plasma volume in adult life. Obstetrics and gynecology 118: 1314-1322, 2011.

52.	Sibai B, Dekker G, and Kupferminc M. Pre-eclampsia. Lancet (London, England) 365: 785-799, 2005.

53.	Staelens A, Tomsin K, Grieten L, Oben J, Mesens T, Spaanderman M, Jacquemyn Y, and Gyselaers W. Non-invasive assessment of gestational hemodynamics: benefits and limitations of impedance cardiography versus other techniques. Expert review of medical devices 10: 765-779, 2013.

54.	Staelens AS, Tomsin K, Oben J, Mesens T, Grieten L, and Gyselaers W. Improving the reliability of venous Doppler flow measurements: relevance of combined ECG, training and repeated measures. Ultrasound in medicine & biology 40: 1722-1728, 2014.

55.	Staelens AS, Van Cauwelaert S, Tomsin K, Mesens T, Malbrain ML, and Gyselaers W. Intra-abdominal pressure measurements in term pregnancy and postpartum: an observational study. PloS one 9: e104782, 2014.

56.	Staelens AS, Vonck S, Mesens T, Tomsin K, Molenberghs G, and Gyselaers W. Type-specific orthostatic hemodynamic response of hypertensive diseases in pregnancy. Clinical and experimental pharmacology & physiology 42: 1036-1044, 2015.

57.	Staelens AS, Vonck S, Molenberghs G, Malbrain ML, and Gyselaers W. Maternal body fluid composition in uncomplicated pregnancies and preeclampsia: a bioelectrical impedance analysis. European journal of obstetrics, gynecology, and reproductive biology 204: 69-73, 2016.

58.	Steegers EA, von Dadelszen P, Duvekot JJ, and Pijnenborg R. Pre-eclampsia. Lancet (London, England) 376: 631-644, 2010.

59.	Tang WH, and Kitai T. Intrarenal Venous Flow: A Window Into the Congestive Kidney Failure Phenotype of Heart Failure? JACC Heart failure 4: 683-686, 2016.

60.	Tay J, Foo L, Masini G, Bennett PR, McEniery CM, Wilkinson IB, and Lees CC. Early and late preeclampsia are characterized by high cardiac output, but in the presence of fetal growth restriction, cardiac output is low: insights from a prospective study. American journal of obstetrics and gynecology 218: 517.e511-517.e512, 2018.

61.	Tomsin K, Mesens T, Molenberghs G, and Gyselaers W. Venous pulse transit time in normal pregnancy and preeclampsia. Reproductive sciences (Thousand Oaks, Calif) 19: 431-436, 2012.

62.	Tomsin K, Vriens A, Mesens T, and Gyselaers W. Non-invasive cardiovascular profiling using combined electrocardiogram-Doppler ultrasonography and impedance cardiography: An experimental approach. Clinical and experimental pharmacology & physiology 40: 438-442, 2013.

63.	Valensise H, Vasapollo B, Gagliardi G, and Novelli GP. Early and late preeclampsia: two different maternal hemodynamic states in the latent phase of the disease. Hypertension (Dallas, Tex : 1979) 52: 873-880, 2008.

64.	Valensise H, Vasapollo B, Novelli GP, Larciprete G, Andreoli A, Altomare F, Di Pierro G, Galante A, Arduini D, De Lorenzo A, and Caserta D. Maternal cardiac systolic function and total body water estimation in normal and gestational hypertensive women. Acta diabetologica 40 Suppl 1: S216-221, 2003.

65.	Veerbeek JH, Hermes W, Breimer AY, van Rijn BB, Koenen SV, Mol BW, Franx A, de Groot CJ, and Koster MP. Cardiovascular disease risk factors after early-onset preeclampsia, late-onset preeclampsia, and pregnancy-induced hypertension. Hypertension (Dallas, Tex : 1979) 65: 600-606, 2015.

66.	Verlohren S, Perschel FH, Thilaganathan B, Droge LA, Henrich W, Busjahn A, and Khalil A. Angiogenic Markers and Cardiovascular Indices in the Prediction of Hypertensive Disorders of Pregnancy. Hypertension (Dallas, Tex : 1979) 69: 1192-1197, 2017.

67.	Vonck S OJ, Staelens AS, Lanssens D, Molenberghs G, Gyselaers W. Optimization of Simple Sphygmomanometric Blood Pressure Measurement in Routine Prenatal Care. Health Care Current Reviews 5: 2017.

68.	Vonck S, Staelens AS, Bollen I, Broekx L, and Gyselaers W. Why non-invasive maternal hemodynamics assessment is clinically relevant in early pregnancy: a literature review. BMC pregnancy and childbirth 16: 302, 2016.

69.	Vonck S, Staelens AS, Mesens T, Tomsin K, and Gyselaers W. Hepatic hemodynamics and fetal growth: a relationship of interest for further research. PloS one 9: e115594, 2014.

70.	Wang Y, Xing F, Liu R, Liu L, Zhu Y, Wen Y, Sun W, and Song Z. Isolated diastolic hypertension associated risk factors among Chinese in Anhui Province, China. International journal of environmental research and public health 12: 4395-4405, 2015.