Mouse Whole Cell Tissue Processing

for 10x Genomics Platform

- Scope and Applicability: This protocol describes how to isolate tissue from various ROIs of the mouse brain followed by preparation of a highly concentrated single cell suspension for transcriptomic profiling on 10x Genomics platform.

- Materials:

  - Acute adult mouse brain tissue slices

  - ACSF.I (RP0218)

  - Trehalose Dihydrate (Sigma T9531 or equivalent)

  - RNase Away RNase Decontamination Solution (VWR 17810-491)

  - 30% BSA (Sigma 126625 or equivalent)

  - Fetal Bovine Serum (HyClone SH30070.02)

  - Trypan Blue, 0.4% (Gibco/Thermo Fisher Scientific 15250-061)

  - DAPI (Thermo Fisher Scientific D1306)

  - Papain (Worthington LK003176)

  - 15 ml conical tubes (VWR 21008-670 or equivalent)

  - 50 ml conical tubes (VWR 21008-951 or equivalent)

  - 1.2 ml Cluster tubes (Sigma CLS4401)

  - Cluster tube racks (Sigma CLS4411)

  - 70 µm cell strainers (CELLTREAT 229483 or equivalent)

  - 250 ml bottles (VWR 28199-758 or equivalent)

  - 1.5 ml microcentrifuge tubes, PCR grade RNase-free (Eppendorf, DNA LoBind, 0030108051)

  - 5 ml Polystyrene round-bottom tubes (VWR 60819-295 or equivalent)

  - Syringe filter, 0.22 µm (EMD Millipore, Millex PES 30 mm sterile syringe filters, SLGP033RB or equivalent)

  - 30 ml and/or 50 ml syringe (luer lock)

  - Sylgard-coated dishes (Living Systems Instrumentation DD-50-S-3PK, or equivalent)

  - Disposable Plastic Handle Microknives, 15 and/or 22.5 degree blade (Fine Science Tools 10318-12 or equivalent)

  - Razor blade (VWR 94-0391 or equivalent)

  - Transfer pipettes

  - P20 pipette tips

  - P200 pipette tips

  - P200 wide-bore pipette tips

  - P1000 pipette tips

  - P1000 wide-bore pipette tips

  - 2 ml Serological pipettes

  - 5 ml Serological pipettes

  - 10 ml Serological pipettes

  - 9” glass Pasteur pipettes

  - 5 ml Syringe (VWR BD309646 or equivalent)

  - 20g needle (VWR BD305175 or equivalent)

  - Kimwipes

  - Wet ice

  - 70% EtOH (RP0032)

  - Hoechst 33342 10 mg/ml (ThermoFisher H3570)

  - MilliQ Water

  - Aluminum foil

  - Dimethyl Sulfoxide (DMSO), molecular biology or cell culture grade (Sigma D8418 or equivalent)

  - Solution 13, AO DAPI 910-3013 (Chemometec 910-3013)

  - LS Columns (Miltenyibiotec 130-042-401)

  - 13 ml Tube for LS Columns (Miltenyibiotec 130-091-596)

  - Myelin Removal Beads (Miltenyibiotec 130-096-433)

- Equipment:

  - Fluorescent dissecting microscope, camera and image acquisition software (Nikon SMZ1500, or equivalent)

  - Refrigerated centrifuge with 1.7 ml and 15 ml or 50 ml tube rotors (Sorvall ST8R or ST40R or equivalent)

  - Nucleocounter (Chemometec NC3000)

  - Dry oven set to 35ºC (Thermo Scientific PR305225G or equivalent)

  - Laboratory Portable Bunsen Burner (PRO-2121)

  - Forceps

  - 4°C refrigerator

  - Graduated cylinder

  - Phase contrast inverted microscope

  - Vortex mixer

  - P2 pipette

  - P10 Pipette

  - P200 Pipette

  - P1000 Pipette

  - Serological Pipettor

  - Disposable hemocytometer (INCTYO C-Chip or equivalent)

  - Manual Hand Tally Cell Counter (Universal Medical HS6594 or equivalent)

  - MACS MultiStand

- Safety:

  - Nitrile Gloves

  - Eye protection

  - Lab coat

Warning:  Personal Protective Equipment (PPE) should be used at all times while operating this protocol. If you are unsure what PPE you should be using, see your immediate supervisor.

- Output:

  - Concentrated cell suspension for 10x Genomics protocol with optional Calcein AM staining.

- Reference Documents:

  - AF0100 Slice Preparation with Tissue Dissociation Mouse

    - https://www.protocols.io/view/slice-preparation-with-tissue-dissociation-mouse-p-bq6wmzfe

  - RP0032 Ethanol Dilutions

    - To be Published

  - RP0218 Artificial Cerebrospinal Fluid (ACSF.I)

    - https://www.protocols.io/view/artificial-cerebrospinal-fluid-i-acsf-i-bezkjf4w

  - TPWI-011 BFI Software Operation

- Setup:

  - Prepare fire-polished pipettes (if needed):

    - Place portable Bunsen burner on stand so that nozzle is facing upwards.

    - Make sure the safety switch is unlocked. Open the gas valve by turning the red knob on the back of the device.

    - Firmly pull the trigger to light the flame.

    - Turn the gas down until the flame is at a steady peak and not wavering in the air.

    - Using forceps, remove the cotton filter from a glass 9” pipette.

    - Hold the skinny neck of the pipette over the peak of the flame about ½” from the point on the pipette where the taper begins.

    - As the glass begins to melt, pull the ends of the pipette away from each other.

    - Discard the broken off skinny end of the pipette.

    - Using a pair of forceps, gently break off the very tip of the fired pipette end. There should now be a small opening at the end of the pipette.

    - Place the broken end of the pipette back into the flame for brief, three second intervals to smooth out the broken glass. Inspect the glass each time to ensure the pipette tip has not closed.

    - Repeat steps 7.1.5 – 7.1.10 varying the time the pipette is held in the flame by a few seconds, until you have pipettes with fire-polished ends of large (~600 µm), medium (~300 µm), and small (~150 µm) thickness. Pipette size is estimated when priming with ACSF, where the largest bore will pull up solution the fastest.

  - Prepare 250 ml 0.132M Trehalose Stock (if needed).

    - Add 200 ml MilliQ water into a clean container with a stir bar. Place beaker on a stir plate at medium speed.

    - Weigh out and add 12.485 g Trehalose dihydrate.

    - Once Trehalose has fully dissolved, transfer solution back to a graduated cylinder.

    - Adjust final volume to 250 ml with MilliQ water and transfer to a clean 250 ml Corning bottle.

    - 0.132M Trehalose can be stored at 4˚C for up to 30 days.

  - Prepare 10% 0.132M Trehalose in ACSF.I (ACSF.IT)

    - Using a serological pipette, add 20 ml 0.132M Trehalose to a graduated cylinder.

    - Using a serological pipette, adjust volume to 200 ml with ACSF.I. Transfer to sterile 250 ml bottle.

  - Prepare papain.

    - Determine volume of ACSF.IT required to prepare 30 U/ml solution.

      - Check the Units of activity for the specific vial/lot of papain to be used and calculate volume of ACSF to add using the following equation:

# Units Stock Vial/30U final volume= Volume ACSF.IT to add (ml)

      - In the example above if stock papain vial contains 125U, then 125/30=4.167 ml of ACSF.IT should be added to reach 30U/ml working volume.

    - Remove metal seal from papain vial, exposing rubber plug.

    - Attach a 20g needle to a 5 ml syringe. Use it to measure out ACST.IT.

    - Insert needle through plug of papain vial and dispense ACSF.IT. Invert 8-10 times or as necessary to ensure all powder has been dissolved.

    - Place vial in dry oven set to 35ºC until ready for use (minimum 30 min before digestion).

  - Prepare 1% BSA solution.

    - Using a serological pipette, add 14.35 ml ACSF.IT to a labeled 50 ml conical tube.

    - Using a P200 pipette, add 150 µl FBS.

    - Using a P1000 pipette, add 500 µl 30% BSA. Invert 4-5 times to mix.

  - Prepare 0.2% BSA solution.

    - Using a serological pipette, add 14.75 ml ACSF.IT to a labeled 50 ml conical tube.

    - Using a P200 pipette, add 150 µl of FBS.

    - Using a P200 pipette, add 100 µl 30% BSA. Invert 4-5 times to mix.

- Methodology/Procedures:

  - ROI extraction

    - Reference AF0100 Slice Preparation with Tissue Dissociation (Mouse) SOP to generate acute slices containing ROI. Cut at 350 µm.

    - Using a P1000 pipette, cover the bottom of a Sylgard-coated dissecting dish with ACSF.IT.

    - Using a wide-mouth transfer pipette, place a slice containing ROI in the Sylgard-coated dish and transfer to a dissecting microscope with an attached camera.

    - Dissect out regions of interest.

      - Using microdissecting tools, dissect the regions of interest, leaving them in the cutout section.

      - Acquire brightfield overview image of the slice with ROI cutouts in place.

      - Using the microdissecting tools, separate the ROI cutouts from the rest of the tissue.

    - Attach a wide-bore tip to a P1000 pipette.

      - Aspirate the ROI cutouts, along with a small amount (~1 ml) of the ACSF that is in the dish and dispense into a 5ml polystyrene tube labeled with sample information.

    - Repeat steps 8.1.3-8.1.5.1 for the desired number of slices, combining all ROI cutouts in the same tube.  Keep sample tube at room temperature until ready for papain incubation.

  - Preparation of Tissue Samples

    - Using a P1000 pipette, remove as much supernatant as possible from tissue pieces in sample tube. Add 0.5-1 ml papain. Repeat twice more, leaving the final 0.5-1 ml of papain in the tube. Ensure all tissue is submerged in enzyme.

    - Gently tap side of tube to ensure tissue pieces are not stuck together or are mostly separated from each other, then place tube in dry oven at 35ºC for 30 minutes. Note: it may be necessary to rest the tube at an angle to keep tissue from sinking to the bottom. If pieces are not separating, it may be necessary to gently swirl tube 15 minutes into the digestion.

    - After papain digestion has completed, wash the sample three times.

      - Using a P1000 pipette, remove as much papain as possible from tube. Add 1 ml 0.2% BSA solution.

      - Remove as much supernatant as possible, then add a second 1 ml of 0.2% BSA solution. Repeat a third time, leaving the final 1 ml of 0.2% in the tube.

    - Let sample sit on wet ice for 5 minutes.

      - While waiting, prime the 600 µm, 300 µm, and 150 µm fire-polished pipettes by drawing up ACSF.IT, 0.2% BSA or 1% BSA and dispensing into a waste container.

      - Prepare a second 5 ml polystyrene tube on ice and label appropriately.

    - Once sample has chilled, gently triturate tissue with a 600 µm pipette approximately 20 times or until tissue pieces appear small enough to fit into bore of 300 µm pipette.

      - Allow tissue pieces to sink to bottom of tube (~1 min), then use a P1000 pipette to transfer supernatant to the second 5 ml polystyrene tube on wet ice that has been appropriately labeled.

    - Resuspend tissue pieces in 1 ml 0.2% BSA solution. Gently triturate tissue with a 300 µm pipette approximately 20 times or until tissue pieces appear small enough to fit into bore of 150 µm pipette.

      - Allow tissue pieces to sink to bottom of tube (~1 min), then use a P1000 pipette to transfer supernatant to the second sample tube used in step 8.2.5.1.

    - Resuspend tissue pieces in 1 ml 0.2% BSA. Gently triturate tissue with a 150 µm pipette approximately 20 times.

      - Allow any remaining tissue pieces to sink to bottom of tube (~1 min).

    - Use a P1000 pipette to again transfer supernatant to the second sample tube (final volume = 3 ml). Add 1 ml 1% BSA to a clean 15 ml conical tube. Using a P1000 pipette, filter contents of second sample tube through 70 µm cell strainer into the 15 ml conical tube.

    - Place filtered sample into a centrifuge set to 4ºC and spin at 100 x g for 10 minutes.

      - If the prep requires sorting, while sample is spinning, prepare cluster tubes.

        - Add 200 µl 1% BSA to each cluster tube. Note: the number of cluster tubes prepared will depend on the estimated yield for that ROI. For large regions, prepare either 6 or 8 tubes. For smaller regions, prepare either 4 or 6.

        - Add 25 µl to another cluster tube to use as a visual guide for how much volume to leave at the end of sorting.

        - Place cluster tubes and guide in a cluster tube rack, cover with lid, and store in a 4ºC refrigerator.

    - Once sample has finished spinning, remove supernatant (~4 ml) and resuspend pellet. If need to perform myelin depletion, proceed to step 8.2.11

      - If FACS sorting, resuspend in 500 µl 0.2% BSA using a wide bore pipette tip and proceed to section 8.3.

      - For no FACs samples remove supernatant and resuspend in 100 µl - 270 µl 0.2% BSA depending on size of ROI

        - Mix well and aliquot 100 µl of sample into a new Epi tube, then proceed to counting step 8.4.

    - Optional Myelin Depletion:

      - Remove supernatant (~4 ml) and resuspend pellet in 100-270 µl of standard ACSF.1T + FBS + 0.2% BSA depending on size of ROI

      - Add 30 µl Myelin Removal Beads to the sample and mix well with the pipette

      - Incubate samples for 15 min at 4°C or on ice

      - Wash cells by adding 2700 µl ACSF.1T + FBS + 0.2% BSA to the sample tube

      - Spin down at 300 x g for 10 minutes at 4°C

        - During centrifugation, place an equal number of LS columns as you have samples in the magnetic field

        - Rinse each column with 3 ml ACSF.1T + FBS + 0.2% BSA

      - After centrifugation, remove all supernatant

      - Resuspend the pellets in 1000 µl ACSF.1T + FBS + 0.2% BSA

      - Aliquot each cell suspension into its respective LS column and collect cells that pass through into 13 ml tube

      - Wash the column 2 times with 1 ml ACSF.1 + FBS + 0.2% BSA, collecting all solution that passes through the column

        - Only add new buffer for washing when the column is completely empty

      - Spin down at 300 x g for 10 minutes at 4°C

      - Remove supernatant and resuspend the pellet in 75 µl (if pellet is small) – 200 µl of ACSF.1T + FBS + 0.2% BSA

      - Add 100 µl sample to a lo-bind epi tube, set the remaining sample aside and proceed to Section 8.4.6

  - Cell Sorting

    - Prepare a collection tube by adding 750 µl 0.2% BSA to a clean 5 ml polystyrene tube.

      - 30,000 cells should be collected per 750 µl of collection buffer. If 60,000 cells can be collected, it is better to double the volume of 0.2% BSA in collection tube and split final volume into 2 prepared cluster tubes once sorting is finished.  The ROI will determine the expected yield.

    - Transfer collection tube and sample tube to FACS operator. Ensure flow operator has cytometer set to “Fine Tune” sort mode.

    - Begin collecting cells from population of interest (in most cases from fluorescent reporter labeling specific population).  Add DAPI or Hoechst counterstain to sample according to project specifications if needed.

      - Flow rate should be adjusted such that reaching cell collection target takes only 5-10 minutes; this typically ranges from flow rate 3-7 but will ultimately depend on sample density.

    - Once collection target has been reached, remove collection tube from flow cytometer. Volume in collection tube should now be about 1 ml (2 ml if 60,000 cells sorted).

    - Using a P1000 pipette, gently mix the sample up and down once, then transfer entire contents of collection tube to the prepared cluster tube containing 200 µl 1% BSA if 30,000 cells were sorted into 750 µl, OR split evenly between two cluster tubes if 60,000 cells were collected in 1.5 ml. Gently press pipette tip to inside wall of cluster tube and slowly dispense sample so that it gently runs down side of tube to avoid disturbing the 1% BSA layer on the bottom.

    - Place cluster tube(s) in rack and cover with lid. Spin sample down at 230 x g for 10 minutes at 4ºC.

    - While sample is spinning, prepare a second collection tube.

      - If sparse sample or low volume remaining, prepare second collection tube with 750 µl 0.2% BSA.

    - Immediately following centrifugation, carefully remove supernatant leaving less than 41.6 µl in each cluster tube.

      - Use the 25 µl cluster tube guide prepared earlier to compare volumes during supernatant removal.

    - Using a wide-bore P200 pipette tip, resuspend cell pellet in cluster tube.

      - Repeat steps 8.3.6-8.3.9 for all cluster tubes filled during sorting, leaving all resuspended 25 µl single cell suspensions at 4°C until ready to combine into final sample.

    - Repeat steps 8.3.2 to 8.3.5 until no sample remains in flow cytometer.

      - Collection tubes sorted after the first round will likely be filled before the first sample has finished spinning down. In this case, prepare them for centrifugation per step 8.3.5 and store in a lidded cluster tube rack in a 4ºC refrigerator until centrifuge is available.

  - Preparation of Final Cell Suspension

    - When finished sorting and all cluster tubes have been spun down, prepare final sample for handoff by combining all single cell suspensions from spun down cluster tubes into a labeled 1.5 ml LoBind tube

    - Using a P200 pipette, estimate final volume in LoBind tube by gradually increasing the pipette capacity until entire sample can be aspirated into tip. Final volume is used to determine the number of ports that can be loaded, for version 3 10x Genomics, 41.6 µl are needed for each port. For version 3.1 10x Genomics, 43.1 µl are needed for each port (most of the time, the # of ports will equal how many times 25k cells were sorted).

    - To verify/count concentration of FACS sorted samples for handoff use the following volumes:

55 µl - 1 port

100 µl - 2 ports

145 µl - 3 ports

190 µl - 4 ports

230 µl - 5 ports

275 µl - 6 ports

315 µl - 7 ports

360 µl - 8 ports

    - To verify/count concentration of no FACS sorted samples for handoff use the following volumes:

81 µl - 1 port

126 µl - 2 ports

216 µl - 3 ports

261 µl - 4 ports

306 µl - 5 ports

351 µl - 6 ports

396 µl - 7 ports

441 µl - 8 ports

    - Bring up final volume of sample using 0.2% BSA if needed, referencing above table. Keep on ice at all times.

    - Verify cell concentration

      - To count suspension using the Nucleocounter:

        - Make sure sample is well mixed before taking for count

        - In a labeled clean micro tube, add 19 µl sample to 1 µl Solution 13 for no FACS and for a FACS sorted sample add 11.4 µl of sample to 0.6 µl Solution 13 for FACS sorted samples.

        - Mix well and load into chamber slide and count using Nucleocounter, the final Total cell range should be between 320/µl – 1,200/µl Total cells for 10x v3.1 and 360/µl – 400/µl Total cells for 10x v3.

        - If sample is concentrated use the following equation to determine the volume needed of final sample to dilute by:

C1 * V1 = C2 * V2

C1 = number of total cells in concentrated sample

V1 = volume of final sample needed

C2 = desired concentration of dilute sample

V2 = desired final diluted sample volume

    - To count nuclei suspension manually using Trypan:

      - Using a P20 pipette, add 10 µl Trypan blue to one of the wells of a disposable hemocytometer.

        - Use a phase-contrast microscope, ensure the Trypan blue is clean and does not contain any debris.

        - If debris or contamination is present, obtain fresh Trypan blue and repeat above steps.

      - Obtain a 0.2 ml PCR tube

        - Using a P20 pipette, add 10 µl Trypan blue to the PCR tube.

        - Mix final solution well with pipette. Then, using a P20 pipette, add 10 µl of the final cell suspension to the PCR tube.

      - Prepare a disposable hemocytometer

        - Using a P20 pipette, mix the Trypan blue and cell mixture, then add 10 µl of the mixture into both wells of the hemocytometer. It is important to load entire 20 µl with no bubbles as cell concentration is determined off this volume.

2*10*(total cell count/8) = cells/µl

Use a phase-contrast microscope to count both the number of live cells present in the four corner squares on each side of the hemocytometer for a total of 8 squares counted, as well as the number of ALL cells (live and dead). Cell density can be calculated using the following formula:

      - Maximum cell concentration to load is 158 TOTAL cells counted (395/µl) for 10x V3 and 152 TOTAL cells counted (380/µl) for 10x v3.1.

        - If the total cell count exceeds the maximum (395/µl), the sample will need to be diluted with 0.2% BSA. To determine the final volume necessary to be within concentration range, use the following formula:

Cells/µl*µl of pooled sample = cells in sample

Cells in sample/395 = final volume after dilution

    - Once cell concentration has been confirmed, hand off final cell suspension in LoBind tube to the RNA-Seq Team for 10x processing and fill out appropriate sample information in required tracking documents.

- Take Down:

  - Wipe down all work surfaces with 70% EtOH.

  - Place any leftover sample and sample containers in a designated biohazard waste container.

    - Dispose of any remaining buffers (from 7.3, 7.5, 7.6) down the sink.

  - Place used papain vial and metal seal in sharps container.

  - Disposal of Tissue and Contaminated Material:

    - All tissue and contaminated material must be disposed of via the Biohazardous Waste stream in an approved Biohazard Waste Container.

- Appendix:

  - Loading Guidelines

    - Maximum concentration to load for v3 is 395 Total cells/µl. Minimum cell concentration to load for 10x Genomics v3 is 100 Live cells/µl for normal/standard ROI, and 85 Live cells/µl if ROI is known to be small/limiting.  If your counts are below these thresholds, please notify supervisor and 10x team.

| Max Cell Loading Concentration Guidelines v3 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total Cell Count (manual) | Total Cell Concentration | Total Cell Capture |  | Volume to load |  | Total Cells Loaded |  |  | Cell Prep Comments |  |  |
| 158 | 395 | 9,531 |  | 41.6 |  | 16432 |  |  | maximum load |  |  |
| Min Cell Loading Concentration Guidelines v3 |  |  |  |  |  |  |  |  |  |  |  |
| Live Cell Concentration | Live Cell Capture | Volume to load |  | Live Cells Loaded |  |  | Cell Prep Comments |  |  |  |  |
| 100 | 2,413 | 41.6 |  | 4160 |  |  | minimum guideline |  |  |  |  |
| 88 | 2,111 | 41.6 |  | 3640 |  |  | OK if ROI is limiting |  |  |  |  |
| 85 | 2,051 | 41.6 |  | 3536 |  |  | OK if ROI is limiting |  |  |  |  |
| 78 | 1,870 | 41.6 |  | 3224 |  |  | do not load/alert |  |  |  |  |
| 68 | 1,629 | 41.6 |  | 2808 |  |  | do not load/alert |  |  |  |  |
| 55 | 1,327 | 41.6 |  | 2288 |  |  | do not load/alert |  |  |  |  |

    - Maximum concentration to load for v3.1 is 380 Total cells/µl. Minimum cell concentration to load for 10x v3 is 88 Live cells/µl for normal/standard ROI, and 73 Live cells/µl if ROI is known to be small/limiting.  If your counts are below these thresholds, please notify supervisor and 10x team.

| Max Cell Loading Concentration Guidelines v3.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Total Cell Count (manual) | Total Cell Concentration (cells/ul) | Total Cell Capture | Volume to load | Total Cells Loaded | Cell Prep Comments |
| 152 | 380 | 10,670 | 43.2 | 16416 | maximum load |

| Min Cell Loading Concentration Guidelines v3.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Live Cell Count (manual) | Live Cell Concentration (cells/ul) | Live Cell Capture | Volume to load | Live Cells Loaded | Cell Prep Comments |
| 35 | 88 | 2,457 | 43.2 | 3780 | minimum guideline |
| 30 | 75 | 2,106 | 43.2 | 3240 | OK if ROI is limiting |
| 29 | 73 | 2,036 | 43.2 | 3132 | OK if ROI is limiting |
| 26 | 65 | 1,825 | 43.2 | 2808 | do not load/alert |
| 22 | 58 | 1,615 | 43.2 | 2484 | do not load/alert |
| 19 | 48 | 1,334 | 43.2 | 1334 | do not load/alert |