Sinking assay measurement protocol

Sinking rate assays were performed by monitoring chlorophyll a fluorescence in the same plate spectrofluorometer (Molecular Devices SpectraMax Gemini EM, Sunnyvale, California). To begin an experiment for a 96 well-plate, 300 µL of culture was carefully pipetted into a well containing a parylene-coated stir bar (VP 711D-1, V&P Scientific, INC). For experiments that required 24 well-plates 2.8 mL of cultures were used in a well along with a parylene-coated stir bar. The culture sample depth was 3.8 mm in both 96 and 24 well plates, with the difference in volume caused by the difference in diameter, not the depth, of the well. The addition of the stir bar aided the thorough re-suspension of cells in all rows and column of well plates and thereby helped to minimize variation generated by the unequal re-suspension of diatoms in the well column during shaking of the well plate.

After acclimation (16 °C, 120 µmol m-2 s-1) for a minimum of 10 minutes, well-plates were mixed on a Cooke Microtiter Micro Mixer for 30 seconds and then placed on the stage of plate spectrofluorometer. Fluorescence (excitation: 445 nm; emission: 680 nm), with the same settings used to measure culture growth, was repeatedly monitored from the top of the well-plate during a determined time course. The distance from the photodioded fluorescence detector to the top of the culture (~2 mm) was consistent between the 96 and 24 well plates and fluorescence was measured from the central region of each culture well. After the sinking assay time course, the spectrophotometer (Molecular Devices, SpectraWorks) generated .txt file was exported to a Dropbox directory for processing by the SinkWORX script (https://www.dropbox.com/sh/w03c2nt97rjk990/AACyg_nJrl75ztT9s6A89zT-a?dl=0) (Fig 1).

Fig 1. Sinking Assay Schematic. One well is depicted at different time points, left to right, during the sinking assay. The well contains phytoplankton culture with isotropic fluorescence emission symbolized by red arrows, a stir bar and the fluorescent detector at the top.  The isotropic characteristic of emitted fluorescence permits the monitoring of phytoplankton sinking.

The largest relative fluorescent (RFU) will be received by the detector when Elapsed Time is 0, when the cells are initially evenly suspended. As the cells sink, the likelihood of the isotropic fluorescent emission reaching the detector decreases following the inverse square law as cells move away from the detector, until the RFU hits a plateau once cells in the well were resting at the bottom and giving the same RFU at the detector with each successive read.

Before and after the time course, during initial sinking assay trials, we also measured fluorescence from the bottom of the clear plates to confirm proportional but opposing changes in fluorescence due to sinking when measured from the top or the bottom (data not presented).

Data conversions

The conversion, analysis and comparison of data produced by the sinking assay was performed in R-Studio Software Version 0.99.486 using our SinkWORX data analysis pipeline. The package segmented [22] was used for break point analysis and ggplot2 [23] for figure plotting. The annotated SinkWORX R-scripts, directory structure and data from this manuscript are available at www.dropbox.com/sh/w03c2nt97rjk990/AACyg_nJrl75ztT9s6A89zT-a?dl=0.

Meta-data on the culture sample in each well in a 96 well-plate used for sinking assay was first entered manually using Microsoft Excel in a SinkingCulture_Catalog.csv data sheet included with the SinkWORX files. Raw data produced from each sinking assay was imported from the specifically named Molecular Devices export file and converted in R-scripts to be plotted as RFU from each plate well against elapsed time (minutes), using the meta-data on the well content to tag each sinking profile.

We then scaled RFU:

Scaled RFUt=RFUt-min⁡(RFU)maxRFU-minRFU 	(3)

RFUt is the relative fluorescent signal of the individual time point on which scaling is being performed. The initial even suspension of cells in each well thus generated a scaled RFU of 1 while the final RFU plateau at min (RFU) scaled to 0 once all cells reach the bottom 3.8 mm below the well surface in our system (Fig 1).

Scaling was performed to minimize the variation in the raw fluorescence data that was produced from arbitrary differences in initial cell suspension density among wells. In particular samples from stationary phase cultures typically had much higher cell densities than cultures from exponential phase.

As cells sink towards the bottom of the well, moving away from the detector, we reasoned that scaled RFU initially declines:

Scaled RFUt = 1(1+ dt)2	(4)

where dt is the distance sunk downwards by a cell starting from the top of well over elapsed time t. Empirical and theoretical estimates show that the bulk of our fluorescence signal results from cells starting at the top of the well because:

a) Cells initially lower in the well receive less excitation light because they are initially farther from the emitter and they are screened by cells above them.

b) Cells initially lower in the well therefore emit less fluorescence light, which in turn is attenuated by re-absorption by cells above them, so less fluorescence reaches the detector from cells low in well.

c) Therefore the received fluorescence signal is weighted towards cells at the top of the well and this weighting becomes progressively stronger as the cell suspension density increases, either through a higher starting cell suspension density or as the cells collapse into a dense layer at the bottom of the well. A more sophisticated analyses could, if needed, be derived from work on mudflat diatoms where steep vertical attenuation of light influences measured fluorescence [24,25].

Our goal is to extract a sinking rate for the phytoplankton cells in suspension.

We reasoned that as cells sink:

dt = t (min) * s (min-1) 	(5)

where dt is the distance between the emitting cell and the detector at elapsed time t, and s (min-1) is the scaled sinking rate.

Combining the two equations:

Scaled RFUt = 1 (1+ (t * s))2	(6)

Then

:Scaled RFUt = 1(1+ (t * s))	(7)

which, by a geometric series, approximates to

Scaled RFUt = ~ 1 - (t * s) 	(8)

This gives us a linear formulation to estimate scaled sinking rate. For comparison with literature values of sinking rates the scaled sinking rate was multiplied by the actual distance travelled, the depth of the culture in the well which in our experiments was 3.8 mm. This value was then converted to meters d-1. It was assumed that cells were not affected compositionally by the experiment and that fluorescence is in fact proportional to concentration of cells, at least over the forty minute to three-hour time scale of the sinking experiment.

Some cultures showed an initial increase of measured fluorescence during the first thirty minutes of the sinking assay. This rise in fluorescence could be attributed to the slow relaxation of a phase of the organism's non-photochemical quenching mechanism [26] that resulted from the decrease in light intensity throughout the preliminary steps of the sinking assay protocol compared to the preceding growth light level. Non-photochemical quenching is a mechanism to dissipate excess light-energy in the form of heat to protect the photosynthetic complexes [26,27][28][27]. The relaxation of this mechanism would increase fluorescence emission from cultures as they re-allocate excitation from heat dissipation pathways back to photosynthetic processes.

The sinking of diatoms in the first thirty minutes of assay can thus be masked by the rising fluorescence signal of NPQ relaxation [26]. For analytical purposes in this study we plotted the square root of scaled RFU versus elapsed time and fit the data with a segmented linear regression to separate the initial rise phase from the sinking phase. We then extrapolated back to the y-intercept and rescaled this y-intercept to one. Such detection and correction of NPQ responses will be necessary for accurate assessment of sinking rate, since the amplitudes and relaxation rates of measured NPQ vary with taxa and with growth conditions [26,29], with complex effects upon fluorescence yields[30]. There is no single pre-measurement incubation treatment or period that would guarantee escape from such variations across diverse phytoplankton taxa. Instead we include the data to demonstrate the need for caution when attributing changes in fluorescence signals in terms of cell sinking.