Post Patch Clamp Slice Fixation

- Scope and Applicability: Process to fix recorded/filled cells from mouse and human brain slices with 4% PFA/2.5% Glutaraldehyde in PBS for future histochemical processing.

- Materials:

  - #0 Paintbrush (Ted Pella 11810)

  - 1.7 ml Microfuge tubes (Olympus Plastics 224-282 or equivalent)

  - 1000 µl pipette tip (Rainin GP-L1000F or equivalent)

  - 20 µl microloader tip (Eppendorf 930001007)

  - 24 well plate (Costar CLS3524 or equivalent)

  - 24 well plate insert (Pelco 36172)

  - 4% Paraformaldehyde/2.5% Glutaraldehyde in PBS (SOP RP0229)

  - ACSF.III (SOP RP0220)

  - Biocytin (Sigma B4261)

  - Forceps (Roboz RS-5045)

  - Ice Bucket with lid

  - Patch-Seq Internal Solution (SOP RP0235)

  - Mouse (SOP AF0099) or human (SOP PF0295) brain slices in ACSF.III

  - PBS (SOP RP0001)

  - 3 ml plastic Pasteur pipette (BD Falcon 357575), tip cut off

  - Wet Ice

  - Petri dish (100 mm outside diameter)

  - Funnel

- Equipment:

  - P20 Pipette

  - P1000 Pipette

  - Balance

  - Vortex

  - Shaker tray

  - 4°C fridge

  - AirClean 600 Workstation

- Safety:

  - Nitrile Gloves

  - Eye protection

  - Lab coat

  - AirClean 600 Workstation

- Output:

  - Fixed brain slices in PBS with target cell(s) used for electrophysiology recordings filled with Biocytin for downstream staining.

- Reference Documents:

  - AF0099 Slice Preparation and Blockface Imaging for Electrophysiology – Mouse

    - https://www.protocols.io/view/slice-preparation-and-blockface-imaging-for-electr-bdpqi5mw/abstract

  - PF0295 Human Neurosurgical Tissue Processing

    - https://www.protocols.io/view/human-neurosurgical-tissue-processing-bdvki64w/abstract

  - PF0301 Patch-Seq Recording and Extraction

    - https://www.protocols.io/view/patch-seq-recording-and-extraction-bepyjdpw/abstract

  - RP0001 Phosphate Buffered Saline (PBS)

    - https://www.protocols.io/view/phosphate-buffered-saline-pbs-bezgjf3w/abstract

  - RP0229 4% Paraformaldehyde/2.5% Glutaraldehyde in PBS

    - https://www.protocols.io/view/4-pfa-2-5-glutaraldehyde-fixative-bg5sjy6e

  - RP0036 20% Paraformaldehyde in 5X PBS

    - https://www.protocols.io/view/20-paraformaldehyde-in-5x-pbs-bg4zjyx6

  - RP0220 ACSF.III

    - https://www.protocols.io/view/artificial-cerebrospinal-fluid-iii-acsf-iii-beptjdnn/abstract

  - RP0235 Patch-Seq Internal Solution with Biocytin

    - https://www.protocols.io/view/patch-seq-internal-solution-with-biocytin-bdvti66n/abstract

Warning: Personal Protective Equipment (PPE) should be used at all times while operating this protocol. If you are unsure what PPE you should be using, see your immediate supervisor.

- Setup:

  - Obtain tissue slices prepared following SOP AF0099 or PF0295.

  - Set-up for Patch-seq using Patch-Seq Recording and Extraction SOP PF0301.

  - Obtain fresh 4% PFA/2.5% Glutaraldehyde per SOP RP0229 and fresh PBS per SOP RP0001.

  - Fill ice bucket with wet ice to be used to keep the Biocytin Solution chilled.

  - Use label maker to label a 24 well plate (without insert) “EPhys_YYMMDD_plate#”. Place the 24 well plate in the AirClean hood.

    - Human tissue should go in a separate PFA/Glut plate, labeled the same way as the mouse PFA/Glut plates, but with a note for human tissue only.

  - Fill each well of a second 24 well plate (w/ plate insert) with ~1 ml 1X PBS(-). Fill through the insert to ensure that the well gets filled properly.

    - Use label maker to label insert with the plate name “EPhys_YYMMDD_plate#” on the flat edge of the insert. This plate can go on the shaker tray in the fridge if not adding fixed slices to it from the previous day/week for staining (Figure 1).

    - Human tissue should go in a separate PBS plate from mouse tissue.

Figure 1: Labeling the plate insert.

- Methodology/Procedures:

  - After recording from a neuron according to the Patch-Seq Recording and Extraction SOP PF0301, pull the pipette away from the cell diagonally until a GΩ seal forms.

    - Switch to the “Approach” setting on the control cube (Figure 2).

| Figure 2. “Approach” switch on the control cube. |  |
| --- | --- |

    - Slowly pull away from the cell in the X direction.

    - Place your finger inside one of the grooves on the control cube, move your index finger counter-clockwise completing roughly one revolution every five seconds.

  - If finished patching in the slice, remove the harp from on top of the cell and allow the cell to rest for 10 minutes.

  - Fill the next empty well of the appropriate (mouse vs. human) PFA/Glut plate with ~500 µl of 4% PFA/2.5% Glutaraldehyde. Use the Pasteur pipette to transfer the slice from the stage to the well. (Note that human and mouse tissue will go into different PFA/Glut plates).

  - Write the specimen ID, as well as your initials, on the lid of the PFA/Glut plate above the well occupied by the slice. (Figure 3).

Figure 3: Labeling the 4% PFA/2.5% Glutaraldehyde plate.

  - Once through recording for the day, place the 24 well plate with slices in 4% PFA/2.5% Glutaraldehyde on the orbital shaker tray in the 4°C fridge.

    - When recording after hours, as typical for human tissue, the PFA/Glut plate should be kept in the 4°C fridge the entire time and only removed when transferring slices to prevent the plate getting left out overnight at room temperature.

  - After slices have fixed for 2 days (>40hrs), transfer slices from the PFA/Glut plate to the corresponding well in the PBS plate.

    - Ideally, slices are fixed for two days and transferred to PBS the following morning, but can be transferred up to four days later without issue (in the case of a weekend or holiday weekend). Ensure all slices are adequately submerged in PBS.

  - When all slices have been transferred to PBS, empty the PFA/Glut plate(s) into the PFA waste bottle in the AirClean hood using a funnel.

- Technical Information:

  - 4% PFA + 2.5% Glutaraldehyde used for patch clamp slice fixation has a shelf life of 8 days when stored at 4ºC.

  - 50% Glutaraldehyde stock solution should be stored at 4ºC.

- Appendix:

  - Preparation of Internal Solution/Biocytin mix. (Note: only required if pre-made solution is not available).

    - Obtain a tube of Internal Solution from the -20ºC freezer.

    - If Biocytin powder has not already been weighed out into a microfuge tube, weigh out between 4 and 5 mg of Biocytin into a microfuge tube using a balance.

    - Determine how much Internal Solution to add to the Biocytin powder:

| mg of Biocytin powder | = volume of Internal Solution needed (µl) |
| --- | --- |
| percent desired Biocytin concentration |  |

Example: To make a 1% Internal Solution/Biocytin mix, weigh 4.2 mg of Biocytin and add 420 µl of Internal Solution to the Biocytin tube.

    - Vortex the tube until all the Biocytin is dissolved into solution.

    - Put the tube of Biocytin internal mix in the ice bucket and cover the bucket with the lid.