Preparation of Gelatin Slide Coating Solution

- Scope and Applicability:  Preparation of gelatin solution to be used for coating glass microscope slides.

- Materials:

  - Gelatin powder (Type A, 275 bloom, Fisher G8-500 or equivalent)

  - Chrome alum, Chromium (III) Potassium Sulfate dodecahydrate (Sigma 243361 or equivalent)

  - MilliQ water

- Equipment:

  - Heated stir plate

  - Thermometer

  - Stir bar

  - Calibrated balance

  - Glass Erlenmeyer flask or beaker (500 ml or appropriate size, as determined by desired yield)

- Safety:

  - Nitrile Gloves

  - Lab coat

  - Safety glasses

- Output:

  - 0.3% gelatin solution with chrome alum

- Reference Documents

  - N/A

Warning:  Personal Protective Equipment (PPE) should be used at all times while operating this protocol.  If you are unsure what PPE you should be using, see your immediate supervisor.

- Setup:

  - Determine the required volume of gelatin solution.

  - Determine the correct size glass container to use, to hold solution.

- Methodology/Procedures:

  - Add the required volume of MilliQ water to a glass container according to Table 1.

Table 1: 0.3% Gelatin Solution

| Reagent | Final Volume 100 ml | Final Volume 250 ml | Final Volume 500 ml |
| --- | --- | --- | --- |
| MilliQ water | 100 ml | 250 ml | 500 ml |
| Gelatin powder | 300 mg | 750 mg | 1500 mg |
| Chromium Potassium Sulfate | 50 mg | 125 mg | 250 mg |

  - Once the water has reached 65C, gently add the gelatin powder (avoid clumps).

  - Heat the water to 65C on a heated stir plate.

  - Stir gently until gelatin goes into solution; avoid introducing bubbles into the solution.  The solution will be clear when gelatin is dissolved.

  - Once the gelatin is fully dissolved, add the Chromium Potassium Sulfate. Stir until dissolved.

  - Reduce heat and cool gelatin to ~50C prior to dipping slides. Slides should be coated immediately.

- Take Down:

  - Unused solution can be diluted ≥2X with warm water and poured down the municipal drain.

  - Rinse the containers thoroughly with hot water then clean appropriately with MilliQ water and 70% EtOH.

- Technical Information:

  - This SOP is based upon the IHC World Online protocol:

http://www.ihcworld.com/_protocols/histology/gelatin_slides.htm

3g Gelatin

After the gelatin dissolves

Add 0.5g Chromium Potassium Sulfate

1L MilliQ water

Heated to 65C

Stir gently to minimize bubbles

Coat slides in gelatin solution for 30 seconds

Staining container

1x3 slides in staining rack

Dry at room temperature in a dust and lint-free environment

Transfer into slide boxes ready for use and for storage at room temperature

Cool solution to 40-50C and filter into staining container

Process modified from:

http://www.rndsystems.com/ihc_detail_objectname_gelatin_slides.aspx