Production of AAV Virus by Iodixanol Gradient Ultracentrifugation

- Scope and Applicability: This protocol can be used to produce AAV of any serotype with high purity and high titer.

- Materials:

  - HEK293T cells: (ATCC CRL-11268)

  - Plasmid DNA

    - pAAV DNA

    - pHelper DNA

    - Serotype-specific Rep-Cap Plasmid

  - Polyethylenimine (PEI) linear MW 25,000 (Polysciences 23966 or equivalent)

  - Benzonase (Sigma-Aldrich E1014 or equivalent)

  - Medium and Buffer

    - DMEM with high glucose, L-glutamine and sodium pyruvate (Thermo Fisher Scientific 11995-065 or equivalent).

    - Opti-MEM I Reduced Serum Medium, GlutaMAX (Thermo Fisher Scientific 51985-034 or equivalent).

    - DPBS (Thermo Fisher Scientific 14190-144 or equivalent).

    - TrypLE Select (Thermo Fisher Scientific 12563-011 or equivalent)

    - FBS (VWR 89510-184 or equivalent)

    - OptiPrep™ Density Gradient Medium (60% (w/v) solution of Iodixanol in water (sterile)) (Sigma D1556-250ML or equivalent)

    - Antibiotic-Antimycotic Solution (100X) (Sigma A5955-100ML or equivalent)

    - Sodium Chloride Solution, 5M, (Fisher Scientific PR-V4221 or equivalent)

    - Magnesium chloride solution, 1M (Sigma 63069 or equivalent)

    - Tris (pH 7.6), 1M (Sigma T2788-1L or equivalent)

    - Phenol Red solution, 0.5%, liquid, sterile-filtered (Sigma P0290 or equivalent)

    - Glycerol for molecular biology, ≥99.0% (Sigma G5516 or equivalent)

    - 70% Ethanol

    - 12N Hydrochloric Acid (VWR JT9535-1 or equivalent)

- Equipment:

  - Serological pipettes (5, 10, 25 and 50 mL)

  - Eppendorf Easypet 3 electronic pipette controller (or equivalent)

  - Gilson P1000, P20, P200, P10 single channel pipettes

  - Diamond 1000, 200 and 20 µL tips filtered tips (Gilson or equivalent)

  - 15 ml Tubes (Falcon 352095 or equivalent)

  - 50 ml Tubes (Falcon 352070 or equivalent)

  - Disposable cell scraper (Fisher Scientific 08-100-242 or equivalent)

  - Culture dish, 150 mm (VWR SC25383-103 or equivalent)

  - Hamilton™ Metal Hub Custom Needles (Luer Lock) Length: 5 inch (5'') Point style: PT3 (Fisher Scientific 22-288678)

  - Water bath

  - Vortex Mixer

  - Ice Bucket

  - Biosafety cabinet

  - Tissue culture incubators

  - Fluorescence inverted microscope (Carl Zeiss, Olympus, or equivalent)

  - Centrifuge (Eppendorf, BioRad or equivalent)

  - Ultracentrifuge (Beckman Optima L-100)

  - Rotor with buckets (Beckman SW70Ti)

  - Polycarbonate Bottle with Cap Assembly (Beckman 355618)

  - Polycarbonate Bottle Only (Beckman 355654)

  - Amicon Ultra-15 Centrifugal Filter Units, 100 KDa (Sigma UFC910024 or equivalent)

  - pH Meter

- Safety:

  - Nitrile Gloves

  - Eye protection

  - Lab coat

- Output:

  - AAV virus of any serotype with high purity and high titer

- Reference Documents:

  - https://www.addgene.org/protocols/aav-purification-iodixanol-gradient-ultracentrifugation/

  - https://www.nature.com/articles/3300938.pdf?draft=collection

Warning:  Personal Protective Equipment (PPE) should be used at all times while operating this protocol. If you are unsure what PPE you should be using, see your immediate supervisor.

- Setup:

  - Prepare DMEM with 10% FBS and 1% Antibiotic-Antimycotic by adding 50 mL of FBS and 5 mL of Antibiotic-Antimycotic to 500 mL of DMEM, and store at 4ºC

  - Prepare DMEM with 4% FBS and 1% Antibiotic-Antimycotic

  - 1 mg/ml Polyethylenimine, linear MW 25,000 Da (PEI)

7.3.1 	Dissolve 1 mg of PEI (Polysciences 23966) in 1 mL of PBS

7.3.2	Adjust to a pH between 4 and 5 with 12N HCl

7.3.3	Sterilize the solution by passing through a 0.22 µm filter unit;

7.3.4 	Aliquot and store the solution at -20ºC

7.3.5	Keep the working solution at 4ºC

  - Prepare 10X Gradient Buffer (GB) (100 mL)

    - 10 mL 1M Tris (pH 7.6)

    - 30 mL 5M NaCl

    - 10 mL 1M MgCl2

    - 50 mL ddH2O

    - Filter sterilize using 0.22 µm vacuum filter system

    - Store at RT

  - Prepare 15% Iodixanol gradient solution (100 mL)

    - 25 mL Optiprep (60%) Iodixanol solution

    - 10 mL 10X GB

    - 20 mL 5M NaCl

    - 45 mL ddH2O

    - Mix Iodixanol solution well and store at 4ºC.

  - Prepare 25% Iodixanol gradient solution (70 mL)

    - 29.2 mL Optiprep (60%) Iodixanol solution

    - 7 mL 10X GB

    - 33.8 mL ddH2O

    - 175 µL Phenol Red

    - Mix Iodixanol solution well and store at 4ºC.

  - Prepare 40% Iodixanol gradient solution (150 mL)

    - 100 mL Optiprep (60%) Iodixanol solution

    - 15 mL 10X GB

    - 35 mL ddH2O

    - Mix Iodixanol solution well and store at 4ºC.

  - Prepare 60% Iodixanol (150 mL)

    - 150 mL Optiprep (60%) Iodixanol solution

    - 225 µL Phenol Red

    - Mix Iodixanol solution well and store at 4ºC.

  - Prepare washing buffer

    - 1X PBS

    - 5% Glycerol

    - 35 mM NaCl

- Methodology/Procedures:

  - Transient transfection of 293T cells

    - Seed 2 × 107 293 cells in 15 cm dishes to achieve ∼70% to 80% confluency the next morning (split confluent 15 cm plates of cells 1:2 into 15 cm plates). Cells are maintained in complete DMEM/10% FBS/1% anti-anti and split before reaching confluency by trypsinizing with TrypLE. Each plate contains 20 mL of culture medium.

    - Ten plates are required for a large-scale production. The number of plates can be scaled based on the production scale.

    - For transfection of 10, 15 cm dishes, combine the following in a 15 mL polystyrene tube:

      - 105 μg Serotype-specific Rep-Cap Plasmid

      - 105 μg rAAV vector plasmid

      - 190 μg pHelper plasmid

    - Add 5 mL of OptiMEM to the tube, and mix by vortex briefly for 5 seconds.

    - Add 1.1 mL of PEI to the mixture, vortex briefly for 5 seconds, and incubate 5-10 min at room temperature.

    - Add 0.61 mL of mixture from step 8.1.6 dropwise to the medium in each and swirl the medium to disperse evenly

    - Change the medium to 20 mL of DMEM/4%FBS/1% Antibiotic-Antimycotic 24 hr post-transfection, and incubate the cells until 72 hr post-transfection

    - At 72 hr post-transfection, each plate of cells is scraped using a cell scraper, and the mixture of cells and culture medium is transferred to a 50 mL tube.

    - Cell pellets are collected after centrifugation at 500 x g at 4ºC for 15 min

    - Remove supernatant, resuspend cells in 10 mL of 1X gradient buffer, mix by pipetting and freeze the 50 mL tube in dry ice in an ice bucket.

    - Thaw cells in a 37ºC water bath, and freeze/thaw 3 more times using dry ice and water bath.

    - After the last thaw, add 50 U/mL Benzonase to the suspension and incubate at 37ºC for 30 min (2.6 µL of enzyme/10 mL)

    - Centrifuge the 50 mL tube at 3,000 x g for 15 min at 4ºC and collect supernatant.

    - Supernatant can be frozen at -80ºC or purified directly.

  - Iodixanol gradient purification.

    - Build an iodixanol gradient in Beckman Polycarbonate Bottle with Cap Assembly. The 4 layers of iodixanol gradient solutions from the bottom of the tube to the top are 60% (3mL), 40% (3mL), 25% (3.5mL), and 15% (6mL).

Figure 1. Preparation of iodixanol gradient Source: https://www.addgene.org/protocols/aav-purification-iodixanol-gradient-ultracentrifugation/

    - Start by adding the 15% solution to the bottom of the Beckman tube using serological pipets, and add subsequent layers below the previous layer, 25%, 40% and 60%, by using a 10 mL syringe with the Hamilton metal hub needle. Place the needle to just above the bottom of the Beckman tube and slowly inject.

    - Add the Benzonase-treated supernatant to the gradient by slowly dripping the solution onto the top layer of the gradient. For 10, 15 cm plates, we usually obtain 10 mL of Benzonase-treated supernatant, which can be divided into 2 tubes for gradient purification. The tubes can be balanced using gradient buffer.

    - Close the top of the tubes and centrifuge the tubes at 58,000 rpm in a Beckman Type 70Ti rotor for 90 min at 18ºC.

    - Add 5 mL wash buffer to Amicon Ultra-15 Centrifugal filter unit and spin at 3000 g to prime filter

    - Carefully move the tubes back to the hood and extract about 4mL of solution using a 10 mL syringe with the Hamilton metal hub needle and inserting the needle approximately 1-2 mm below the interface between the 40% and 60% gradient buffer layers.

    - The collected solution is transferred to a primed Amicon Ultra-15 centrifugal filter unit, added with washing buffer and centrifuged at 3,000 rpm at 4ºC. The final volume is reduced to 150 µL after sufficient washing.

    - Viral samples should be aliquoted and stored at -80ºC for long term storage.

- Take Down:

  - Clean the hood with 70% Ethanol after use.

  - The biohazard waste generated during the process is disposed according to the biohazard waste procedure guidelines.

  - The aluminum caps of the Polycarbonate Bottle with Cap Assembly (Beckman, cat. no. 355618) can be autoclaved, but the bottles are discarded after each use.

- Technical Information:

  - Tubes containing iodixanol gradient and AAV lysate need to be weighted before loading to the ultracentrifuge. The difference in weight between tubes is controlled to be less than 0.05 g.

  - The purification process can be temporarily stopped at step 8.1.10 when cell pellet is collected and can be kept at -80ºC, and at step 8.1.15 when AAV cell lysate can be kept at -80ºC.

  - If a fluorescent reporter is part of the AAV design, check the transfection efficiency using an epifluorescence microscope.  The transfection efficiency should be around 70-80% or higher in order to obtain high titer of AAV.