Protocol of LRP37 method

- Equilibrate the temperature of 0.6% dextran solution at 37 °C by the incubation of the solution for about 5 minutes in advance.

- Blood is collected with a heparinized syringe.

- Allow the blood to equilibrate at 37 ° C for a few minutes.

- Add 400 μl of the blood to 800 μl of dextran.

- Incubate the mixture at 37 °C for about 10 to 30 minutes until a supernatant is formed.

- Use the supernatant as leukocyte-rich plasma (LRP).

- Add 100 μl of the LRP to 900 μl of the pretreatment solution (0.02% tween, FUJI FILM Wako Pure Chemical Incorporation, Osaka, Japan).

- The 10-times diluted LRP was mixed five times with a vortex mixer at 3000 rpm  for 10 s to ensure the destruction of the white blood cells.

- Heat the mixture at 70 °C for 10 minutes to inactivate the interfering factors in blood for the limulus test.

- Cool the mixture at 0 °C for 5-10 minutes.

- Add 200 μL of the mixture to a freeze-dried endotoxin-specific limulus test; Endotoxin Single test Wako (FUJI FILM Wako) to dissolve the reagent.

- Set the test tube containing the mixture and the reagent in the TOXINOMETER (FUJI FILUM Wako) and react at 37 °C for up to 200 minutes.

- The amount of transmitted light at the start is defined as100%, and the time during which turbidity is reduced to 92% by endotoxin is defined as the gelation time (min).

- A calibration curve is created based on the gelation times of at least three different doses of standard endotoxin (E. coli O111:B4), and the endotoxin content of the sample will be calculated by the gelation time of the sample to be measured.

- Since the sample is diluted 10 times with the pretreatment solution at protocol 7), the calculated endotoxin amount will be 10-times.

- As the plasma is also diluted by the dextran solution, final endotoxin content in LRP should be calculated by the formulae as follows; (A + dextran volume)/A, wherein A = blood volume - blood volume × haematocrit value/100

(When LRP0 method will be performed, the operating temperature in procedures 1) to 5) should be 0°C by dipping a test tube into crush ice bath.)