RNA Isolation and Reverse-transcriptase PCR

RNA Extraction

- Before beginning RNA extraction, clean up the RNA room: change counter-top coverings and wipe everything with RNAase-zap or equivalent product. Use RNAase free plastic wear and rinse all glassware with RNAase-zap.

- This protocol can be done at room temperature

- The exact protocol will depend on the tissue type. Refer to the kit-specific instructions for more details:

  - Liver: http://www.qiagen.com/products/rnastabilizationpurification/rneasysystem/rneasymini.aspx#Tabs=t0

  - Muscle: http://www.qiagen.com/products/rnastabilizationpurification/rneasysystem/rneasyfibroustissuemini.aspx

RNA Isolation: Liver (quick guide to QIAGEN kit)

- All steps to be done at room temperature, but do not allow frozen tissue to thaw before disruption in RLT buffer

- Work quickly, but deliberately

- Do NOT use bleach around these reagents

Reagent Prep

- Buffer RLT may form a precipitate upon storage. If needed, re-dissolve by gently warming, then return to room temperature

- Add 10 uL B-Mercaptoethanol (14.5M) per 1 mL RLT Buffer. Prepared buffer may be stored for up to 1 month at room temperature. Prepare and dispense buffer in fume hood.

- Add 4 volumes 100% ETOH to buffer RPE before first use.

Method

- Obtain 20 mg pieces of liver tissue and place into bead beater tubes (BioSpec Products, Inc., Oklahoma, USA) containing 0.7mm zirconia beads (BioSpec Products, fill tubes ~1/4 to ~1/3 full). Keep on dry ice until ready to begin procedure.

- Add 600 uL prepared RLT buffer to each tube, and secure the lids TIGHTLY.

- Homogenize tissue in bead beater (Mini Beadbeater 16, BioSpec) for 1 minute, rest, then another minute. Ensure good tissue disruption.

- Centrifuge samples for 3 minutes at full speed (in the bead beater tube)

- Remove 500 uL of supernatant in to clean Eppendorf tubes. Avoid pipetting beads or other debris with the supernatant. Discard bead beater tubes.

- Add 500 uL 50% ETOH to the cleared lysate and mix by pipetting.

- Transfer up to 700 uL of sample to an RNeasy spin column, placed inside a 2 mL collection tube.

- Close the lid and centrifuge for 15 seconds at 10,000xg

- Discard the flow-thorough, and repeat for remaining lysate

- Discard the flow-through

- Add 700 uL buffer RW1 to the spin column. Centrifuge 15 seconds at 10,000xg

- Discard the flow-through

- Add 500 uL buffer RPE to the spin column. Centrifuge 15 seconds at 10,000xg

- Discard the flow-through

- Add 500 uL buffer RPE to the spin column. Centrifuge 2 minutes at 10,000xg

- Replace collection tube with a clean one

- Centrifuge 1 minute at full speed

- Discard collection tube and flow-through

- Place spin column in clean, labelled Eppendorf

- Add 50 uL RNase-free water to spin column membrane (do not disturb membrane)

- Centrifuge 1 minute at 10,000xg

- Remove the eluate from the collection tube and replace into spin column, replace spin column into collection tube

- Centrifuge once more for 1 minute at 10,000xg.

- Quantify RNA via spectrophotometric determination of absorbance at 260 nm (Epoch plate reader with Take3 microplate, BioTek, Vermont, USA)

- Evaluate purity via spectrophotometric determination of absorbance at 260 nm divided by absorbance at 280 nm (BioTek)

- Store RNA at -80 °C

RNA Isolation: Muscle (quick guide to QIAGEN kit)

- All steps to be done at room temperature, but do not allow frozen tissue to thaw before disruption in RLT buffer

- Work quickly, but deliberately

- Do NOT use bleach around these reagents

Reagent Prep

- Buffer RLT may form a precipitate upon storage. If needed, re-dissolve by gently warming, then return to room temperature

- Add 10 uL B-Mercaptoethanol (14.5M) per 1 mL RLT Buffer. Prepared buffer may be stored for up to 1 month at room temperature. Prepare and dispense buffer in fume hood.

- Add 4 volumes 100% ETOH to buffer RPE before first use.

Method

- Heat bead bath (Benchmark Iso-Block, Benchmark Scientific, New Jersey, USA) to 55°C

- Obtain 20 mg pieces of muscle tissue and place into bead beater (BioSpec Products, Inc., Oklahoma, USA) tubes containing 0.7mm zirconia beads (BioSpec Products, fill tubes ~1/4 to ~1/3 full). Keep on dry ice until ready to begin procedure.

- Add 300 uL prepared RLT buffer to each tube, and secure the lids TIGHTLY.

- Homogenize tissue in bead beater (Mini Beadbeater 16, BioSpec) for 1 minute, rest, then another minute. Ensure good tissue disruption.

- Add 590 uL RNase-free water and 10 uL Proteinase K to each tube. Mix. Incubate at 55°C for 10 minutes.

- Centrifuge samples for 3 minutes at full speed (in the bead beater tube)

- Remove 500 uL supernatant in to clean Eppendorf tubes. Avoid pipetting any of the beads or other debris with the supernatant. Discard bead beater tubes.

- Add 250 uL of 100% ETOH to the cleared lysate and mix by pipetting.

- Transfer up to 700 uL of sample to an RNeasy spin column, placed inside a 2 mL collection tube.

- Close the lid and centrifuge for 15 seconds at 10,000xg

- Discard the flow-thorough, and repeat for remaining lysate

- Discard the flow-through

- Add 350 uL buffer RW1 to the spin column. Centrifuge 15 seconds at 10,000xg

- Discard the flow-through

- Add 350 uL buffer RW1 to the spin column. Centrifuge 15 seconds at 10,000xg

- Discard the flow-through

- Add 500 uL buffer RPE to the spin column. Centrifuge 15 seconds at 10,000xg

- Discard the flow-through

- Add 500 uL buffer RPE to the spin column. Centrifuge 2 minutes at 10,000xg

- Replace collection tube with a clean one

- Centrifuge 1 minute at full speed

- Discard collection tube and flow-through

- Place spin column in clean, labelled Eppendorf

- Add 50 uL RNase-free water to spin column membrane (do not disturb membrane)

- Centrifuge 1 minute at 10,000xg

- Remove the eluate from collection tube and replace into spin column, replace spin column into collection tube

- Centrifuge once more for 1 minute at 10,000xg.

- Quantify RNA via spectrophotometric determination of absorbance at 260 nm (Epoch plate reader with Take3 microplate, BioTek, Vermont, USA)

- Evaluate purity via spectrophotometric determination of absorbance at 260 nm divided by absorbance at 280 nm (BioTek)

- Store RNA at -80 °C

CDNA Synthesis

- Perform this procedure on wet ice

- Dilute RNA samples to a total volume of 1ug RNA in 10uL RNAase free H2O. Use PCR clean plastic ware.

  - Dilute each sample in triplicate (to make 3 batches of CDNA for each sample)

- Pick a sample to use for the standard curve. Make an additional set of triplicates.

- Make Master Mix (High Capacity cDNA Reverse Transcription Kit, Life Technologies, California, USA):

  - Calculate the total number of reactions (with 3 per sample)

  - Add a couple of extra reactions to account for pipetting loss

  - Add the following ingredients for each reaction:

    - 2.0uL Reaction Buffer

    - 0.8 uL 25X DNTP

    - 2.0 uL 10X random primers

    - 1.0 uL Reverse Transcriptase

    - 4.2 uL RNAase free H2O

- Put 10 uL of master mix into the tubes containing 10 uL diluted RNA. Mix by pipetting.

- Flash spin samples, then load into thermal cycler (Eppendorf, Thermostat, Hamburg, Germany).

- Run CDNA reactions as follows:

  - Step 1: 25°C for 10 minutes

  - Step 2: 37°C for 120 minutes

  - Step 3: 85°C for 5 minutes

  - Step 4: Hold at 4°C indefinitely

Reverse-transcription PCR

- Prepare PCR mater mix (TaqMan, Applied Biosystems, California, USA):

  - Calculate the total number of reactions (with 2 per CDNA sample)

  - Add a couple extra reactions to account for pipetting loss

  - Make the master mix by combining the following in a separate tube:

  - For each reaction:

    - 1 uL TaqMan Probe

    - 10 uL TaqMan master mix

    - 8 uL RNAase free H2O

  - Mix well.

- Pipette 19 uL  of master mix into each well on the plate that will contain a sample, including the standard curve, but skipping the first two wells of the standard curve

- Pipette 1 uL CDNA into each sample well. Run each CDNA sample in duplicate. Mix by pipetting.

- Prepare the standard curve (in duplicate) using the CDNA sample you prepared for this purpose:

  - Load the first two wells of the standard curve with 100ng RNA:

    - 2 uL CDNA

    - 1 uL TaqMan Probe

    - 10 uL TaqMan master mix

    - 7 uL RNAase free H2O

  - Load the next two wells of the standard curve with  50 ng RNA:

    - 1 uL CDNA

    - (should already contain 19 uL master mix)

  - Prepare a 5ng stock:

    - In a separate tube make a 1:10 dilution from 50ng (1ul of 50ng into 9ul of H2O)

    - Load 1 uL into the next two wells of the standard curve

    - (should already contain 19 uL master mix)

  - Using the 5ng solution, prepare a 0.5ng stock:

    - In a separate tube make a  1:10 dilution from the 5ng stock you just made (1ul of 5ng into 9ul of H2O)

    - Load 1 uL onto the next two wells of the standard curve

    - (should already contain 19 uL master mix)

  - Load no template control

    - Load 1 uL RNAase free H2O into the last wells of the standard curve

    - (should already contain 19 uL master mix)

- Use a plate sealing sticker to seal the plate

- Centrifuge the plate briefly on a plate spinner

- Load the plate into the RT-PCR instrument (Applied Biosystems, California, USA)

- Perform the PCR reactions with the following conditions:

  - Step 1: Hold at 50°C for 2 minutes

  - Step 2: Hold at 95°C for 10 minutes

  - Step 3: Denature at 95°C for 15 seconds

  - Step 4: Anneal/Extend at 60°C for 1 minute

  - Repeat Steps 3 and 4 for 40 cycles

  - Reaction volume: 20 uL

- Software setup:

- Open SDS 2.4

- File > New > Standard Curve

- 96 wells

- Blank Template

- Add detector > Copy to plate

- Highlight wells containing samples and select the detector to be used.

- Set up standard to be used

- Label the no template controls as “NTC”

- Set up the rest of the standard curve as “Standard,” indicating the amount of RNA loaded, and label the samples.

- Set up the rest of the plate as “Unknown”

- Label the samples but not the quantities

- Instrument>Standard>Volume:20ul>Real Time>Connect

- Save run

- Open>Close>Start

- After run is completed, analyze the results:

- Select the whole plate

- Make sure that:

- The threshold is in the logarithmic portion of the graph and that it matches the thresholds that pertain to the same detector. They should only be 0.2-0.3 units apart in order to normalize the plates.

- The slope of the graph is around -3.3+10%; this reflects an efficiency of 100% +10%.

Data Reduction

- The RT-PCR program outputs the starting quantity of RNA for each well

- Check that sample replicates match up well with each other and average all of the replicates for each sample

- Divide each sample average of the target gene by the corresponding sample average of the housekeeping gene control

- This is the normalized RNA content; use these values for all other calculations.

- Calculate the mean and standard error for each treatment group

- To calculate the fold change from control to treatment use the following equation:

  - Fold Change= (treatment)/(control)

  - Multiply by 100 to get the percent change