| Reagent | Company | Product no. | Stock Conc. | Final Conc. |  |
| --- | --- | --- | --- | --- | --- |
| Sucrose | Sigma-Aldrich | S0389 | 2 M | 250 mM |  |
| Sodium-pyrophosphate | Sigma-Aldrich | 221368 | 100 mM | 20 mM |  |
| HEPES (pH 7.4) | Sigma-Aldrich | H7637 | 150 mM | 5 mM |  |
| Magnesium chloride | EMD Millipore | 1.05832 | 150 mM | 5 mM |  |
| DTT | Sigma-Aldrich | D0632 | 1 M | 1 mM |  |
| Dextran 70 | Applichem | A1847.0100 | 25 % | 5 % |  |
| Β-glycerophosphate | Sigma-Aldrich | G9422 | 1 M | 20 mM |  |
| Sodium fluoride | Sigma-Aldrich | S1504 | 1 M | 10 mM |  |
| #PMSF | Sigma-Aldrich | P7626 | 200 mM | 2 mM |  |
| EGTA (pH 8.0) | Sigma-Aldrich | E4378 | 200 mM | 1 mM |  |
| *Aprotinin | Sigma Aldrich | A1153 | 2 mg/ml | 10 µg/ml |  |
| *Leupeptin | Sigma Aldrich | L2884 | 2 mg/ml | 10 µg/ml |  |
| *Sodium orthovanadate | Sigma-Aldrich | S6508 | 100 mM | 2 mM |  |
| Benzamidine | Sigma-Aldrich | B6506 | Δ | 3 mM |  |

Supplemental I - Homogenization Buffer

Start by adding distilled demineralized water (ddH20) corresponding to 1/3 of the final volume. Then add the different reagents, store the buffer on ice and adjust pH to 7.5.

PMSF is dissolved in Isopropanol while all other reagents are dissolved in ddH2O.
# indicates that PMSF is dissolved in isopropanol 
* indicates that stock solution is stored at -30 °C. All other reagents were stored at +4 °C.
Δ indicates that 4.65 mg Benzamidine is added to every 10 ml Buffer.